µ §

Claudio Filiberti; Imaging Applications

The diskette contains the following files:

- A sample grab program which allows for the control of the exposure and  transfer    times. Both the source and the executable can be found in the "grab" directory.

- The "logcam" directory contains both the .CAM file and the source code of the

logical camera.

-The "camkit" directory contains all the necessary files to generate the .CAM file. This can be performed by running the mkcam.bat file. This batch file requires two parameters, the path of the source code of the logical camera and the name of this file.

The Mega_16.cam logical camera  is designed for the Kodak Megaplus 1.6. It is interfaced to the IM-ASD through the RS422 digital video port of the camera.

Cable Specification

The cable is constructed with  the Matrox open ended cable for the IM-ASD (IM-DICBL/CSM) and a 68 pin male honda type connector.

It is important to note that the most significant data bit on the Kodak 68 pin digital video port is bit 0 and the least significant is bit 15.

Signal Name			IM-ASD RS422 50 			Kodak RS422 68

Pin female					Pin Male

________________________________________________________________________

GROUND			 1						 1

422DAT0+			 2						20

422DAT1+			 3						19

422DAT2+			 4						16

422DAT3+			 5						15

422DAT4+			 6						14

422DAT5+			 7						13

422DAT6+			 8						11

422DAT7+			 9						10

422DAT8+			10						 9

422DAT9+			11						 8

422DAT10+ 			12						 7

422DAT11+			13						 6

422DAT12+			14						 5

Signal Name			IM-ASD RS422 50 			Kodak RS422 68

Pin female					Pin Male

________________________________________________________________________

422DAT13+			15						 4

422DAT14+			16						 3

422DAT15+			17						 2

422DAT0-			27						54

422DAT1-			28						53

422DAT2-			29						50

422DAT3-			30						49

422DAT4-			31						48

422DAT5-			32						47

422DAT6-			33						45

422DAT7-			34						44

422DAT8-			35						43

422DAT9-			36						42

422DAT10- 			37						41

422DAT11-			38						40

422DAT12-			39						39

422DAT13-			40						38

422DAT14-			41						37

422DAT15-			42						36

422EXPOSE+			24						30

422EXPOSE-			49						64

CLKIN+			22						29

CLKIN-			47						63

HSYNC+			19						26

HSYNC-			44						60

VSYNC+			20						25

VSYNC-			45						59