�������������������������������������������������������������������������ͻ

�         CLASSIC ABOVE BOARD Installation Problems and Solutions         �

�������������������������������������������������������������������������ͼ

This file contains information about:

Token Ring and Expanded memory conflicts

16-bit VGAs and Expanded memory conflicts

Microsoft Windows 3.0

Quarterdeck QRAM

Above Board Expanded memory conflicts in general

Notes on TESTAB and CHKMEM:

TESTAB shows every other column bad

CHKMEM shows 0k extended or counts less than you should have.

CHKMEM shows 0k expanded even though EMM is loaded.

Technical installation documents are also available through our 24-hour

FaxBack service.  If you have a touch-tone phone and a fax machine, you

can order a catalog of all Above Board documents or an individual document.

The FaxBack number is (800) 525-3019.

INSTALLING AN ABOVE BOARD WITH AN IBM TOKEN RING ADAPTER

����������������������������������������������������������

A memory conflict can occur when an Above Board is installed in

a system with an IBM Token Ring board.  The Token Ring board and

the Above Board Expanded Memory Manager (EMM) use some reserved

memory in the range from C000 to DFFF in ATs and up to EFFF in

PCs.

Follow the directions below to eliminate a reserved memory conflict.

For IBM AT or compatibles:

1.  After running Softset, edit the CONFIG.SYS and add EP=C800,CC00

to the end of the EMM.SYS command line.  For example, if your

Above Board I/O address is 258, your EMM.SYS line would be:

Device=EMM.SYS AT 258 EP=C800,CC00

This parameter assumes the Token Ring board switches are set for

the default ROM address of CC00.

2.  Set the Shared RAM address for the Token Ring to C800:

If CONFIG.SYS has the following drivers, edit CONFIG.SYS.

With DXMC0MOD.SYS, use:  Device=DXMC0MOD.SYS ,C800

OR

With DXMC1MOD.SYS, use:  Device=DXMC1MOD.SYS ,C800

(Don't add a space after the comma, otherwise the computer

will report an Invalid Early Token Passing Parameter error.)

If AUTOEXEC.BAT has TOKREUI, edit AUTOEXEC.BAT and use:

TOKREUI ,C800

If using the Banyon LAN software, use the PCCONFIG program to

change the Shared RAM address to C800.

For further information on resetting the Shared RAM address, see

your Token Ring manual.

For IBM PC, XT or compatibles:

1.  Add EP=CC00,D800 to the end of the EMM.SYS driver in the CONFIG.SYS

file.  For example, if your Above Board I/O address is 258, your

EMM.SYS line would be:

Device=EMM.SYS PC 258 EP=CC00,D800

This parameter assumes the Token Ring ROM address is set for the

default address CC00 and the Shared RAM address is set for the default

address D800.

16-BIT VIDEO BOARD COMPATIBILITY

����������������������������������������������������������

If the Above Board is set to supply all extended memory, 16-bit VGA

video boards should work with no problems.  But if the Above Board

supplies Expanded memory, there could be a memory conflict.

The problem is that both the Above Board and the video board use the

same reserved memory addresses in the range C000-DFFF.

The solution is to exclude the video board reserved memory address

on the EMM.SYS line, and set the video board to work in "8-bit" mode.

(This will not degrade or slow down the VGA if a cache is provided.)

For example, if your Above Board's I/O address is 258, your EMM.SYS

line would be:

Device=EMM.SYS AT 258 EP=C000-C7FF

ABOVE BOARD EXPANDED MEMORY CONFLICTS

����������������������������������������������������������

The Above Board Expanded Memory Manager (EMM.SYS) can conflict with

another board in the way they both use reserved memory.

EMM.SYS searches reserved memory from C000 to DFFF in an AT-type

computer and from C000 to EFFF in a PC-type computer to find unused

pages.  EMM needs at least four 16K-byte pages of contiguous memory,

but will use as many free pages as it finds.

If your computer contains a ROM that isn't active at start-up (such as

IBM Token Ring), EMM may assume the page is free for its own use and

a conflict will occur.  In this case, you must include one of the page

frame parameters on the EMM command line.

You can tell EMM where to start its page frame address, where to

end its page frame address or how to exclude a page.  The third

option leaves EMM free to manage any unused pages.  Use this

method when using Quarterdeck's QRAM, so the extra pages can be

used to load device drivers or TSRs out of conventional memory.

To specify the EMM page frame starting address, edit CONFIG.SYS and

add to the DEVICE=EMM.SYS line the parameter EXPF=D000.  Check your

Above Board documentation for other starting and ending addresses.

To exclude a page, use the EP parameter to specify the page or

range of pages that EMM should not use.  For example, if your

Above Board I/O address is 258 in an AT and you have a VGA

video card using up to C800, your EMM.SYS line would be:

Device=EMM.SYS AT 258 EP=C000-C7FF

This excludes the range from C000 up to C800.

MICROSOFT WINDOWS 3.0

����������������������������������������������������������

All Above Boards are compatible with Microsoft Windows 3.0.

For 386 enhanced mode in a 386-based machine, set your Above Board

for all extended memory.  A minimum of 2MB of memory is needed to

run Windows in this mode.  Windows 3.0 provides expanded memory

for applications that run under Windows.  You can install

Microsoft's EMM386.SYS to provide expanded memory for your

applications that need expanded memory but don't run under Windows.

For standard mode in a 286 machine, you can set your Above Board

for both extended and expanded memory if needed.  A minimum of

256K of extended memory is required to run Windows in this mode.

Be sure EMM.SYS is BEFORE HIMEM.SYS in the CONFIG.SYS.

For real mode in an 8086/8088 machine, set your Above Board for

expanded memory.  Above Boards can supply conventional memory if

needed (up to the 640k maximum).

With Microsoft's HIMEM.SYS installed, CHKMEM will report 0K-bytes

of extended memory.

Quarterdeck QRAM

����������������������������������������������������������

The QRAM manual mentions an Above Board Plus upgrade.  The Above

Board 286 is the only Above Board that needs the upgrade to take

better advantage of QRAM's abilities.

EMM requires a minimum 64K-byte page frame to supply expanded memory

for applications.  QRAM will use any extra 16K-byte pages (beyond

the 64K-byte page frame that EMM uses) to load device drivers and

TSRs out of conventional memory.  The pages QRAM uses are dedicated

to what is loaded into them.  QRAM will report "Nothing useful to do"

if there are no extra pages.

If there are no extra pages available, QRAM can use EMM's page frame.

This page frame could then be used only to load drivers or TSRs

high.  In this case, there would be no expanded memory available for

applications, so a choice must be made between having more

conventional memory and any expanded memory.

If you have problems with a program that uses expanded memory,

remove QRAM to see if the problem persists.

TESTAB SHOWS EVERY OTHER COLUMN BAD

����������������������������������������������������������

If TESTAB shows every other column bad, another board may be

conflicting with the Above Board.   Check for network boards or other

16-bit boards using reserved memory addresses that may conflict with

TESTAB.  Remove the suspected board, and run TESTAB again.

CHKMEM DOESN'T COUNT ALL OF THE EXTENDED MEMORY

����������������������������������������������������������

When certain memory-resident programs (such as SMARTDRIVE, IBMCache

and PC-Kwik) are installed, CHKMEM reports less than the total amount

of extended memory in the computer.  The difference between the total

memory in the computer and the amount that CHKMEM reports is equal

to the amount that the resident programs are using.

CHKMEM AND OS/2

����������������������������������������������������������

CHKMEM reports "no extended memory available" when run in OS/2's

compatibility box, even when the Above Board is working and

providing extended memory.

This is because OS/2 reserves all extended memory for itself, so

none is available when CHKMEM looks for extended memory.

Check your OS/2 documentation for directions on how to see the

extended memory amount available in your system.

COMPUTER LOCKS UP ON WARM BOOT

����������������������������������������������������������

Early versions of the IBM ROM BIOS can conflict with the expanded

memory manager's reserved memory address.  That BIOS expects EGA ROM

at address C000.  If the EMM driver uses address C000 for its page

frame, the computer sounds a series of beeps and locks up during warm

boot.

To work around this, boot from a DOS diskette and edit CONFIG.SYS;

add to the DEVICE=EMM.SYS line the parameter EXPF=C800.  This tells

EMM not to load where the BIOS expects EGA ROM.

EMM LOADS, BUT CHKMEM SHOWS NO EXPANDED MEMORY

����������������������������������������������������������

This problem means another device driver or program is conflicting

with EMM.  Check the CONFIG.SYS and AUTOEXEC.BAT files for other

device drivers.  Other EMM drivers, such as DEVICE=VEM.SYS for a

program called Above Disk, are incompatible with EMM.

"EXTENDER CARD SWITCHES DON'T MATCH MEMORY SIZE"

����������������������������������������������������������

This message can mean the Above Board has an I/O address conflict

with VDISK.

If you use VDISK and the Above Board Plus, set the Above Board I/O

address to ANYTHING BUT 218.  Otherwise, VDISK won't install.

Instead, it will display the message, "Extender card switches do

not match memory size."

VDISK uses I/O port addresses 210 and 213.  An Above Board Plus uses

these same ports if the board is set to I/O port 218 and it's using

expanded memory.

KEYBOARD LOCKUP WITH TESTAB

����������������������������������������������������������

If your computer's keyboard locks up when you run TESTAB, another

board in the computer may be using an I/O port at which TESTAB expects

an Above Board.  TESTAB searches for the Above Board at a specific set

of I/O addresses.  If another board uses one of these addresses,

TESTAB may cause the keyboard to lock up.  To avoid this problem,

either change the I/O port address of the other board or remove that

board before running TESTAB.