.po 10

.he#Turbo DOS 1.42 Z80 Update                 T-Function 38#

.fo#Programmer's Guide         5-41-#         GET COMM BAUD RATE#

Turbo DOS 1.42 Update

Prepared by Advanced Digital Corporation

________________________________________________________________

#T-Function 38     Get Comm Baud Rate#

Entry Arguments     |-------------------------------------------|

| Reg |        Description                  |

|-------------------------------------------|

|  C    = 38                                |

|  D    = channel, plus bit 7 if remote     |

|___________________________________________|

Returned Value      |-------------------------------------------|

| Reg |        Description                  |

|-------------------------------------------|

|  A    = baud rate code (bits 3-0)         |

|         0 for   50 baud  8 for  1800 baud |

|         1 for   75 baud  9 for  2400 baud |

|         2 for  110 baud  A for  2400 baud |

|         3 for  134.5     B for  3600 baud |

|         4 for  150 baud  C for  4800 baud |

|         5 for  300 baud  D for  7200 baud |

|         6 for  600 baud  E for  9600 baud |

|         7 for 1200 baud  F for 19200 baud |

|                                           |

|         bit 7 = 1 if attention dection on |

|         bit 6 = 1 if 'CTS' handshake on   |

|         bit 5 = 1 if input is disabled    |

| (ADC)   bit 4 = 1 if Xon/Xoff Protocol on |

|___________________________________________|

Explanatio�        The Get Comm Baud Rate function interrogates

the current baud rate and options for the

specifie䠠 com� channe� an� return� thi� �

information in the A register.

Note�� Th� valu� returne� i� softwar� statu� �

and does not reflect the actual hardware baud

rate code or the signal sense of the option

bits.

#############################################################################################################