Page 1

GEARGEN  Copyright 1995  ATS Software

Version 1.1x  USER MANUAL

Table of Contents

Page

Legal Notice . . . . . . . . . . . . . . . . 2

Introduction . . . . . . . . . . . . . . . . 3

Screen Command Buttons . . . . . . . . . . . 4

Cavity Expansion Setting . . . . . . . . . . 4

Electrode Overburn Setting . . . . . . . . . 4

Menus and Data Box Reference . . . . . . . . 5

Tutorial . . . . . . . . . . . . . . . . . . 8

Wire list  . . . . . . . . . . . . . . . . . 10

Cut Plot . . . . . . . . . . . . . . . . . . 11

ANSI and ASA Gear Specifications . . . . . . 12

Page 2

------------------- Legal Notice -------------------------------

ATS SOFTWARE DISCLAIMS ALL WARRANTIES RELATING TO THIS

SOFTWARE, WHETHER EXPRESS OR IMPLIED, INCLUDING BUT NOT

LIMITED TO ANY IMPLIED WARRENTIES OF MERCHANTABILITY AND

FITNESS FOR A PARTICULAR PURPOSE.  NEITHER STEVEN C. AUSTIN

OR ANYONE ELSE WHO HAS BEEN INVOLVED IN THE CREATION, PRO-

DUCTION, OR DELIVERY OF THIS SOFTWARE SHALL BE LIABLE FOR

ANY INDIRECT, CONSEQUENTIAL, OR INCIDENTAL DAMAGES ARISING

OUT OF THE USE OR INABILITY TO USE THIS SOFTWARE EVEN IF

ATS SOFTWARE HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH

DAMAGES OR CLAIMS.  IN NO EVENT SHALL ATS SOFTWARE'S

LIABILITY FOR ANY DAMAGES EVER EXCEED THE PRICE PAID FOR

THE LICENSE TO USE THE SOFTWARE, REGARDLESS OF THE FORM OF

CLAIM.  THE PERSON USING THE SOFTWARE BEARS ALL RISK AS TO

THE QUALITY AND PERFORMANCE OF THE SOFTWARE.

Some states do not allow the exclusion of the limit of

liability for consequential or incidental damages, so the

above limitation may not apply to you.

This agreement shall be governed by the laws of the

state of New York and shall inure to the benefit of ATS

SOFTWARE and any successors, administrators, heirs, and

assigns.  Any action or proceding brought by either party

against the other arising out of or related to this agree-

ment shall be brought only in a state or federal court of

competent jurisdiction located in St. Lawrence County in

the State of New York.  The parties hereby consent to in

personam jurisdiction of said courts.

Page 3

---------------------- Introduction -----------------------------

Geargen was developed as an aid to those engaged in the

design and manufacture of gear forms.  In its original form I

used it to size electrodes in the moldmaking process.  After

playing with it for a while I realized that it could be used

as a aid for designing gears.  After playing with it some more

I realized that it could be used to generate code for wire

edms.

I took a couple of courses in Autocad and played around

with Designcad at home.  The routines I came across for gear

drawing with these programs seemed to be pretty lame.  They

would draw something that looked like a gear form but were not

even close to the mathematical reality.  I have seen some cad

drawings of gears that did look real --- from the base circle

up.  Below the base circle is another matter.  Most if not all

of the cad drawings of gears I've seen have had some kind of

arbitrary lines and radii below the base circle.  They com-

pletly ignored the effects of the trochoidal shape of a gener-

ated root radius and of the undercut in the smaller numbers of

teeth.

Geargen uses a mathematical representation of a gear

generator as the name implies.  You can readily see the effects

of changing the root radius, the pressure angle, or extending

the addenum on the gear form, especially on gears of smaller

numbers of teeth.

If you find this program to be of some use, I urge you

to send in your registration.  If there is enough response I

will continue to develope Geargen.  Some of of things I have

been thinking of include generating cycloidal gears, generating

gears from gears (useful for internal gears and pinions), gener-

ating chain sprockets, and going to Windows.  If you have any

ideas or comments I would be glad to hear them.  Send me a note

along with your registration or just send me a note.

Page 4

------------------ Screen Command Buttons -----------------------

"Max. Fillet"

-----                                                      -----

The "Max. Fillet" button will calculate the largest fillet

radius that will fit the other specifications and enter the

value into the Fillet Rad. text box of the gear chosen.

"Mold"

-----                                                      -----

The "Mold" button is a tool for the design of mold cavities

and EDM electrodes for the production of those cavities.  "Mold"

will calcuate and enter the gear data values for Gear2 as if it

were the mold cavity to produce Gear1.  "Mold" calculates the

data values for Gear3 as the EDM electrode to cut the cavity

Gear2.  The "Cavity Expansion" text box must be set for the

amount the mold must be oversize (or the shrinkage factor) for

the material and process used.  The "Electrode Overburn" text

box must be set to the overburn allowance (on the diameter) for

the electrode.

"Generate All"

-----                                                      -----

The "Generate All" button will generate all three of the

gear forms as long as the data exists.  This has the same effect

as choosing "Generate Gear" from the Run menu for all three gears.

"Calc. Values"

-----                                                      -----

The "Calc. Values" button

given:

- a value entered in "No. of teeth"

- a value entered in "Diam. Pitch"

- a value in "Press. Ang." chosen from the list

will return values, according to the ANSI and ASA gear

specifications for the pressure angle chosen, for:

- "Pitch Dia."

- "Outside Dia."

- "Root Dia."

- "Fillet Rad."

- "NCTT" (normal circular tooth thickness)

Page 5

----------------- Menus and Data Box Reference -----------------

DATA BOXES

-----                                                      -----

The pressure angle box will accept a typed in value as well

as one chosen from the list.  In this case, however, the calcu-

late button will not function and the values for the data boxes

below the button will have to be calculated and entered manually.

If the values entered are unreal (for example a fillet radius

that is too large and would not be tangent to the side and root

of the rack) an error will occur when the gear is generated.

The number of teeth does not have to be an integer.  This

allows a plot of the normal section of spiral teeth but the out-

side diameter and the root diameter will display as circular and

will not be correct.  The other data boxes will accept an entered

value up to 15 digits precision.

When the "Calc. Values" button is used, the values for the

Pitch diameter box, Outside diameter box, Root diameter box,

Fillet radius box, and NCTT (normal circular tooth thickness) box

are determined according to the standards ANSI B6.1-1968 or

ASA B6.1-1932 depending upon the pressure angle chosen from the

list box.  The formuli from these standards are included at the

end of this manual.

FILE MENU

-----                                                      -----

The "Save Data" selection (registered version only) will

prompt you for a file name and then write the information in

the data boxes as well as the wire list and the generated form

to that file.  The configuration information is also saved with

the data.

The "Load Data" selection (registered version only) will

prompt you for a file name and then load the information from

that file (if it is a valid file name) into the program.  If

Geargen is started with the file name on the command line

(registered version only) then that file is loaded into Geargen

on startup.

The "Save Configuration" selection will save the display

options (plot colors, plot line options, video setting, and

prompt setting), the generation increment, the resolution setting,

and the overburn and cavity expansion settings to a file named

gg112.cfg.  If the file exists it will be overwritten.  Geargen

looks for this file when it initializes.  The default settings

may be reset by deleting this file.

"Exit" will terminate Geargen.

Page 6

RUN MENU

-----                                                      -----

"Calculate Values" has the same function as the calculate

button on the data screen.

The "Generate Gear" selection causes the program to mathema-

ticly "hob" the gear form using the values in the data boxes.

The gear must be generated before it can be plotted or before a

wire list can be generated.

"Wire List" will generate a list of paired diameters and

distances.  This feature is intended primarily for those actually

cutting gears. See Wire list on page 10.

"Cut Plot" is another feature for cutting gears.

See Cut Plot on page 11.

PLOT MENU

-----                                                      -----

The "To Screen" selection brings up the Screen Plot Options

box.  Any gears that have been generated will be displayed in the

"Gear(s) to plot column.  If "1 space cen....." is chosen from

the "Type of screen plot" column and a wire size and distance was

highlighted in the wire list on the data screen, the option "with

wire" will display along with the generated gears.

"To File" brings up the File Plot box.  As the name implies,

this feature will output a text file.  The text file will be a

point by point representation of the gear profile.  The spacing

of the points is predetermined by the setting of the genera-

tion increment in the options menu.  To change the spacing,

first change the generation increment and then regenerate the

gear.  The resolution of the plot points is determined by the

setting of the Output Resolution box in all plot types except

DXF File plot.  For a DXF File plot the resolution of the plot

points is the maximum that the program can produce.

This version of Geargen (version 1.xx) will only plot to a

file using a gear center of X000 Y000.

The name box accepts a standard DOS filename and an optional

path.  If no path is specified the file will be output to the

directory that Geargen was started from.

Three basic types of file plots are available and are chosen

from the "Plot Format" box.  The first type of plot is an X value

and Y value, separated by a comma.  This is intended to be used

as input to a user written program.  The second type of file

plot contains G codes and prints "X" and "Y" along with its value.

This is code for operating machine tools.  The selection without

(Model) at the end will print a G-code at the begining of every

line.  The (model) selection will only print a G-code when it

changes.

Page 7

When a G-code type of plot is selected, an option box ap-

pears for choosing the way the arcs (O.D. and R.D.) are to be

plotted.  A selection of "Linear incre." will output the arc as

a series of straight lines.  A selection of "G02(G03)" outputs

the arc with the endpoint and an I and J offset to the center.

The center of all of the arcs will be the center of the gear,

which in this version of Geargen is fixed at X000 Y000.

The selection of DXF File will plot the gear form to a file

that is readable as input to most CAD programs.  The radii for

this type of plot will be output as arc segments only.

More than one plot may be done of the same gear (using dif-

ferent start points for example) by changing the name of the out-

put file.  When the program is done plotting the gear to a file

the "Cancel(Done)" button is highlighted.  This button may be

used to get back to the data screen at any time (as long as a

gear is not being plotted).

OPTIONS MENU

-----                                                      -----

"Plot Colors" sets the colors for the screen plots of the

three gears with their associated optional wires and definition

lines.

"Plot Line Options" sets the definition lines that will be

screen plotted along with a gear.

"Generation Increment" sets the spacing of the points for

both the screen plots and the file plots.  A fine pitch gear

should be generated with a relatively small setting and a course

pitch gear should be generated with a larger setting.  If a

gear is generated with a setting that is too small then an "out of

memory" error will occur.

"Plot Screen Prompt" toggles the message "press any key to

continue" on the plot screen.

"Intro Screen" (registered version only) toggles the intro

screen off when saved with the configuration file.

"Video" forces the program to display in the chosen video

mode.  Normally Geargen detects the highest mode it can use.  If

problems with the display arise, try setting to a different mode.

Page 8

----------------------- Tutorial -------------------------------

The instuctions given in this tutorial are specifically

for those with only a keyboard for input.  If you have a mouse

or a tablet it is enabled and you can obviously navigate with-

out the use of the tab and ALT key.  The instruction [enter]

means to press the enter or return key.

Lets go through the process of designing an electrode to

edm a gear cavity.  We'll do a gear with 8 teeth, a pressure

angle of 20 degrees, and a diametral pitch of 40.  Tab to the

pressure angle box of Gear 1 and use the up and down arrow keys

to select 20.  Tab to the diametral pitch box and type 40.  Tab

to the No. of teeth box and type 8.  Tab to the calculate values

button and hit [enter].  The rest of the boxes will fill with

values according to ANSI B6.1-1968.

Type ALT-O to drop down the options menu.  Select

"Generation increment" and hit [enter].  Select ".0003" from

the selection box (or enter .0003 from the keyboard) and tab to

the "Ok" button (or press ALT-O).  Hit [enter].

Type ALT-R to drop down the run menu.  Select "Generate

Gear" and "Gear 1".  The program is now generating the gear.

The amount of time this takes will vary tremendously depending

upon the equipment you are running.

Type ALT-P to drop down the plot menu and select "To

Screen".  Use the arrow keys to select any type of screen plot.

Tab over to the Gears(s) to plot column and hit the space bar

to check Gear 1.  Type ALT-P to highlight the Plot button (or

tab to it) and hit [enter].

Lets design the mold cavity and an EDM electrode to cut

that cavity.  Hit a key to get back to the data screen.  We'll

assume that the cavity should be 12% larger than our part to

allow for shrinkage.  Tab to the "Cavity Expansion" box and

type in 1.12 .  Lets assume that our electrode will have an

overburn of .005 per side.  Tab to the "Electrode Overburn" box

and type in .010 .  (Shift)Tab back to the "Mold" button and hit

[enter].  From the run menu, generate Gear 2 and then Gear 3.

Alternately, tab to the "Generate All" button and hit [enter].

Try plotting all three gears from the Plot menu.  If you change

a value in any of the boxes, that gear will have to be generated

again before it can be plotted.

Back to the data screen.  Type ALT-R to drop down the

run menu.  Select "Wire list" and Gear 3.  This is another

process that can be very time intensive if you have a slow

machine.  When the list is finished loading, scroll down through

until you get to a wire size of around .070.  Type ALT-O to drop

down the options menu and choose "Plot line options".  Tab to the

Gear 3 column and check all the boxes with the spacebar.  Type

ALT-O to move to the "Ok" button and hit [enter].  Type ALT-P and

Page 9

choose "To screen".  In the "Type of screen plot" column choose

"1 space centered on Y+ axis".  Tab to the "Gear(s) to plot" col-

umn and select Gear 3 and "with wire".  Plot it.

Lets now assume that we'll make our electrode on a wire

edm machine.  Hit a key to get back to the data screen.  Choose

"To File" from the plot menu.  In the "Output File Name" box,

type in any name.  Use a name thats easy to remember or write it

down on paper so it can be deleted later.  Tab to the "Output

Resolution" box and type in .0001 .  Tab to the "Gear to

Plot" box and choose gear 3. Tab to the "Plot Type box and pick

"1 Space".  Tab to the "Start Point" box and choose "Y+ Axis".

In the "Plot Format" box pick G01(Modal) and then pick "G02(G03)"

from the "Arc Options" box.  Tab to the "Plot" button and hit

[enter].  When the program is finished creating the file the

"Cancel(Done)" button will be highlighted.  Press [enter] to

bring up the data screen. The file you named in the "Output File

Name" box now exists as a text file in the directory that

Geargen was started from.  Exit Geargen and use any text editor

or word processing program with text capabilities to examine

the file.  Obviously, if we were actually to cut a gear we would

need to add a few lines of code.

Page 10

----------------------- Wire list ------------------------------

Let's suppose we are to cut a gear given these specifications:

Pressure angle = 14.5

Diametral pitch = 24

Number of teeth = 32

A measurement of 1.4224" over .070" wires

Make sure the generation increment (under the Options menu)

is set to .0001.  Choose 14.5 in the pressure angle box and type

the values for Diametral pitch and number of teeth into their

respective boxes.  Calculate the rest of the values with the

Calc. Values button.  From the Run menu, generate the gear and

then the wire list.  Lets see how this standard gear measures up

against the values we are given above.  To compare the wire

measurement given above to the wire list, a simple conversion is

in order.  Subtract the wire size of .070 from 1.4224 to get

1.3524 .  Divide 1.3524 by 2 to get .6762 .  This is our gear

center to wire center distance which occupies the right hand

column of the wire list.  Scroll dowm the wire list until .6762

is highlighted (in the right hand column).  The figure high-

lighted in the left hand column is the wire size for the gear

we just generated.  With a wire size of .0686, our gear seems

to be a little thick in the tooth.  Lets adjust it.

Subtract .0686 from .070 to get .0014 .  Divide .0014 by

the cosine of the pressure angle (cosine 14.5 = .968148) to get

.001446 .  Subtract .001446 from the NCTT (.0654498 - .001466

= .064004).  Enter .064004 into the NCTT box.  The wire list

will unload when you change a value in a data box.  Again, gen-

erate the gear and the wire list.  Scroll down the wire list

until .070 is highlighted in the wires column.  The right hand

column should be showing a value of .6762 which brings our gear

right to the spec. we are given above.

Now suppose that we don't have any .070 wires.  We do have

.0732 wires (in fact we have three of them but we'll only need

two).  Scroll through the wire list until .0732 is highlighted

in the left column.  This gives us .6813 for the gear center to

wire center distance.  Double .6813 and add .0732 to get 1.4358 .

If the form of the cut gear is correct, a measurement of 1.4358

over .0732 wires will give us the same gear as a measurement of

1.4224 over .070 wires.

If we can't find two wires of the same diameter we can use

two different gear center to wire center distances added togeth-

er with 1/2 of each wire diameter.  Alternately, if we are wire

rich we can step through the wire list at intervals to verify

that the pressure angle is correct.  We can also use this

technique to find the unknown pressure angle of bad gear that

needs to be replaced.

Its good practice to run a plot of the gear space with the

outside diameter (check the appropriate box in the "Plot Line

Options" under the Options menu) and the wire before a measure-

ment is physically taken.  This will show if the wire actually

protrudes above the O.D.

Page 11

----------------------- Cut Plot -------------------------------

"Cut Plot" is a utility that I use when cutting gear elec-

trodes (for ram edm) on an NC mill.  The 'trode blank is set up

on a horizontal rotary axis aligned with the X axis.  The cutter

is a single lip carbide half round.  It is ground to form a

single tooth of the hob, ie. tapered at the pressure angle and

the fillet radius ground at the intersection of the tapered side

and the bottom.  The width is left small.

The gear is cut in the same manner as any type of cam form.

Move two axis, cut in the third, move two axis, cut in the third.

In this case the two preliminary axis that are moved are the Y

axis and the rotary axis.  The cutter is moved in the Y axis by

any fixed amount.  The rotary axis is moved so that the surface

of the cylinder formed by the pitch diameter is also moved the

same fixed amount.  A cut is then taken along the X axis.  By re-

peated these steps 1 space of the gear form is generated.  If

the cutter is given a box motion for the cut (ie. an increment in

Y+, an X- movement, a double increment in Y-, an X+ movement, an

increment in Y+) then the size of the gear can be adjusted by

varying the Y increment.  The wire list is intended to be used

for this type of work.

This is how "Cut Plot" comes into play.  If I intend to step

off my Y axis and rotary axis in .005 increments, I set the gener-

ation increment (under the Options menu) to .005 and then run

"Cut Plot".  I then count the number of dots on the screen ignor-

ing the bottom dot.  Multiplying this number by .005 gives me the

Y distance from center to start my cam cutting.  In this way I'm

not spending a lot of time finishing material that will be cut

away later.

Page 12

ANSI  B6.1-1968   20 DEGREE INVOLUTE FULL-DEPTH TEETH

25 DEGREE INVOLUTE FULL-DEPTH TEETH

Diametral pitch = P

Number of teeth = N

Addendum = 1.000 / P

Dedendum = 1.250 / P

Working Depth = 2.000 / P

Clearance = .250 / P

Fillet Radius = .300 / P

Pitch Diameater = N / P

Outside Diameter = (N + 2) / P

Root Diameter = (N - 2.5) / P

NCTT = 1.5708 / P

ASA B6.1-1932   14.5 DEGREE INVOLUTE FULL-DEPTH TEETH

Diametral pitch = P

Number of teeth = N

Addendum = 1.000 / P

Dedendum = 1.157 / P

Working Depth = 2.000 / P

Clearance = .157 / P

Fillet Radius = .209 / P

Pitch Diameater = N / P

Outside Diameter = (N + 2) / P

Root Diameter = (N - 2.314) / P

NCTT = 1.5708 / P

ASA B6.1-1932    20 DEGREE INVOLUTE STUB TEETH

Diametral pitch = P

Number of teeth = N

Addendum = .800 / P

Dedendum = 1.000 / P

Working Depth = 1.600 / P

Clearance = .200 / P

Fillet Radius = .300 / P

Pitch Diameater = N / P

Outside Diameter = (N + 1.6) / P

Root Diameter = (N - 2) / P

NCTT = 1.5708 / P