RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

By Lydia Rosa

Copyright 1995,1997  by Lydia Rosa

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

CONTENTS

WHAT'S NEW IN THIS RELEASE ....................................... 1

DISCLAIMERS ...................................................... 2

INTRODUCTION ..................................................... 3

REQUIREMENTS ..................................................... 4

STARTUP .......................................................... 6

General Description .............................................. 6

THE DISPLAY ...................................................... 8

VFO Frequency.  .................................................. 8

Modulation.  ..................................................... 8

Channel #.  ...................................................... 8

VFO step value.  ................................................. 9

Priority Indicator .............................................. 10

Digit Scrolling ................................................. 10

Freq/Memory Scroll Buttons ...................................... 10

Signal Meter .................................................... 10

Attenuator ...................................................... 11

Scan Mode Buttons.  ............................................. 11

Manual Mode ................................................... 11

Scan Mode ..................................................... 12

Search Mode ................................................... 13

Load and Set Buttons ............................................ 14

Load ALL ...................................................... 15

Set  ALL ...................................................... 15

Mem->VFO ...................................................... 15

VFO->Mem ...................................................... 16

Option Buttons .................................................. 16

COMPUTER/RADIO Button ......................................... 16

VFO Select Button ............................................. 16

Priority ...................................................... 17

Banks On ...................................................... 17

Resume ........................................................ 17

Logging ....................................................... 18

Radio Selection ............................................... 18

Speed Control ................................................. 18

MEMORY  CHANNEL DATA FIELDS ..................................... 18

CHANNEL number ................................................ 19

VALID (Vld) ................................................... 19

DELAY (Dly) ................................................... 19

ATTENUATION (Att) ............................................. 19

RESUME (Rsm) .................................................. 20

FREQUENCY ..................................................... 20

MODE .......................................................... 20

COUNT ......................................................... 20

TIME .......................................................... 21

SIGNAL ........................................................ 21

SERVICE ....................................................... 21

TIME STAMP .................................................... 21

Contents  ii

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

SCAN BANK  DATA FIELDS .......................................... 21

CHANNEL number ................................................ 22

Valid ......................................................... 22

Start ......................................................... 22

STOP .......................................................... 22

SERVICE ....................................................... 23

SEARCH BANK DATA FIELDS ......................................... 24

CHANNEL number ................................................ 24

VALID ......................................................... 24

Start-Freq .................................................... 25

Stop-Freq ..................................................... 25

Incr-Freq ..................................................... 25

Mode .......................................................... 25

Service ....................................................... 26

Status Window ................................................... 26

SPECTRUM DISPLAY ................................................ 26

The Menu ........................................................ 27

File menu ....................................................... 27

Database menu ................................................... 27

Global Parms panel .............................................. 28

Priority ...................................................... 28

Delay ........................................................ 28

Resume ....................................................... 28

Radio  Parms panel .............................................. 28

Radio Name .................................................... 29

Radio Type .................................................... 29

ADDRESS ....................................................... 29

Max Channel ................................................... 29

Min Channel ..................................................... 30

Max Frequency ................................................... 30

Min Frequency ................................................... 30

Baud Rate ....................................................... 30

Com Port ........................................................ 30

Data Bits ....................................................... 30

Stop Bits ....................................................... 31

Flow Control .................................................... 31

Signal Detect ................................................... 31

Timeout ......................................................... 32

SPECIAL KEYS .................................................... 33

ENTER ......................................................... 33

tab keys ...................................................... 33

PgDn/PgUp ..................................................... 33

ctl-PgDn/ctl-PgUp ............................................. 33

alt-PgDn/ctl-PgUp ............................................. 33

DATA FILE FORMATS ............................................... 34

LOGFILE format ................................................ 34

FREQLIST.DBF .................................................. 34

data file format .............................................. 35

RS232 INTERFACE ................................................. 42

PROGRAM LIMITS AND RANGES ....................................... 43

Contents  iii

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

WHAT'S NEW IN THIS RELEASE

Version 0.1

First Beta test release

Version 0.2

Fixed problem with COM port open failure aborting program.

Fixed various ICOM modulation problems.

Changed DIGITs to FONTS instead of BITMAPS to reduce flashing.

Version 0.3

Fixed problem with Program restarting when closed from ICON.

Fixed hangs and errors caused when switching radios.

Fixed functional problems with AOR3000 code.

Added code to process commands in File (including CHAIN)

Version 0.4

Fixed various AOR8000 problems.

Fixed more bugs.

Made NT and Win95 versions separate (due to DLL differences).

WHAT'S NEW IN THIS RELEASE  1

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

DISCLAIMERS

IBM is a trademark of IBM corporation.

ICOM is a trademark of Icom incorporated.

KENWOOD is a trademark of Kenwood Corporation.

AOR     is a trademark of AOR inc.

DBASE III is a trademark of Borland International.

RADIO SHACK is a trademark of Tandy Corporation.

The author  takes no  responsibility for  any damages that  may

occur  to your radio or  computer due to  the operation  of  this

program.

This program is being distributed as SHAREWARE. This means that

voluntary  contributions  are  the  only  source of  remuneration

obtained for this work. A contribution of $25.00  gives  you  the

following:

1.  Fixes for any bugs.

2.  Schematics and instructions for the interfaces.

3.  A clear conscience.

Please mail  all contributions, problem reports  and  suggestions

to

Lydia Rosa

P.O. Box 414

Marlboro, N.Y.

12542

The author (and  the  latest versions of  RADIOCTL) can also be

contacted on the following Bulletin Board System:

Rosa's Folly BBS

(914) 236-7607

300/1200/2400/9600/14400 baud

Thank you for your support.

DISCLAIMERS  2

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

INTRODUCTION

The  RADIO REMOTE CONTROL PROGRAM for Windows will allow you to

control selected radios from any  computer  serial  port  (RS232)

using  Windows.  This program  expands  the  capabilities of your

radio  to  1000 channels and  100  programmable  scan  or  search

banks.  In addition, scanning, searching,  and logging facilities

are also provided.  Some radios will need an additional interface

to  utilize the  signal detection  facilities of this program.  A

circuit  diagram  and  build  instructions  are  provided free of

charge to all registered users.

The program allows you to select various baud rates, COM ports,

and other parameters  associated with  individual  radios. Up  to

eight  different radio  configurations can be  defined  for  easy

switching.  A LOCAL mode  is also  provided  to  run the  program

without the need for a radio connection.

Frequency information can be loaded or saved  from a text file.

An interface to a DBASE type file  is also provided for frequency

to service lookup. Most setup information is saved in a .INI file

for ease in restarting the program.

This  program  is  one  of  a  series  of programs  for  various

operating environments. A version of  RadioCtl is  available  for

DOS  and OS/2 Operating systems  as well. Data files created with

these programs  are  fully compatible with  the Windows  version.

Data files from this version can also be read by the DOS and OS/2

versions as well. The functionality of  all versions is,  for the

most part,  the same.  However,  the Windows  version  has  a few

differences from the OS/2 and Dos versions.  The differences  are

listed in the following table:

��������������������������������������������������Ŀ

� feature � DOS        �  Windows    �   OS/2      �

��������������������������������������������������Ĵ

�Display  � Text based � Gui based   � Gui Based   �

��������������������������������������������������Ĵ

�Threads  � Single     � Single      � Multi-Thread�

��������������������������������������������������Ĵ

�Channels � 1000       � 1000        � 10,000      �

��������������������������������������������������Ĵ

�language �  C         � Basic       � C++         �

����������������������������������������������������

The OS/2  version  has more functionality and features then the

others  due to the flexability  of the Operating System. However,

all provide enhancement of your radio's operation.

INTRODUCTION  3

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

REQUIREMENTS

In order to run this program you will need the following:

*    A 486 or Pentium based computer.

*    At least 8mb of memory (16mb is recommended).

*    about 2mb of free disk space (for the program and document).

*    Windows 3.1,  Windows 95, or Windows NT.

*    one of the following models of radio:

-   ICOM R-7000*

-   KENWOOD R-5000*

-   AOR3000a

-   AOR8000

-   RadioShack  radio equipped with  OPTOSCAN interface (from

OptoElectronics).

*NOTE:Although not tested,  this  program should work  with other

models of this manufacturer equipped with a serial port.

For Kenwood  and Icom Radios,  a modified interface is required

for full program functionally  (signal detection).  For all other

radios,  the manufacturer's interface will work.  However, signal

detection can be speeded up by use of the modified  interface for

all of  the radios. Plans  for this interface are made  available

free of charge for all registered users.

If you are controlling the R7000, you will need either the ICOM

CI-V interface adapter, or the modified RS232 interface.  If  you

wish  to use the signal  detection feature  of this program,  you

will need to use the modified interface.

For  the  R-5000,  you  will  need  the  IC-10  option  kit.  A

modification must be made to the R-5000  if you wish to  use  the

signal  detection  feature of this program.  Some  other  KENWOOD

radios provide  for software signal detection.  Although RADIOCTL

can  make use of this, the  hardware detection  will increase the

speed of scanning.

The  AOR3000,  and  OPTO equipped  radios already have  a RS232

interface. No modification is needed for these  radios, as signal

detection is done in  software.  However,  use  of  the  modified

signal detection will allow an increase in the speed of scanning.

In addition,  the modification instructions  include  a change to

REQUIREMENTS  4

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

the AOR3000 to allow 9600 baud  operation  (from the default 4800

baud) .

For the AOR8000, an interface is  also needed. The  OPTOLINK from

Optoelectronics has been tested with this program. This interface

allows  the use of hardware signal detection for faster scanning.

This interface will  also work with ICOM radios as well. However,

most interfaces for the AOR8000 should work with this program.

It is  highly recommended that  the modified  interface be used

for ICOM and  KENWOOD  radios.  Most  of  the  features  of  this

program  make  use  of  the  detection  of  an  available  signal

(SQUELCH).  These radios do NOT currently  have squelch detection

as  part  of  the standard interface. Although this  program will

work fine without this signal detection,  operations such as SCAN

and SEARCH will have no usability.

The interface circuit is a inexpensive, easy  to build project.

All the  parts  are readily  available at  any  electronic store.

Estimated  cost for each interface is $15. Plans and instructions

for these interfaces  are  provided  to you  when  you  become  a

registered user.

REQUIREMENTS  5

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

STARTUP

To  install the program,  Unzip all the files into a  temporary

directory and run the program SETUP from the WINDOWS RUN command.

Be sure to  install the package for  the version of  Windows that

you  will be running.  The  package  RADW3xxx (where "xxx"  is  a

version  number) is for Windows 3.1. The  package RADW5xxx is for

Windows  95,  and the package  RADWNxxx  is  the NT version.  The

install  process will allow you to  specify a  directory and copy

all the needed files to the appropriate places.

GENERAL DESCRIPTION

RADIOCTL provides the user with a flexible way of controlling a

radio. It also provides a  uniform  interface for different types

of radios. This is a high-level block diagram of the program:

��������Ŀ

|search  |---�

|banks   |   |

����������   |

|

|

|      ������Ŀ

��������Ŀ   �----->|      |   serial  ��������Ŀ

|memory  |<---------|VFO   |---------->|  radio |

|channels|--------->|      |<----------|        |

����������          ��������  �------->|        |

^  ^                      | port   ����������

|  |                      |

|  |    ���������Ŀ       |

|  �----|channel #|-------�

|       �����������

������Ŀ

|scan  |

|banks |

��������

Memory  channels  store  information  on frequency, modulation,

squelch  count, duration, and  other  important  parameters.  The

program   provides  for  a  maximum  of  1000  memory   channels.

Information  can be  directly  entered  from  the control  panel,

loaded  from  a  text file, loaded  from  the  radio's memory, or

transferred from the program's VFO.

The VFO  represents  a  single frequency and  modulation.  This

setting can  be sent to the radio, or stored in a memory channel.

The VFO can be set from  the control  panel  direct input, loaded

from a memory channel, loaded from a search bank, or set from the

radio.

STARTUP  6

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

Scan banks provide the ability to group blocks of  channels for

scanning.  Scan  banks  allow  the  user  to  define  up  to  100

different blocks of memory channels for selection.

Search banks provide groups of frequencies to search. Up to 100

different ranges of frequency, modulation and step  value can  be

defined.

The Channel # provides selection of the memory channels. It can

also be used to address the memory channels of the current radio.

Radio  control  information  is  sent  to and received from the

radio  by way  of  the  serial  port.  The program  provides  for

selection of up to eight different radios, each with a unique set

of  operation  characteristics.  The  program  can  also  provide

simulation  of a  radio.  The  actual  commands  to the radio are

defined  by  a  protocol setting.  The  program  current supports

protocols for KENWOOD, ICOM, AOR8000, AOR3000, and OPTO  equipped

radios. A LOCAL protocol is also  provided which does not require

a radio to be attached.

STARTUP  7

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

THE DISPLAY

The control panel shows you the status of  your  radio, as well

as the status  of the program.  There  are numerous controls, and

various status  fields. Controls  are toggled by moving the mouse

cursor over the control  and  clicking.  Entry  fields  require a

ENTER or TAB key to complete the entry.  There is also a separate

window  for Messages  from  the  program,  as  well as a Spectrum

display window.

VFO FREQUENCY.

This  display shows the current frequency of the program.  This

frequency  may or may not reflect the VFO state  of  the attached

radio. Any frequency information sent to the radio  is  displayed

in this field. The value of  this field can be  changed using the

scroll buttons to the  left of the display or  by using the mouse

buttons  when the  mouse  pointer  is over  the  digit displayed.

Clicking the  left  button  will  increase  a  digit  by  one.  A

frequency value can also be entered in the entry  field under the

display. If a  value is entered, changed with mouse  buttons,  or

scrolled with the VFO scroll bar, it is also sent to the radio.

MODULATION.

This is a combination display and  selection box. It shows  the

current  modulation used  by the program. This  may  or  may  not

reflect the  state  of the radio.  A  modulation  can  be set  by

pressing the  drop-down  control on  the  right of  the box,  and

double-clicking  on  the desired modulation.  When selected,  the

radio's modulation will be set.

CHANNEL #.

This display shows the current channel number being used by the

program. Depending on the state of the COMPUTER/RADIO switch,  it

will represent  the internal memory  channels  in the program, or

the radio's  memory  channels. The memory channel  can be changed

using  the  scroll buttons to the left of the display, using left

mouse button  on a  digit to  increment, or  entering  a  channel

number in the  entry  field  under  the display.  When  a  memory

channel  number is changed, and the COMPUTER/RADIO  switch is set

to   COMPUTER,   the  corresponding   computer   memory   channel

THE DISPLAY  8

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

information  is  set  to  the  VFO,  and  to  the radio.  If  the

COMPUTER/RADIO switch  is in  the  RADIO  position,  the  radio's

memory channel is changed.  If the program is in MANUAL mode, and

the MEM SYNC switch is selected, changing the channel number will

display the corresponding Memory Channel information.

NOTE:The AOR8000 is organized into 20 banks of 50 channels.  Each

bank is identified as a lower or upper case letter. RADIOCTL maps

these channels as follows:

��������������������������������������������������������Ŀ

� radio   �   computer       �    radio    �   computer  �

��������������������������������������������������������Ĵ

�A00-A49  � 000 - 049        �    a00-a49  � 050 - 099   �

��������������������������������������������������������Ĵ

�B00-B49  � 100 - 149        �    b00-b49  � 150 - 199   �

��������������������������������������������������������Ĵ

�C00-C49  � 200 - 249        �    c00-c49  � 250 - 299   �

��������������������������������������������������������Ĵ

�D00-D49  � 300 - 349        �    d00-d49  � 350 - 399   �

��������������������������������������������������������Ĵ

�E00-E49  � 400 - 449        �    e00-e49  � 450 - 499   �

��������������������������������������������������������Ĵ

�F00-F49  � 500 - 549        �    f00-f49  � 550 - 599   �

��������������������������������������������������������Ĵ

�G00-G49  � 600 - 649        �    g00-g49  � 650 - 699   �

��������������������������������������������������������Ĵ

�H00-H49  � 700 - 749        �    h00-h49  � 750 - 799   �

��������������������������������������������������������Ĵ

�I00-I49  � 800 - 849        �    i00-i49  � 850 - 899   �

��������������������������������������������������������Ĵ

�J00-J49  � 900 - 849        �    j00-j49  � 950 - 999   �

����������������������������������������������������������

VFO STEP VALUE.

This display  shows  the  current  step value that is used for

Search mode. This value will be added  to or subtracted from  the

VFO frequency to obtain the next VFO frequency. This value can be

set using the left  (increment)  mouse button, or direct entry in

the  field under  the display.  During  a bank search, this field

will be loaded from the selected bank.

THE DISPLAY  9

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

PRIORITY INDICATOR

When  the  unit  is  scanning,  and  the  PRIORITY  button  is

selected, a specific channel is  defined as the priority channel.

When  this channel is being scanned, this channel  number will be

highligted in red. You can change the  priority channel to be any

channel.

NOTE:The VALID flag is ignored for the priority channel. Be sure

that the selected channel has valid values for the radio.

DIGIT SCROLLING

The  VFO Step  Value, VFO frequency, and Channel # displays can

be changed using the left  mouse  button  to increment the digit.

The  mouse pointer  selects  the  digit to  change,  and the left

button will increment that digit.  This  allows quick changing of

any of the information in the display.

FREQ/MEMORY SCROLL BUTTONS

Both frequency and memory  channel number can be incremented or

decremented  using  the  UP/DN  buttons  next  the  the  affected

display.  For the Frequency,  the increment  or  decrement  value

will  be  the current VFO  STEP setting.  For the memory  channel

number, increment or decrement value is always 1.

SIGNAL METER

The signal is a 12 step indicator corresponding  to the  signal

strength  being detected. For ICOM and KENWOOD radios, the signal

indicated will always  be 9. This is due to  the  fact  that  the

signal detection  is  a  binary (On or OFF) line, with no way  to

read  the actual strength.  For  all  other  radios,  the  actual

strength detected by the radio  will  be  displayed.  For a LOCAL

radio, clicking any of the signal display  buttons  will turn  on

the signal at  that strength. For all radios, the OFF button will

temporarily turn  off the signal. If scanning or searching,  this

will cause  the  radio  to  resume  scan  or  search.  For  LOCAL

operation, the OFF button is the only way  to turn off the signal

detection.

When not running in LOCAL mode, the signal buttons can be  used

to define  a minimum  squelch trigger. Clicking  on a button will

THE DISPLAY  10

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

cause the program  to  ignore any signal at or  below that signal

level. When the squelch minimum is active, the signal limit level

buttons are displayed in green.

NOTE:This feature is not useful for the ICOM and KENWOOD radios,

as  these  radios do  not have  the  ability to  determine signal

strength values.

ATTENUATOR

The AOR3000 and AOR8000 have a built in attenuator. This can be

switched  on to reduce overload from  a strong  signal.  Pressing

this button will toggle the current  attenuator  setting  in  the

radio. When  in SCAN  mode,  this  button  will  be  set  by  the

currently scanning memory channel's attenuator setting.

SCAN MODE BUTTONS.

These buttons select the scanning mode. The program runs in one

of three scanning modes. MANUAL, SCAN and SEARCH  modes determine

the primary operation of the program. MANUAL mode is the default.

Manual Mode

Of three  scanning modes. MANUAL mode  is the default. In  this

mode, the program will continuously monitor  the  signal strength

from the radio. For the  ICOM, changes  in the radio frequency or

modulation will be logged in the VFO displays.

The program will switch  to manual mode if one of the following

conditions is met:

*   The MANUAL button is pressed.

*   SCAN mode detects no valid channels or banks.

*   SEARCH mode detects no valid  banks

THE DISPLAY  11

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

Scan Mode

The  SCAN button starts  the program scanning through  selected

memory channels. Memory channels that are  flagged with the VALID

field will  be scanned. Channels with the VALID  flag off will be

bypassed. When a signal  is  detected,  scanning  will  stop, the

signal  strength will  be  displayed  in the  Signal  Meter,  and

information (duration,  squelch count, etc) will be logged in the

memory channel.  If the MEM  SYNC switch  is selected, the Memory

Channel display will change to the Memory Channel number  that is

being logged.  When  the  signal stops,  scanning will  continue.

Memory  channels are normally  incremented.  The direction of the

scan can be changed by pressing  the CHANNEL NUMBER scroll button

to the side of the channel display.

Several  parameters  and  memory  channel  settings  affect the

operation of SCAN. Each  channel has a VALID flag associated with

it.  If this  flag is set off, the  corresponding  memory channel

will  NOT be  scanned. If  the  DELAY field of  the corresponding

memory channel is  set, the  program will  delay for a number  of

seconds  after the signal stops before  resuming  scan. The DELAY

value can be set from the PARMS panel.

The SPEED CONTROL setting determines how fast each channel will

be scanned. In  the event that a  scan is too fast to respond  to

certain frequencies,  it can be  slowed  by  adjusting  the SPEED

CONTROL.  Setting the SPEED  CONTROL to a lower  value causes the

computer to wait on each channel when there is no signal present.

This gives the radio time for the squelch to detect a signal.

The  RESUME  switch determines the  action  after  the  squelch

breaks. If RESUME  is off,  scanning  will not continue until the

signal  ends. If RESUME  is on, scanning will  resume  after  the

number of seconds  specified by  the RESUME value. This value can

be set from the PARMS panel.

NOTE:The  RESUME  switch  overrides  any  RESUME  setting for an

individual memory channels.

If a channel  is entered  in the  PRIORITY channel,  it will be

scanned  every interval specified by  the  priority value if  the

PRIORITY switch is set.  If a  signal is detected on the priority

channel, normal  scanning stops, and  the program switches to the

priority channel. Normal scanning will resume when  the signal is

no longer  present  on  the  PRIORITY  channel.  The interval for

priority checking is set on the PARMS panel.

Note:The RESUME counter does not  function  when  stopped on the

priority  channel. Scan  or Search  will  resume  from a priority

channel ONLY when the signal disappears.

If BANKS ON button is on, the  order  of  channel scan will  be

determined by the START  and END of the current bank. The current

bank is scanned from  0 to 99.  When  BANKS  ON is selected, each

valid bank will determine starting and  ending channels for scan.

Channel numbers  will  be incremented for a selected bank  if the

START  channel  number is less  than  the  STOP  channel  number.

Otherwise, channel numbers will be  decremented.  Failure to find

THE DISPLAY  12

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

a valid bank, or a  valid channel  for  the specified banks  will

force MANUAL mode.

The  MEMORY  SYNC  button  allows the Memory Channel display to

follow active channels. If this  button  is selected for  SCAN or

SEARCH mode, the displayed Memory Channel information will change

to any channel  where  a  signal is detected.  This allows you to

automatically  display  the   channel  information  whenever  the

squelch breaks.  If  this button is off,  then the memory channel

display will NOT change when a signal is detected.

The   COMPUTER/RADIO  switch  determines  the  source   of  the

frequency  information  for the radio. If COMPUTER  is  selected,

frequency  and  modulation information  is sent to the radio from

the computer's memory channels. If RADIO is set, then the program

will change  the  radio's  memory  channel  to  correspond to the

channel  being sent.  This  allows  use  of  the  radio's  memory

instead of the information in the computer. If RADIO is selected,

only  the channel numbers supported by the selected radio will be

allowed. The maximum  channel  number  can  be changed  using the

PARMS panel.

NOTE.The  OPTO protocol does  not  provide  for  memory  channel

control. When using the OPTO protocol, this switch remains in the

COMPUTER position, and cannot be changed.

Search Mode

Search  Mode allows  you  to  scan  a range of frequencies  for

action.  Up to 100 different ranges can be programmed. Each range

consists  of a starting frequency, stopping  frequency, increment

value and modulation.  In addition,  searching can  start  at the

current VFO  frequency and be incremented or  decremented  by the

current  STEP value. For  non-bank  processing, the program  will

normally increment the frequency. Pressing VFO UP/DN buttons will

change the search direction.

When  a  range  is  searched,  the radio is set to  the initial

frequency and modulation.  If a signal is detected, the frequency

and and modulation are recorded in the next  free log record, and

the  VALID  flag is set for  this  record.  If this  frequency is

already recorded in the  log, then COUNT,  TIME and SIGNAL values

are updated. If  the log entry has the VALID flag off, the signal

is ignored.  If  there are no more free channels  (frequency = 0)

then the message "SEARCH FULL" is displayed, and  the information

is NOT saved.

Several  parameters  and  memory  channel  settings affect  the

operation of SEARCH. Each  channel  has a  VALID  flag associated

with  it. If  this  flag is set off, the corresponding  frequency

will NOT respond to a squelch break.  If the DELAY field of the a

memory channel is set, and the corresponding frequency breaks the

squelch, the program will delay for a number of seconds after the

signal stops before resuming scan. The  DELAY  value  can  be set

THE DISPLAY  13

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

from the PARMS panel.

The SPEED CONTROL setting determines  how fast  each  frequency

will be  monitored. In the event that  a  search  is too fast  to

respond to certain frequencies, it can be slowed by adjusting the

SPEED CONTROL.  Setting the SPEED CONTROL to a lower value causes

the computer  to  wait on each frequency  when there is no signal

present. This gives the radio  time for  the squelch  to detect a

signal.

The  RESUME  switch  determines  the action after  the  squelch

breaks. If  RESUME is off, searching will not continue until  the

signal  ends. If  RESUME is on, searching will resume  after  the

number of seconds specified by the RESUME value. This  value  can

be set from the PARMS panel.

NOTE:The  RESUME  switch  overrides  any RESUME  setting  for an

individual memory channels.

If  BANKS ON button  is on, the order of frequency scan will be

determined by the START and END of the current bank.  When  BANKS

ON  is selected, each  valid  bank  will determine  starting  and

ending  frequency  ,  increment  and  modulation.  If  the  START

frequency  is less  than the  STOP frequency, then the  frequency

will  be   incremented.   Otherwise,   the   frequency   will  be

decremented.  Failure  to  find  a  valid bank will force  MANUAL

mode.

The MEM SYNC button allows the Memory Channel display to follow

active channels. If this button is selected, the displayed Memory

Channel information  will change to any channel where a signal is

detected.  This  allows you to automatically display  the channel

information  whenever the squelch breaks.  If this button is off,

then the  memory channel display will NOT change when a signal is

detected.

The PRIORITY  button does  NOT have any effect on  Search Mode.

Priority is not supported during  a search  operation.  Likewise,

the  COMPUTER/RADIO switch  is also ignored.  Search Mode  always

uses the computer's VFO.

LOAD AND SET BUTTONS

Four buttons  provide load and set operations. These operations

transfer  information between  the VFO, memory  channels  and the

computer. When one of these buttons  is pressed, the current scan

mode is suspended,  and the operation is started.  Selecting  the

MANUAL  scan   button  before  the  operation  is  complete  will

terminate the  operation.

THE DISPLAY  14

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

Load ALL

When  pressed, all of the  radio's memory  channels are  loaded

into the computer's memory channels. The load starts at the first

channel  in  the  radio (0  or  1) and  continues to the  maximum

channel. The maximum channel can be  set on  the PARMS panel. The

contents of computer memory channels are overwritten with the new

information. Any  memory  channel  with  a non-zero frequency  is

marked  as  valid. The count,  duration  and  strength fields are

reset to  0.  The  service string  is also cleared.  For  a LOCAL

radio, a set of random frequency and modulations is set for  each

channel.

Note:  The OPTO protocol  does not  provide  for  memory  channel

control. When using the OPTO protocol, this switch is disabled.

Set  ALL

When pressed, the contents of the  computer memory channels are

stored into the radio's memory channels. The  set  starts at  the

first  channel in the radio (0 or 1) and continues to the maximum

channel. The maximum channel can  be set on the PARMS panel.  The

contents of radio  memory  channels  are overwritten with the new

information. For  ICOM and KENWOOD  radios, a zero frequency will

cause  the memory channel to be cleared. For AOR3000 radios, zero

frequency channels will NOT  be set.  For LOCAL mode, each memory

channel will be transferred to the VFO in turn.

NOTE.The  OPTO  protocol  does  not  provide  for memory channel

control. When using the OPTO protocol, this switch is disabled.

Mem->VFO

This  button  will transfer the contents of the Memory  Channel

indicated  by  the  MEMORY  #  display,  to  the  VFO.   If   the

COMPUTER/RADIO switch is in the COMPUTER position, the Computer's

memory channel will  be  transferred.  If  the  RADIO position is

selected, the information will be transferred  from  the  radio's

memory. For a LOCAL radio, the input will  be  a random frequency

and modulation. This  operation is the same as a Load All for one

channel.

THE DISPLAY  15

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

VFO->Mem

This button will transfer the contents of the VFO frequency and

modulation  to the  Memory  Channel  indicated by  the  MEMORY  #

display.  If  the  COMPUTER/RADIO  switch   is  in  the  COMPUTER

position, the  Computer's memory channel will be updated.  Count,

Strength,  and Duration fields  are reset. The  Service string is

also cleared.  If the RADIO position is selected, the information

will be  transferred to the radio's memory. For a LOCAL radio, no

data  is transferred.  This operation  is the same as a Save  All

for one channel.

OPTION BUTTONS

Under the Channel # display is a column of  buttons that affect

various operations. These buttons determine  what memory is used,

priority, resume,  bank selection,  acknowledgment  and the radio

being used. Each of  these buttons is a  toggle  switch. Move the

mouse cursor over the button and left click to toggle on or off.

COMPUTER/RADIO Button

For  various  Scan  modes and operations, the  program needs to

determine  who's memory  is  the  source  or  destination of  the

operation.  When in the  COMPUTER position, the program is  using

the  internal  Memory Channels for  the operation.  If the  RADIO

position  is  selected,  the  operation will  affect  the  memory

channels in the selected radio.

Note:  The  OPTO protocol  does not  provide  for memory  channel

control. When using the OPTO protocol, this switch is disabled.

VFO Select Button

Below the Frequency display is a button used to select a VFO (A

or B)  to  be  used in the selected radio.  Some radios  have two

selectable frequency memories (VFO). This button determines which

VFO will be selected when setting the frequency on the radio.  If

the radio that is currently connected  does not support two VFOs,

then this button will not have any effect.

The MEM SYNC button allows the Memory Channel display to follow

THE DISPLAY  16

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

active channels. If this button is selected  for SCAN  or  SEARCH

mode, the displayed Memory Channel information will change to any

channel  where   a  signal  is  detected.   This  allows  you  to

automatically  display   the  channel  information  whenever  the

squelch  breaks.  If  the  program is in MANUAL  mode,  then  the

memory channel  display  will  change  in sync  with  the  memory

channel number.

Priority

Priority scanning allows  a selected channel to be tested for a

signal once every interval. The  channel number  to  select  as a

Priority channel is  entered in the entry field in the Operations

Area.  If the  Priority switch is ON, then  the  priority channel

will be used.  The  priority  checking interval can be set  using

the PARMS panel. Priority only works during SCAN mode.

Banks On

When in SCAN or SEARCH mode,  the program can make use of banks

of scan or search information.  These banks allow the grouping of

channels or  frequencies for the SCAN mode. The  Banks On  switch

allows the scan  mode  to  use the bank associated with  the SCAN

mode.  When  selected,  the  indicator  next  to  the switch will

display what bank number (00-99) is currently being used.

Resume

The  Resume  switch will  allow  the Scan or Search to continue

when a  signal is present. This is useful when a  Scan  or Search

encounters a  constant  signal.  With  Resume  on, the  signal is

ignored  after a  set amount of  time. This  time interval is set

using the Parms panel.  The  indicator next to the Resume  switch

shows   approximately  how  many  seconds  are   left  until  the

SCAN/SEARCH resumes.

Note:This  switch is a global setting. If on, the  RESUME switch

for each memory channel is ignored.

THE DISPLAY  17

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

Logging

When this  switch is selected, all actions are logged to a data

file. This includes mode changes and squelch breaks. A time stamp

is also logged with each event. This file is  cumulative,  and is

not erased by  this program. The  name of the logging file can be

changed with the FILE menu selection.

Radio Selection

Up  to  eight different  radios can be defined to  the program.

These radios can have different BAUD rates, Com ports, protocols,

etc. Each radio is assigned a name. You can change the  name (and

other parameters) for a  given radio from the  Parms panel. Radio

Selection is a  drop-down  selection field.  Click  on  the right

button to  display the radio list.  Double  click  on  a radio to

select it. The Baud and  Com port used will be displayed  next to

the Radio Selection control.  NOTE: When a new radio is selected,

Manual mode is forced.

Speed Control

For SCAN or SEARCH mode, the speed control determines the time

between  each channel or frequency. When the  control is  all the

way to the left, the time between each setting is minimal. If the

slider  is moved  to  the  right, the program will  delay  for  a

certain time  before changing the  setting  on  the radio. During

this  time,  the program will monitor the radio for a signal. The

SPEED  control  allows you to  reduce the SCAN or SEARCH speed to

give the radio more time to respond to a signal.

MEMORY  CHANNEL DATA FIELDS

Program  Memory channels contain  information  for  a  specific

radio setting.  Each  channel  contains information on Frequency,

Modulation  and attenuation to be  sent to (or received from) the

radio. In addition,  information on  signal  strength,  number of

squelch breaks and the frequency assignment are also displayed.

Each  program channel displayed  has  several fields associated

with it. Some of these  fields are entry fields that  information

can be  entered.  Other  fields  are toggle  switches that can be

turned on and off.  Note  that  entry field  information  is  NOT

THE DISPLAY  18

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

accepted  until ENTER  is  pressed, a  TAB key  is pressed, a new

entry field is selected with the mouse, or the channel display is

changed with the scroll bars.

CHANNEL number

This  is  the  first field on the  display. It determines which

memory  channel   is  being  displayed  or  changed.  The  memory

channel can  be  changed by direct entry in the channel field, or

using  the  scroll bars to  the left of the  channel number.  The

PgUp and PgDn  keys will  also scroll the memory  channel  up and

down.  If entering the channel number, keep in mind that the data

will NOT be entered unless you press ENTER or a TAB key.

VALID (Vld)

The  VALID field determines whether or not SCAN and SEARCH will

use the frequency currently entered into that  channel.  For SCAN

mode, the program will not scan the memory channel if VLD is OFF.

For the SEARCH operation, a frequency entered  in a memory record

that has the VALID flag off will NOT cause the search to stop and

log.

DELAY (Dly)

The DELAY  field is use  during the SCAN and SEARCH operations.

If delay  is set ON, scanning  will continue DELAY seconds  after

the signal  ends  on that channel.  If delay  is  set  OFF,  then

scanning  will continue  immediately after the  signal ends.  The

DELAY time value can be set on the PARMS panel.

ATTENUATION (Att)

The  ATTENUATION field  selects the  attenuator of  the  radio.

This is a circuit that reduces the signal strength so  that  high

level signals do not  overload the radio. The AOR3000 and AOR8000

radios  support this feature.  This  option  has  no  effect  for

THE DISPLAY  19

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

KENWOOD and ICOM radios.

RESUME (Rsm)

The RESUME field selects the RESUME function for the individual

memory channel. If this switch  is  on, and a signal is  detected

for this memory  channel in SEARCH or  SCAN mode, SEARCH or  SCAN

will continue  after a set number of second if the signal has not

gone away. This works the same as the global RESUME switch except

that it only affects the single channel.

FREQUENCY

The FREQUENCY field is used to record or set the current memory

channel frequency.  Frequency  information  is  entered  in  Mhz.

Note that INS and DEL keys do not work when editing the FREQUENCY

field.

MODE

The  MODE field represents  the  modulation  being  used  for a

specific frequency. This  field is  a drop down selection box. To

select a modulation, click on the button to the right of the box.

Double  click  on  the desired  modulation.  Note  that  not  all

modulations will be accepted by your particular radio.

COUNT

This field displays the number  of times  a signal was detected

on  a specific channel. This  field  cannot  be edited.  However,

left-clicking on this field with the  mouse cursor will reset the

field to 0.

THE DISPLAY  20

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

TIME

This field displays the average number of seconds that a signal

was  detected on a specific channel. This field cannot be edited.

However, left-clicking  on this field  with the mouse cursor will

reset the field to 0.

SIGNAL

The  signal  strength of the last  recorded signal is  reported

here.  This  field cannot be  edited.  However,  left-clicking on

this field with the mouse cursor will reset the field to 0.

SERVICE

This  field  is  used  to  record  the  service for  a specific

channel.  This  is a  scratch pad location to  record information

about  a  specific  frequency.  If  the  squelch breaks  on  this

channel during  SCAN or SEARCH mode,  the program will attempt to

lookup the service for the frequency entered. If the frequency is

found in the current frequency database, and the SERVICE field is

blank, then it will be  filled in  with  the service found in the

database.

TIME STAMP

When  ever  a  squelch break occurs for  this channel, the time

stamp is updated with the time and  date of the occurrence.  This

field  cannot be  edited.  However,  left-clicking on  this field

with the mouse cursor will reset the field to 0.

SCAN BANK  DATA FIELDS

Scan Bank entry  allows you  to  define banks of  channels  for

scanning.  RADIOCTL allows you to assign any range of channels to

various banks. Up to 100  banks (0-99) can be assigned. Banks can

overlap, and can be  turned on or  off  selectively. In order for

SCAN mode to recognize banks, the Banks On button must be active.

The bank displayed has several fields associated with it.

THE DISPLAY  21

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

CHANNEL number

This  is  the first  field  on the display. It determines which

scan bank  is  being  displayed or changed.  The  range  for this

channel number is 0 to 99.  The Channel number  can be changed by

direct  entry in the channel  field, or  using the scroll bars to

the  left  of the channel number.  The shift-PgUp and  shift-PgDn

keys will  also  scroll  the  memory  channel  up  and  down.  If

entering the channel number, keep in mind that the  data will NOT

be entered unless you press ENTER or a TAB key.

Valid

The VALID field determines  whether  or not SCAN  will use this

bank.  Scanning banks  without the  VALID flag will be ignored at

all times.

Start

START  identifies  the first channel to start scanning with for

the given bank. When this bank is active, START will be the first

channel  scanned  if  the  channel's VALID flag  is  set. If  the

channel's VALID flag is NOT set,  then  the first channel  to  be

scanned will be the next sequential channel with a VALID flag.

STOP

STOP  identifies  the  last  channel to scan when the  bank  is

active.  After  this channel  is  scanned, scanning will continue

with the START channel of the next VALID scan bank.

If  the STOP channel is set  lower than the START channel,  the

SCAN will decrement  channel  numbers.  If STOP  is  greater than

START, channel  numbers  will  increment. If STOP  and  START are

equal, the single  channel  number  in  START  and  END  will  be

scanned.

THE DISPLAY  22

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

SERVICE

This field is provided to  enter  comments  about a given bank.

It is not used or modified by any program function.

THE DISPLAY  23

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

SEARCH BANK DATA FIELDS

The search bank  panel  allows you  to define banks of channels

for  searching.  RADIOCTL  allows  you  to  assign any  range  of

frequencies,  along  with  a  modulation  and   step   value  for

searching.  Up  to  100  search banks (0-99) can be  defined.  In

order  for SEARCH  mode  to recognize  banks, the Banks On button

must be active.  The bank displayed has several fields associated

with it.

When  searching,  the program will start  with the first search

channel marked as valid, set the  radio  VFO to  the frequency in

the  START field and the modulation in the MODE field, and  check

for  a  signal.  If  no  signal  is  detected,  the program  will

increment  or decrement the frequency  by  the  amount in the INC

field, and repeat the process. When the  frequency  in  the  STOP

field is  reached, the program will select the  next valid search

bank and repeat the process.  When all valid search channels have

been processed, the program returns to the first search bank. For

more details on the SEARCH operation, see the description of  the

SEARCH mode.

CHANNEL number

This  is the first  field  on the display.  It determines which

search bank  is being displayed or changed.  The  range for  this

channel number is 0 to  99.  The Channel number can be changed by

direct entry in the  channel field,  or using the scroll  bars to

the left  of the channel number.  The alt-PgUp  and alt-PgDn keys

will also scroll the memory channel up and down.  If entering the

channel number, keep in mind that the  data  will NOT be accepted

unless you press ENTER or a TAB key.

VALID

The VALID field determines whether or not SEARCH  will use this

bank.  Search banks without the VALID flag will be ignored at all

times.

SEARCH BANK DATA FIELDS  24

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

Start-Freq

Start-Freq  identifies  the  first  frequency  to set  for  the

search.  Please note that the program does NOT check the range of

this value.  If it is outside the  range of the radio, the  radio

may  ignore the setting.

Stop-Freq

Stop-freq  identifies the last frequency to set for the search.

The search bank ends when the vfo frequency reaches the Stop-freq

value.  If the Stop-freq value is less than the Start-freq value,

the frequency  is decremented. If the Stop-freq  value is greater

than the Start-freq value,  the frequency is incremented.  Please

note that the program does NOT  check the range of this value. If

it is outside the range  of the  radio, the radio may  ignore the

setting.

Incr-Freq

Inc-Freq determines the  steps  for the given search bank.  The

Inc-Freq value is added to (or subtracted from) the VFO frequency

for each step. If  an  Inc-Freq value of 0 is entered, the SEARCH

mode will automatically reset the VLD flag for that bank.

Mode

The MODE  field  represents the  modulation being used  for the

search range. The radio  will be set to  this  modulation at  the

beginning  of  the search  range.  This  field  is  a  drop  down

selection box. To select a modulation, click on the button to the

right of  the box. Double  click on the desired modulation.  Note

that not  all  modulations will  be accepted  by your  particular

radio.

SEARCH BANK DATA FIELDS  25

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

Service

This field is provided  to enter  comments  about a given bank.

It is not used or modified by any program function.

STATUS WINDOW

The status window displays messages from the  program. This may

be  error messages, or  normal operation messages.  The  time and

date are included with all messages  as an indication of when the

error occurred.  Up  to 10 messages at a  time are displayed. The

most recent message is displayed in red at the top of the panel

SPECTRUM DISPLAY

The  spectrum  display  shows a graph of frequency  vs.  signal

strength.  An active channel will be shown as a red  line (if the

graph is in  the  range  of  the  active frequency).  Frequencies

representing past activity will be displayed in green.

In the lower  left hand corner,  the starting frequency will be

displayed. The ending frequency of the spectrum will be displayed

in  the  lower  right hand corner  of  the  display.  The  center

frequency of the  spectrum is  changed  by  moving the horizontal

scroll bar. The minimum start frequency  is 0 mhz and the maximum

stop   frequency   is  9999.9mhz.  The  horz  step  increment  is

determined by the spectrum resolution.  The resolution is changed

by moving the  vertical  scroll bar.  Resolution goes from 100khz

per pixel to 1khz per pixel in 1khz steps.

Tic  marks are displayed for reference. Tic marks are displayed

every 10 mhz when the resolution is  above 10khz/pixel.  When the

resolution is  10khz or below, the tic marks are displayed  every

1mhz.

SEARCH BANK DATA FIELDS  26

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

THE MENU

The menu on the top of the panel provides additional functions.

these functions provide the ability to load and save  information

to disk,  customize your radio setup, and  manipulate the  memory

channel information.  Menu items can  be selected by placing  the

mouse cursor over  the  item and pressing the right mouse button.

indicated by an  underline  under the character  in the  specific

menu  item.  For  the  primary menu, the  ALT  key  must  also be

pressed.  For  drop-down  menus,  only the key  itself is need to

select an item.

FILE MENU

The File menu provides the ability to load  or save the current

memory  channel, scan banks,  and search banks to  or from a text

file.  The  file format for  this  file is described in  the DATA

FORMAT section.  The  scan information  can be saved to or loaded

from many  different files.  When  a  file is  loaded, only those

memory channels, scan bank channels, or  search channels that are

included  in  the  file  are  changed. For  a  save,  only memory

channels with a frequency greater than zero , and scan and search

banks that are marked as valid are written.

A  new DBASE file  can  also be selected from  this panel. This

file is used for service lookup  operations. The format  of  this

file is also discussed in the DATA FORMAT section.

The  name of the logging file is also set from this menu.  This

is the file that is written to when the LOGGING switch is on.

The  File menu also contains the Quit for the program. Function

key 3 will also terminate the program.

DATABASE MENU

The Database menu  allows you to  manipulate various aspects of

the memory channels. A sort of memory channel by frequency can be

done from this  panel.  With a sorted  frequency list, the  radio

being controlled requires less settling time  for  each frequency

change, allowing for faster signal detection.

The  Service  search will  update  all  Service fields for  any

memory channel with a non-zero frequency. The service information

is obtained from the current DBASE file.

The  Clear option allows  you to clear all memory channels, all

The Menu  27

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

service fields from all memory channels, or  all  count, time and

signal logs from all memory channels.

GLOBAL PARMS PANEL

Various parameters determine the operation of the program.  For

the  most  part, these parameters  are  set  once when  you first

install  the  program.  This  panel allows  you  to  set  program

operation parameters.

Priority

This  common  panel parm  sets  the time  (in  seconds) between

priority checks.

Delay

The  number of seconds  that the program will wait on a channel

after a  signal  disappears.  This value only has  affect  if the

DELAY flag is set on the current memory channel.

Resume

If RESUME is selected, this is the number  of seconds that SCAN

or SEARCH will continue after a signal is detected.

RADIO  PARMS PANEL

The program provides  settings  for up  to  eight radios.  Each

radio can be setup with different operation characteristics.  For

the  most  part, these parameters  are  set  once when you  first

The Menu  28

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

install  the  program. However, you  may  need  to  update  these

parameters  if  you connect different radios.  Except  for  Name,

Baud rate and  Com  port,  the  defaults for the particular radio

types should only be  changed if a non-standard  radio  is  being

supported.

Radio Name

This  is the  name  that  you  give  the radio.  There  are  no

restrictions  on  the  name,  other  than  is is  limited  to  30

alpha-numeric  characters.  This  name  is  used  in  the   Radio

Selection control on the main panel.

Radio Type

This  field defines  the  type of  radio (protocol). There  are

currently six types of radios supported. ICOM,  KENWOOD, AOR3000,

AOR8000,  OPTO  and  LOCAL.  Other  radios may be  added  in  the

future.

ADDRESS

ICOM  and  OPTO  radios require an address. For the R-7000, the

address  is  normally 8. If you connect a  different ICOM or OPTO

radio, or you  have  more than one ICOM radio  connected, you can

change this address  to match. This parameter  has no affect  for

other radio types.

Max Channel

This  field  defines the maximum  memory channel in the  radio.

For the  ICOM R-7000, and KENWOOD R-5000, this would normally  be

99. For the AOR3000 this value should be 399. Changing this field

allows  the  program to support other radios with  more (or less)

memory.  This parm has no effect  for the OPTO  protocol,  as the

OPTO protocol does not support memory channels.

The Menu  29

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

MIN CHANNEL

This is the  first  memory channel for  the radio. ICOM  radios

start with memory channel  1. The AOR3000 and KENWOOD  start with

memory channel 0.  This parm has no effect for the OPTO protocol,

as the OPTO protocol does not support memory channels.

MAX FREQUENCY

This is the maximum frequency that the radio will accept.

MIN FREQUENCY

This is the minimum frequency that the radio will accept.

BAUD RATE

The baud setting for this radio. Most  radios can be configured

to accept baud rates from 1200  to 9600 baud. The default setting

for ICOM and  AOR3000 is 9600. For the  KENWOOD,  the  default is

4800 baud. Adjust this setting to match  the  baud rate that your

radio is set for.

COM PORT

This is  the com port  that your  radio  is  attached  to.  The

program allows you to select com 1 thru com 4.

DATA BITS

Most of the radios supported work with an 8 bit data format. If

you have a radio that requires 7 bits, this can be changed.

The Menu  30

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

STOP BITS

Most  of the radios supported use  2  stop bits. If needed, you

can change this to 1 stop bit.

FLOW CONTROL

Flow  Control   provides   the  ability   to  stop  and   start

communications based on the ability of the radio and  computer to

accept data.  Flow control requires the use of additional control

lines (RTS/DTS).  These lines are used as signals to indicate  to

the radio and the computer to start and stop sending information.

The ICOM  interface that  this  program is  designed to work with

does NOT provide  for RTS/CTS  control lines (flow  control). The

KENWOOD and  AOR3000 interfaces will use RTS/CTS flow control. It

is recommended that flow control be left on  for these radios, as

they may not work properly with it off.

SIGNAL DETECT

There  are three ways  to  detect a squelch break. The first is

using a spare signal on the RS-232 port. This signal detection is

part  of  the  modified  interface,  and  also provided with OPTO

equipped radios. This is referred to as HARDWARE detection.

The  second detection  method involves using  a command  to the

radio to detect squelch break. This  is referred to  as  SOFTWARE

detection.  This  method also  uses a  command to  the  radio  to

determine  the  signal  strength.  SOFTWARE   detection  is   NOT

available for the ICOM protocol.  Some  KENWOOD radios provide  a

software signal detection.  The  KENWOOD R5000 does NOT have this

feature.

The  third method  is a  combination of HARDWARE  and SOFTWARE.

With  this  technique,  the  signal line  is checked for  squelch

break, and  a signal  strength  command  is  issued  ONLY  if the

squelch break  signal has been detected. This  method is the most

efficient for scanning speed. However, it cannot be used for ICOM

radios, as there are no signal strength commands available.

Please note that  to make use of the HARDWARE, or  HARDWARE and

SOFTWARE  detect  methods,  either  the modified interface or the

OPTO interface must be present.

The Menu  31

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

TIMEOUT

All  radios  require  a  delay  between  each  packet sent.  In

general this delay should be between 60 and 90 milli-seconds.  If

you  are seeing error messages that indicate overflow or bad data

from  the radio, you should increase  this  value.  Setting  this

value  too  high will  slow  down  the  maximum  scanning  speed.

However,  setting  this value  too  low will  cause  transmission

errors.

The Menu  32

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

SPECIAL KEYS

ENTER

The ENTER key finalizes input to the  program.  For  data entry

fields, the information is stored into the computer's memory, and

the cursor advances to the next field.

tab keys

The tab keys  will move the cursor to the next field. A forward

or reverse tab key will complete data entry of the current field.

PgDn/PgUp

These keys  increment or decrement the  current  memory channel

being displayed.

ctl-PgDn/ctl-PgUp

These keys increment or decrement the current scan bank channel

being displayed.

alt-PgDn/ctl-PgUp

These  keys  increment  or  decrement  the current search  bank

channel being displayed.

SPECIAL KEYS  33

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

DATA FILE FORMATS

There are several files that this program  makes  use of. These

files are used to store and retrieve data used by the program.

LOGFILE format

When LOGGING is  enabled, certain transactions are logged to a

file. The  name  of this file  is  set by the  user.  The initial

setting for this filename is RADIOCTL.LOG.

For a given transaction, the following information is logged:

*   Timestamp- The Date and Time of the transaction.

*   The scanning mode (SCAN/SEARCH/MANUAL).

*   The VFO frequency.

*   The VFO modulation.

*   The current memory channel.

*   The signal strength at the time of transaction.

*   The length of the transaction.

*   The service information for the current memory channel.

FREQLIST.DBF

This  is  a standard  DBASE  III+  type data file. It  contains

frequencies  and allocations for  the  radio  spectrum. There are

three fields that are used by this program.

field name     allocation       description

-----------------------------------------------------------

FREQUENCY      decimal 10.5     holds the frequency in Mhz

DESCRIPT       character 50     usage of the frequency

ACTION         logical          flags the frequency in use

If  the ACTION field is not  present, the  program will use the

DATA FILE FORMATS  34

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

DESCRIPT  field  to  determine  ACTION. A  blank  DESCRIPT  field

signals an inactive channel.

The length of the DESCRIPT field can be changed, if so desired.

This program only uses the first 80 chars of this field.

Any other fields can be added or deleted from this file without

affecting  the operation of the program.  However, if  either the

FREQUENCY or DESCRIPT  field is not present in the  database, the

DBASE function will not operate.

data file format

The SAVE command creates a  file  with the  current settings of

memory channels, and various scan parameters. The file created is

an  ASCII  readable file. Each record  is  terminated  by a CR/LF

sequence.  One record represents one frequency channel, one  scan

bank channel, one search bank channel, one command, or a comment.

Which  type if  record  is  determined  by  the  first  non-blank

character  on the line.  The following table shows the identifier

for the various types of records:

��������������������������������������������Ŀ

� identifier   �  type of record             �

��������������������������������������������Ĵ

� number       � frequency channel info      �

��������������������������������������������Ĵ

� @            � scan bank information       �

��������������������������������������������Ĵ

� #            � search bank information     �

��������������������������������������������Ĵ

� keyword      � command to program          �

��������������������������������������������Ĵ

� *            � comment                     �

����������������������������������������������

Each record contains one or more fields delimited by blanks.  The

file can be  altered (or created) using any standard text editor.

The maximum  record length that  the program  can  handle  is 200

characters.   If   the  comment  record  flag  is  detected,  all

information following the '*' is  ignored on that line.  When the

program reads the file, any missing required parameter, parameter

range error, or unrecognized keyword will terminate the file read

at that line.

The format for a frequency channel entry is as follows:

DATA FILE FORMATS  35

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

���������������������������������������������������������������Ŀ

�field�  name     � value range    � description                �

���������������������������������������������������������������Ĵ

�   1 � CHANNEL   � 0-999          �channel number to set       �

�     �           �                � required-positional        �

���������������������������������������������������������������Ĵ

�   2 � FREQUENCY � 0-999.9999     �frequency in Mhz            �

�     �           �                � required-positional        �

���������������������������������������������������������������Ĵ

�   3 � MODE      � 3 chars        �valid modulation for radio  �

�     �           �                � required-positional        �

���������������������������������������������������������������Ĵ

�   4*� COUNT     � 0-999999       �count of squelch breaks     �

���������������������������������������������������������������Ĵ

�   5*� DURATION  � 0-999999       �total duration of squelch   �

���������������������������������������������������������������Ĵ

�   6*� SIGNAL    � Sxx            �Signal strength value (xx)  �

�     �           � xx=00 to 15    � optional                   �

���������������������������������������������������������������Ĵ

�   7*� VALID     � VALID/NOTVALID �indicator of channel valid  �

���������������������������������������������������������������Ĵ

�   8*� ATTEN     � ANTEN/NOANTEN  �indicator of attenuation    �

���������������������������������������������������������������Ĵ

�   9*� RESUME    � RESUME/NORESUME�indicator of resume         �

���������������������������������������������������������������Ĵ

�  10*� TSTAMP    � TSTAMP value   �squelch break time stamp    �

�     �           � TSTAMP 0       �time value is UNIX time     �

�     �           �                �(number of seconds since    �

�     �           �                � 1970) 0 = no stamp.        �

���������������������������������������������������������������Ĵ

�  11*� TONE      � PL 0 0 0       �future enhancement.         �

�     �           �                �If included, all three      �

�     �           �                �numbers must be specified.  �

�     �           �                �                            �

���������������������������������������������������������������Ĵ

�  12*� DELAY     � DELAY/NODELAY  �indicator of channel delay  �

���������������������������������������������������������������Ĵ

�  13*� SERVICE   � 80 chars       �service for channel         �

�����������������������������������������������������������������

*optional fields

The  first three fields MUST be specified and  be in the proper

order. Optional fields do NOT have to be specified.  However, the

following positional restrictions apply:,

*   If COUNT is specified, it MUST follow the MODE field.  If not

specified, the COUNT value for the channel is set to 0.

*   If DURATION  is specified, COUNT must also  be specified, and

DURATION  must  follow  the  COUNT.  If  not  specified,  the

DURATION value for the channel is set to 0.

*   Duration  value  MUST  be specified  if any  of the following

fields are specified:

DATA FILE FORMATS  36

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

SIGNAL, VALID, ANTEN, RESUME, TSTAMP, TONE, DELAY, SERVICE.

*   If SIGNAL value is not specified, then SIGNAL value  for  the

channel is set to 0.

*   If VALID  is not specified, the channel will be marked VALID.

*   If ANTEN,  RESUME  or  DELAY is not specified, the respective

flags will be reset for the channel.

*   If DELAY is specified, it MUST precede the service field.

*   If  SERVICE is specified, DELAY must also  be specified.  All

characters (up to 80) following the DELAY value and up to the

end of  the  line  (CR/LF)  are  taken  as  a SERVICE string.

Leading blanks and any special ASCII characters are removed.

NOTE:If you are reading a data file that was  created by the OS/2

version  of  RADIOCTL, the time stamp represents  Greenwich  Mean

Time  (GMT).  The  time   stamp  saved  by  the  Windows  version

represents local time.

DATA FILE FORMATS  37

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

The format for a scan bank entry is as follows:

���������������������������������������������������������������Ŀ

�field�  name     � value range    � description                �

���������������������������������������������������������������Ĵ

�   1 � BANKID    � @              �@ identifies a scan bank    �

���������������������������������������������������������������Ĵ

�   2 � BANK      � 1-100          �bank number                 �

���������������������������������������������������������������Ĵ

�   3 � START     � 0- MAX         �starting channel for bank   �

���������������������������������������������������������������Ĵ

�   4 � END       � 0- MAX         �ending   channel for bank   �

���������������������������������������������������������������Ĵ

�   5*� SCAN      � SCAN/NOSCAN    �valid bank                  �

���������������������������������������������������������������Ĵ

�   6*� DESCRIPT  � 80 chars       �description of bank         �

�����������������������������������������������������������������

*optional fields

If NOSCAN is not specified, the bank is marked as valid.

DATA FILE FORMATS  38

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

The format for a search bank entry is as follows:

���������������������������������������������������������������Ŀ

�field�  name     � value range    � description                �

���������������������������������������������������������������Ĵ

�   1 � BANKID    � #              �@ identifies a search bank  �

���������������������������������������������������������������Ĵ

�   2 � BANK      � 1-100          �bank number                 �

���������������������������������������������������������������Ĵ

�   3 � START     � 0- 9999.99999  �starting frequency for bank �

���������������������������������������������������������������Ĵ

�   4 � END       � 0- 9999.99999  �ending   frequency for bank �

���������������������������������������������������������������Ĵ

�   5 � INC       � 0- 9999.99999  �frequency increment         �

���������������������������������������������������������������Ĵ

�   5 � MODE      �AM,FM,FMN,etc.  �modulation for bank         �

���������������������������������������������������������������Ĵ

�   5*� SEARCH    � SEARCH/NOSEARCH�valid bank                  �

���������������������������������������������������������������Ĵ

�   6*� DESCRIPT  � 80 chars       �description of bank         �

�����������������������������������������������������������������

*optional fields

If NOSEARCH is not specified, the bank is marked as valid.

DATA FILE FORMATS  39

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

Certain commands can be entered in a text  file for  execution.

These commands provide the ability to execute certain  operations

automatically,  without  having  to select  them from the various

panel controls.

COMMANDS RECOGNIZED

��������������������������������������������������������������Ŀ

�command         � option(s)      � description                �

��������������������������������������������������������������Ĵ

�BANK            � ON|OFF         �BANK mode on or off.        �

�                �                �Equivalent to pressing BANK �

�                �                �button.                     �

��������������������������������������������������������������Ĵ

�CHAIN           � filespec       �start reading another text  �

�                �                �file. Maximum CHAIN depth   �

�                �                �is 3 files.                 �

��������������������������������������������������������������Ĵ

�CHANNEL         � 0 to 9999      �set channel number.         �

�                �                �Equivalent to setting       �

�                �                �CHANNEL display value.      �

��������������������������������������������������������������Ĵ

�CLEAR           �    -           �Clear all memory channels   �

�                �    ALL         �Clear all memory channels   �

�                �    COUNTS      �Clear all squelch counts    �

�                �    SERVICE     �Clear all service strings   �

�                �    SIGNAL      �Clear all strength readings �

�                �    TIME        �Clear all squelch times     �

��������������������������������������������������������������Ĵ

�DBNAME          � filespec       �change DBASE filename for   �

�                �                �service lookups.            �

��������������������������������������������������������������Ĵ

�DELAY           �    xxxx        �Set DELAY value to xxxx.    �

��������������������������������������������������������������Ĵ

�F               �  frequency     �Set VFO frequency           �

��������������������������������������������������������������Ĵ

�HALT            �     -          �Force MANUAL mode           �

��������������������������������������������������������������Ĵ

�LOAD            �     -          �load memory from radio      �

��������������������������������������������������������������Ĵ

�M               �   modulation   �set VFO modulation          �

����������������������������������������������������������������

DATA FILE FORMATS  40

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

COMMANDS RECOGNIZED (cont)

��������������������������������������������������������������Ŀ

�MANUAL          �     -          �Force MANUAL mode           �

��������������������������������������������������������������Ĵ

�MEMORY          �     -          �use radio's memory          �

��������������������������������������������������������������Ĵ

�PDELAY          �   0 to 9999    �set priority interval       �

��������������������������������������������������������������Ĵ

�QUIT            �     -          �exit RADIOCTL               �

��������������������������������������������������������������Ĵ

�PRIORITY        �     ON         �set priority switch on      �

�                �     OFF        �set priority switch off     �

�                �   0 to 999     �set priority channel        �

��������������������������������������������������������������Ĵ

�RESUME          �     ON         �set resume   switch on      �

�                �     OFF        �set resume   switch off     �

�                �   0 to 9999    �set resume  interval        �

��������������������������������������������������������������Ĵ

�SCAN            �     -          �Start SCAN mode             �

��������������������������������������������������������������Ĵ

�SEARCH          �     -          �Start SEARCH mode           �

��������������������������������������������������������������Ĵ

�SORT            �     -          �Sort memory channels        �

��������������������������������������������������������������Ĵ

�VFO             �     -          �use computer's memory       �

����������������������������������������������������������������

Note:Certain commands are processed on a deferred basis. In other

words,  they are not processed until the  entire  file  (and  any

chained files) are  read.  Because  of  this,  only one  of these

commands (the last  one read in)  will be processed. As a result,

only one of these commands  should be present in  the  file.  The

commands are:

CHANNEL, F, HALT, LOAD, M, MANUAL, SCAN, and SEARCH.

The F and M commands CAN co-exist in the same file.

Any record  that is not  recognized  as  a Frequency,  Scan bank,

Search bank, command or comment will be ignored.

DATA FILE FORMATS  41

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

RS232 INTERFACE

This program is designed to use the serial  communications port

(RS232) of the IBM  computer. In  order  to  interface a radio to

this  port,  an  interface  adapter is  needed  designed for  the

specific radio. Each radio  manufacturer  makes such an  adapter.

However,  none  of  the  adapters supplied from the  manufacturer

provide for squelch detection. Without this detection, the signal

sensing functions of this program will not work.

For  each  radio  supported,  a  circuit  and instructions  are

available.  This circuit provides the squelch detection needed by

this program.  The interface  circuits are  easy  to  build,  and

require  few parts.  All of the  parts can be obtained cheaply at

any electronics supply store. The schematics and instructions are

provided to all registered users.

RS232 INTERFACE  42

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

PROGRAM LIMITS AND RANGES

MEMORY REQUIRED:       4 meg (Windows Minimum). 8 Meg+ suggested

PROCESSOR REQUIRED:    80386 or Higher

WINDOWS LEVEL REQUIRED:3.1 or Windows 95

MAXIMUM MEMORY CHANNELS    1000

MAXIMUM SCAN BANKS          100

MAXIMUM SEARCH BANKS        100

FREQUENCY RANGE           .01khz to 9999.99999mhz*

MINIMUM FREQUENCY STEP     10hz*

MAXIMUM FREQUENCY STEP     9999.99999mhz

*may be limited by radio model.

PROGRAM LIMITS AND RANGES  43

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

INDEX

Shift PgDn 22

A                               Shift PgUp 22

AOR3000 2

attenuation 19

Attenuator 11                            I

ICOM

B                                  m 2

interface 4

Banks On 17

K

C

KENWOOD 2

Channel # 8

channel RESUME 20

COMPUTER/RADIO Button 16

COUNT 20                                 L

Load ALL 14

Load/Set Buttons 14

D                            Logging 17

DBASE 2

delay 19

Digit Scrolling 10                       M

Manual Mode 11

Mem Sync 16

E                            Mem->VFO 15

Memory Channel

ENTER key 33                          CHANNEL number 19

Memory Channels 18

Menu 26

Database 27

F                               File 27

Parms 28

Freq/Memory Scroll Buttons 10      MODE field 20

frequency 20                       modulation 20

FREQUENCY field 20

O

H

Option Buttons 16

Hot Keys

Alt PgDn 24

Alt PgUp 24

PgDn 19

PgUp 19

Index  44

RADIO REMOTE CONTROL PROGRAM FOR WINDOWS

SPEED 12

P                               VALID 12

Search Bank 23

Parms                                 Channel number 24

ADDRESS 29                         Incr-freq 25

Baud Rate 30                       Mode 25

Com Port 30                        Service 25

Data Bits 30                       Start-freq 24

Delay 28                           Stop-freq 25

Flow Control 31                    Valid 24

Max Channel 29                  Search Mode 13

Max Frequency 30                   BANKS 14

Min Channel 29                     DELAY 13

Min Frequency 30                   MEM SYNC 14

Priority 28                        RESUME 14

Radio Name 29                      SPEED 14

Radio Type 29                      VALID 13

Resume 28                       SERVICE 21

Signal Detect 31                Set ALL 15

Stop Bits 30                    shareware 2

Timeout 31                      SIGNAL 20

PgDn key 33                        Signal Meter 10

Pgup key 33                        Spectrum Display 26

Priority 17                        Speed Control 18

Priority Channel 9

T

R

tab key 33

Radio Selection 18                 TIME  20

Resume 17                          TIME STAMP 20

RS232 4

V

S

valid 19

Scan Bank                          VFO Frequency 8

Channel number 22               VFO Modulation 8

Service 22                      VFO Select Button 16

Start 22                        VFO step value 9

Stop 22                         VFO->Mem 15

Valid 22

Scan Mode 11

Scan Mode Buttons 11

MANUAL 11

SCAN 11

SEARCH 13

Scan Mode

BANKS 12

COMPUTER 13

DELAY 12

MEMORY SYNC 13

PRIORITY 12

RADIO 13

RESUME 12

Index  45