Windows 3.1 and Windows 95 Driver Installation For

Micro-Labs Ultimate TrueColor /XL Series Boards      Version 3.7

Please refer to the driver installation section that corresponds to the

version of Windows that is installed.

Windows 95 Video Driver Installation Instructions

The Tseng Labs ET4000/W32 driver that Windows 95 installs as the default

works correctly with our /XL series boards. However, by installing the

Micro-Labs driver, performance in TrueColor display modes will be improved.

Our driver also automatically sets the refresh rate to match the Monitor

Type listed in the Windows 95 Display Type screen.

Follow these steps to install the Windows 95 driver:

1) Insert the Windows 3.1, 95 Driver disk in disk drive A:

2) Open "My Computer"

3) Open "Control Panel"

4) Open "Display"

5) Choose the "Settings" Tab

6) Select "Change Display Type"

7) In the "Adapter Type" section select "Change"

8) Select "Have Disk"

9) In the "Install from Disk" window select "OK"

10) In the "Select Device" window select "OK"

11) Select "Apply" or "Close"

12) Select "Yes" to restart the computer

These steps only need to be performed one time. After that, you can change

the display resolution, color depth, or font size by making the following

series of selections from the Windows 95 desktop:

1) My Computer

2) Control Panel

3) Display

4) Settings

If the font size or number of colors is changed then Windows 95 must be

restarted for the new selection to take effect.

A desktop icon can be created to go directly to the Display program by

making the following series of selections:

1) My Computer

2) Control Panel

3) right click on Display

4) click on Create Shortcut

5) click on Yes to place the shortcut on the desktop

END of WINDOWS 95 Instructions

Windows 3.1 Video Driver Installation Instructions

This driver disk is designed to allow you to select or change the video

driver for Windows 3.1 using Windows Setup. Setup can be executed from

within Windows, or from DOS. Note that these driver files are in Microsoft

compressed format and can only be accessed and executed by installing

through Windows Setup. Simply copying the files into your \WINDOWS\SYSTEM

subdirectory WILL NOT work and may corrupt your Windows installation.

Installing the drivers correctly is, however, a simple process. It is

accomplished by following these instructions:

*** Installation From Floppy Disk ***

1) Insert Windows 3.1, 95 Driver disk in the appropriate floppy drive.

2) From your WINDOWS directory, type SETUP and select DISPLAY to change the

video driver. Or, with Windows running, select Setup from the Program

Manager Main group. Refer to your Microsoft Windows User's Guide for

assistance in running Setup from inside Windows.

3) Select "Other disk..." to direct the program to the floppy disk holding

the new drivers. Note that after the drivers are installed, you will not

have to select "Other disk..." again should you decide to change

resolutions. Look for the Ultimate VGA drivers in the Display selection

menu to select the display resolution you want.

4) When asked where the drivers will be found, state the following:

A:\

Skip down to steps 6-7 below.

*** Installation From Hard Disk ***

1) Having the drivers loaded on the hard disk will make it easier and faster

to change resolutions in the future. Make a subdirectory to hold the

driver files.

example: C:>md \win31drv

and switch to that subdirectory (C:>cd \win31drv)

2) Insert the Ultimate VGA Windows 3.1, 95 Driver disk into the appropriate

drive and type the following command:

COPY A:\*.*

3) From your WINDOWS directory, type SETUP and select DISPLAY  to change the

video driver. Or, with Windows running, select Setup from the Program

Manager Main group. Refer to your Microsoft Windows User's Guide for

assistance in running Setup from inside Windows.

4) Select "Other disk..." to direct the program to the subdirectory holding

the new drivers. Note that once the drivers are installed, you should not

have to select "Other" again should you decide to change resolutions. Look

for the Ultimate VGA drivers in the Display selection menu to select the

display resolution you want.

5) When asked where the drivers will be found, state the following

(deleting what the program has displayed in the input window):

[drive:]\[directory]     e.g.: C:\win31drv

6) A standard Windows selection menu will appear with additional video

drivers from which to select.

7) Proceed normally through the remainder of the process. Check Microsoft

Windows installation procedures if necessary.

** IMPORTANT **

Note that you must have 2MB of memory on your video board to display

1280 x 1024  with 256 colors.

Note that only the /XLp boards support 32768 or 65535 colors at 1024 x 768,

and 16.8 million colors at 800 x 600.

If your select a driver which your board can not support, a brief message

will appear in the upper left of your screen

"video driver unable to set desired mode"

before returning you back to the DOS prompt. If this occurs, simply run the

SETUP program again from DOS and choose a supported display mode.

END of WINDOWS 3.1 Instructions