NW32W95.DOC

Kingston Technology Company

INSTALLATION GUIDE

Novell Client 32 ODI driver for Windows 95

for

EtheRx PCI 10/100 Fast Ethernet Adapter (KNE110TX)

Copyright (C) 1996 Kingston Technology Company.

All rights reserved.

To use QStart under Windows 95, you need to run the program in

Windows 95's MS_DOS mode. To go to MS_DOS mode, click on 'Start', then

click on 'Shut Down'.  Select 'Restart the computer in MS_DOS mode?'.

Driver Installation Procedure:

This installation procedure assumes WIN 95 is properly installed on

your computer. You should only install the Netware Client32 software

after Windows95 driver has been loaded and functioning.

1.

Make sure your computer is turned off. Secure the PCI adapter in

an available PCI slot.

2.

Boot up Windows95/98. The system will detect the presence of the

new PCI Ethernet Hardware and ask you to insert the Manufacturer's

provided diskette. Insert the Kingston Drivers Diskette, change the

directory path to A:\ and continue.

3.

Windows will display the driver name "Kingston EtheRx PCI 10/100 Fast

Ethernet Adapter (KNE110TX)". Select the driver and continue.

4.

Follow Windows95/98 's on screen instruction to finish the

configuration.

5.

Restart you computer to finish the installation.

6.

Install Netware Client32 for Windows95 and follow on-screen instructions.

Reboot your system when finish.

7.

The installation is now complete.

After your adapter installation has been completed successfully,

you can view and use network resources in Windows 95 by clicking

on 'Network Neighborhood'.  Windows 95 will search for and then

display the network resources it finds.  See your Windows 95

documentation for more information about using and sharing

resources with Windows 95.

Kingston Technology, EtheRx, and QStart are registered

trademarks of Kingston Technology Company.  All others are

trademarks or registered trademarks of their respective holders.