BATSET        Set the batterie on HP100LX

Weasel Words:

I am not a batterie engineer. HP made it very difficult to change the fast

charge time. I doubt that was by accident. Please make your own decisions about

what you think this value should be. I just wanted to change it for me, and

thought some of my friends on CIS might like to also.

Disclaimer:

Every effort has been made to test thoroughly and verify the functions of

this program.  However, THIS PROGRAM IS PROVIDED "AS-IS" WITHOUT WARRANTY OF ANY

KIND.  THE ENTIRE RISK AS TO THE QUALITY AND PERFORMANCE OF THE PROGRAM IS WITH

YOU, SHOULD THE PROGRAM PROVE DEFECTIVE, YOU ASSUME THE ENTIRE COST OF ALL

NECESSARY SERVICEING, REPAIR OR CORRECTION. Make backup copies of your important

files often and use this program at your own risk.

Usage:

Since the setting of the battery to Alkaline, Nickel Cadmium and charging

status are reset by a hard reset, I wanted to be able to set them properly in my

AUTOEXEC.BAT file. BATSET will do these for you.

BATSET   (  /A  |  /N  |  /C  )

If you specify /A then the 100 will be set for Alkaline batteries and no

charging. /N for Nickel Cadmium and no charging. /C for Nickel Cadmium and

charging.

In addition BATSET will allow you to change the 6 hour fast charge timer on

the 100LX. When the AC adapter is plugged in or charging is specified the 100LX

will charge the batteries at a rate of 100mA for 6 hours. Then it will continue

to charge at a rate of 45mA.

BATSET   /T=hh:mm

Minimum value is 1 minute (00:01), Maximum value is 24 hours and 59 minutes.

Using normal Nickel Cadmium AA cells usually rated about .400AH they will be

properly charged from empty in about 6 hours at 100mA. However if you should be

using high capacity Nickel Cadmiums, they can have ratings as high as .850AH and

would be much better served by a longer fast charge, on the order of about 12

hours.

If you use the parameter to change this fast time, then BATSET will terminate

and stay resident. That means about 500 bytes of memory will be kept by BATSET,

and BATSET will chain to the Timer Tick interrupt and do its own counting. It

will look for the system counter to be reset to 0. This reset will happen when

the AC adapter is plugged in or it was plugged in but you just specified

charging. Now BATSET will set the system's charge timer to 1 hour and hold it

there until BATSET's own timer reaches the specified time. At this time BATSET

will set the system's charge timer to 6 hours, and the system will notice that

value and switch to slow charging.

Since BATSET is chained to the timer tick, it is possible it will cause serial

programs which run at high baud rates some difficulty. Be forwarned

If you have programs which read this system counter (e.g. Mark Scardina's

BATT100) you should be aware that with BATSET installed you will only ever see 1

hour in that field as long as fast charging is active. Then for slow charging

the number will start from 6 hours. An Example: Say you have BATSET set for 10

hours, and you run BATT100. BATT100 says you are slow charging and the timer

says 7:40. That means that you have fast charged for 10 hours and slow charged

for an additional 1 hour and 40 minutes (7:40 - 6:00).

BATSET is not a sophisticated terminate and stay resident program. Once loaded

it will stay there until you reboot. There is no way to remove it. Also do not

load it while you have system manager loaded, because when you close the DOS

application the timer tick interrupt will still point to where BATSET was and

... well ... uhhh. Guess it is reset time. So put it in your AUTOEXEC.BAT file

ahead of system manager command (100).

If you regularly keep your 100 plugged into AC, then you probably should not

need to increase your charging time. You may want to consider decreasing it. If,

on the other hand, you run your high capacity Nickel Cadmiums down until you are

getting a low batterie message then you probably want to increase the time to

about the capacity of the batterie times 150% divided by the charge current

hours. For example if you have Radio Shack's High Capacity .850AH, multiply by

150% for 1.275, then divide by charge current .100A to get a charge time of

12.75 hours. Remember this assumes we start with completely exhaused batterie

and charge it to 100%. I doubt we would be starting with a completely exhaused

batterie, but probably pretty low if you getting low batterie messages.

Freeware by Alex Patterson

Enjoy.