CS4235/36B/37B/38B/39

Release Notes

for the

Windows 3.1 Audio Driver

Version 2.00

Release Date:  01/10/98

This document includes the following sections:

- Release Information

- Driver installation

- Checking the driver version

- List of files

- File Summary

Release Information

This release adds support for the CS4235 and CS4239 chips, and drops support for the CS4232 and CS4236 chips.  Please continue to use version 1.80 for the CS4232 and CS4236 family chips.

This release supports the mode 3 mixer.  The Input mixer is now a true mixer instead of a mux.  The Input and Output Mixer applications and the Business Audio Transport application have been replaced by the Voyetra application that is included on a separate diskette.  The driver is on one diskette and Voyetra is on the other.

Several other applications were removed from the install including: the splash application (the panel that comes up during Windows startup which identifies the audio driver and plays a little tune), the Windows Audio Power application (APM still works), the DOS Hot Key Mixer TSR, the DOS Configuration program, the DOS CD Player Utility, the DOS Diagnostics program, and the WSS Initialization program.

Driver Installation

Installing from Diskette:

Insert the Driver diskette into the A: drive.  Go to the Windows Program Manager File pull down.  Select “Run…” Enter “A:\Setup.exe.  Answer all the questions and reboot when complete.

Now repeat the above only with the Voyetra diskette in the A: drive.

Installing from the Hard Drive:

When installing the software from the hard drive instead of using floppies, all of the files from the install disks must be copied to a single directory.  SETUP can then be run from there.  The driver diskette and the Voyetra diskette must be copied to different directories.

Checking the driver version

The Windows driver, VxD, and FM driver must have the same internal version number in order to work.  The driver has version checking code for the various pieces.

Version Numbers:

2.710 		CWDINIT.EXE

2.00  		CWBFM.DRV

2.00    	CWAUDI3.DRV

4.05.2700  	CWBMIX.EXE

2.00	  	CW3DW31.EXE

14	  	CS4237B\38B Microcode

List of Files

Disk 1

LOGO.BMP

SETUP.INS

SETUP.DLL

SETUP.EXE

SETUP.INF

RESOURCE.DLL

WINCM.DLL

DISK1.ID

CWPNP.DLL

CWDAUDIO.BIN

CWDAUDIO.INI

_INST16.EX_

_ISDEL.EXE

_SETUP.DLL

_SETUP.LIB

R200D1.Z

BWCC.DLL	COMPRESSED IN R200D1.Z

CWBFM.DRV	COMPRESSED IN R200D1.Z

CWBAUDI3.DRV	COMPRESSED IN R200D1.Z

MIDIMAP.CFG	COMPRESSED IN R200D1.Z

MPU401.DRV	COMPRESSED IN R200D1.Z

OEMSETUP.INF	COMPRESSED IN R200D1.Z

CWBAUDIX.386	COMPRESSED IN R200D1.Z

CWBAUDI3.INI	COMPRESSED IN R200D1.Z

DIALOG.FON	COMPRESSED IN R200D1.Z

CWDMIX.EXE	COMPRESSED IN R200D1.Z

CWDINIT.EXE	COMPRESSED IN R200D1.Z

MSMIXMGR.DLL	COMPRESSED IN R200D1.Z

THREED.VBX	COMPRESSED IN R200D1.Z

CW3DW31.EXE	COMPRESSED IN R200D1.Z

File Summary:

SETUP.EXE	CrystalWare Windows 3.1 Installation Program.  It will install the necessary files, install the drivers, modify CONFIG.SYS and AUTOEXEC.BAT, backup important files, and over install existing Crystal drivers.

SETUP.INF	Script file used by SETUP.EXE for installations.

SETUP.INS	File used by SETUP.EXE for installations.

SETUP.DLL 	File used by SETUP.EXE for installations.

_INST16.EX_		File used by SETUP.EXE for installations

_ISDEL.EXE		File used by SETUP.EXE for installations

_SETUP.DLL		File used by SETUP.EXE for installations

_SETUP.LIB	File used by SETUP.EXE for installations

LOGO.BMP	File used by SETUP.EXE for installations.

WINCM.DLL 	File used by SETUP.EXE for installations.

CSGROUP.EXE	Installation program used when over installing existing Crystal drivers.

RESOURCE.DLL	File used by SETUP.EXE for installations.

CWPNP.DLL	File used by SETUP.EXE for installations.  Used to detect the PnP configuration of the host machine as a PnP, Configuration Manager, or override PnP configuration.

DISK1.ID	Disk 1 ID used by SETUP.EXE.

R200D1.Z	Compressed archive of disk 1 that is uncompressed at installation time.

CWBAUDI3.DRV	Windows 3.1 wave, aux, and mixer driver.

CWBAUDI3.INI	File used by CWBAUDI3.DRV to save configuration and feature information.

CWBAUDIX.386	Windows 3.1 virtual device driver.

MIDIMAP.CFG	MIDIMAP Configuration that contains pre-configured midimaps for the Crystal FM driver and the MPU-401 driver.

MPU401.DRV	Windows 3.1 MPU-401 MIDI Driver.

OEMSETUP.INF	Windows install file for CSFM and MPU-401 MIDI driver.

CWBFM.DRV	Crystal FM MIDI driver.

MSMIXMGR.DLL	Windows 3.1 multimedia support file that provides enhanced mixer functionality for the Crystal Windows 3.1 driver.

CWDMIX.EXE 	DOS mixer used to set or adjust SoundBlaster mixer settings.  It will run as a GUI application or as a command line utility.

CWDINIT.EXE   	CS4232/36/36B/37B/38B Configuration Program.  A DOS device driver used to initialize and configure the card.  It runs in CONFIG.SYS or on a command line.

CWDAUDIO.INI   	Default initialization parameters for CS4232C.EXE.

CWDAUDIO.BIN	Data file containing Microcode for the CS4237B and CS4238B.  It is used by CWBINIT.EXE.

CW3DW31.EXE	Windows 3.1 applet used to enable, disable, and set mixer values for 3d audio.