Thank you for purchasing the Thunder Board!  We hope you use it often and

enjoy it with your favorite games and applications.  However, please take

a few minutes and read this entire text file before attempting to use any

of the software that we have included with the Thunder Board.

Included with the Thunder Board are the following programs:

1.	TBTEST.EXE

This program performs diagnostics on your Thunder Board to ensure

its correct installation.  It will report to the screen any error

conditions it senses during program execution.  The possible error

conditions include:

(0010)	Thunder Board I/O Error. Hardware Not Found!

a,  This problem will occur if the Thunder Board is not

properly seated in its slot.  Check the Thunder Board

to be sure it is squarely and evenly seated, and be sure

the Thunder Board bracket is securely fastened with a

bracket screw.

b,  It is also possible that the I/O address being used by

the Thunder Board conflicts with another device in your

system.  Refer to the hardware installation guide for

instructions on how to idenify and set the I/O address

for the Thunder Board.

(0020)	Different Hardware Found. Thunder Board Not Detected!

(0031)	Unable to identify the IRQ generated from the Thunder Board!

This condition will occur if the jumpers used to set the IRQ

for the Thunder Board are not correctly set.  Check your

Thunder Board to be sure that the shunts are squarely seated

on the jumper that corresponds to the IRQ that you would like

to use.

(0032)	Unable to identify the IRQ generated from the Thunder Board!

This condition will occur when the IRQ setting being used by

the Thunder Board conflicts with another device in your

system.  Refer to the hardware installation guide for

further instructions on how to modify or set the IRQ for

the Thunder Board.

(0041)	Unable to perform DMA transfers to/from the Thunder Board!

This condition will only occur if the DMA circuitry used for

transferring data and from the Thunder Board has failed.  If

this error message is displayed	contact your authorized

Thunder Board dealer, or Media Vision Technical Support

at (415) 770-9905.

2,	SETVOL.EXE

Run this program to set or modify the volume level of your audio

output.  SETVOL.EXE It is designed to produce a continuous audible

tone to help you adjust the volume using the thumbwheel on the back

of the Thunder Board.

SETVOL.EXE contains self guiding instructions for its use.  However,

if you have difficulty in using this program, check the audio cable

connections between the Thunder Board and your external speakers.

It is also possible that the FM synthesis circuit has been disabled.

Refer to the hardware installation guide for instructions on how

to determine whether this circuitry is currently enabled or

disabled.  The installation guide also contains information for

modifying the switch settings to enable the FM synthesis circuitry.

3,	RECFILE.EXE

To Use:  DOS>RECFILE [FILE.xxx] [C] [Ix] [Rxx] [Vxxx] [V+xxx]

Where:   [FILE.WAV] to record into the .WAV format (Microsoft compatible)

[FILE.VOC] to record into the .VOC format (SoundBlaster compatible)

[C]        to record with 2-to-1 compression

[Ix]       sets the IRQ channel number (2,3,5,7)

[Rxxxxx]   is the sample rate (6000 to 22050)

[Vxxx]     record only Voice activated levels (periods of

silence will not be recorded)

xxx is in the range of 0-127.  A lower number

makes recfile more sensitive to sound input; a

higher number has the opposite effect

[V+xxx]    record all when Voice activated at threshold (once

recording begins all sounds are recorded, including

periods of silence)

xxx is in the range of 0-127.  A lower number

makes the threshold value more sensitive to sound;

a higher number has the opposite effect

4,	PLAYFILE.EXE

To Use:  DOS>PLAYFILE [FILE.xxx] [C] [Ix] [Rxx] [S] [Sxxx]

Where:   [FILE.WAV] to play a .WAV format file (Microsoft compatible)

[FILE.VOC] to play a .VOC format file (SoundBlaster compatible)

[C]        to record with 2-to-1 compression

[Ix]       sets the IRQ channel number (2,3,5,7)

[Rxxxxx]   is the sample rate (6000 to 22050)

[S]	    specifies stereo playback

[Sxxx]     changes the playback speed by xxx percent

(0 to 200)