MCS-51 Microcontroller Family Cross Assembler

AAA     SSSSSS  EEEEEEE  MM   MM        5555555   11

AA AA   SS       EE       MMM MMM        55       111

AA   AA  SS       EE       MM M MM        55        11

AA   AA   SSSSS   EEEEEEE  MM   MM  ====  555555    11

AAAAAAA       SS  EE       MM   MM             55   11

AA   AA       SS  EE       MM   MM             55   11

AA   AA  SSSSSS   EEEEEEE  MM   MM        555555   1111

U S E R ' S    M A N U A L

Version  1.2

January 22, 1996

Copyright (c) 1994, 1996  by  W.W. Heinz

TABLE OF CONTENTS

-----------------

Foreword to Version 1.0

Foreword to Version 1.2

I.    Introduction

II.   Getting started

II.1  Files

II.2  ASEM-51 Installation

II.3  Invoking ASEM

II.3.1  DOS Command Line Operation

II.3.2  Running ASEM-51 in the Borland-IDE

II.3.3  Running ASEM-51 from Windows 3.1

II.3.4  Running ASEM-51 from BRIEF

II.3.5  Running ASEM-51 in a UNIX Environment

II.4  The HEXBIN Utility

II.5  The DEMO Program

III.  The ASEM-51 Assembly Language

III.1  Statements

III.2  Symbols

III.3  Constants

III.4  Expressions

III.5  The 8051 Instruction Set

III.6  Pseudo Instructions

III.7  Segment Type

III.8  Assembler Controls

III.8.1  Primary Controls

III.8.2  General Controls

III.9  Predefined Symbols

III.10 Conditional Assembly

IV.   Compatibility with the Intel Assembler

IV.1  Restrictions

IV.2  Extensions

IV.3  Further Differences

V.    List File Format

VI.   Support of 8051 Derivatives

Appendix A:  ASEM-51 Error Messages

A.1  Assembly Errors

A.2  Runtime Errors

Appendix B:  HEXBIN Error Messages

B.1  Conversion Errors

B.2  Runtime Errors

Appendix C:  Predefined Symbols

Appendix D:  Reserved Keywords

Appendix E:  Specification of the Intel-HEX Format

Appendix F:  The ASCII Character Set

Appendix G:  Literature

Appendix H:  Trademarks

Appendix I:  8051 Instructions in numerical Order

Appendix J:  8051 Instructions in lexical Order

Foreword to Version 1.0

=======================

Today microcontrollers are used in a wide range of applications from simple

consumer electronic products to complex avionic components. Thus I was not

very surprised to find an 80C31 on the videotext decoder board, I purchased

some time ago. Since it had a poor user interface and many bugs, I thought

I could do it better and so I began to look for an 8051 cross assembler.

But in contrast to the huge number of hardware components sold, the number

of people developing microcontroller software seemed to be remarkable small,

and so was the number of development tools on the market.

There was a very small number of good professional cross assemblers for $250

and up - too expensive for hobby purposes. Aside of useless demos, there were

no restricted starter kits or school versions available.

I found also a few shareware and public domain assemblers. But either they

were poor and not very reliable, or not very 8051-specific, or they used some

kind of fantasy syntax that was 100 % compatible to itself, but far away from

the Intel standard. I didn't like them all!

There seems to be a general lack of useful and affordable microcontroller

development software. This is a pity, because their universality, simple

architectures and low prices make microcontrollers so interesting especially

for hobby and education.

So I decided to write a handy 8051 cross assembler for the PC.

And here it is:             ASEM-51 V1.0

I hope it will help to discover the wonderful world of microcontrollers.

Have fun!

Deisenhofen,  July 19, 1994

W.W. Heinz

Foreword to Version 1.2

=======================

More than one year has passed, since I had released ASEM-51 V1.1 in

October 1994. Although I didn't spend all the time on ASEM-51, V1.2

comes with several extensions, bug fixes, and numerous functional or

internal improvements!

Highlights of the new version are a nearly perfectly featured list

file with cross-reference and some new printing options, a bootstrap

program for MCS-51 evaluation boards, and plenty of new *.MCU files.

For detailed information see the ASEM-51 V1.2 Release Notes.

What I have learned through the last two years is that freeware is

not free, neither for the author nor for the users.

ASEM-51 could not be made with nothing but numberless free hours,

spent on pure software development. I also had to purchase a PASCAL

development system, lots of microcontroller literature, and an 80C535

evaluation board.

The distribution of freeware seems to be a bigger problem than its

development. First of all, one has to buy a modem. After that, it

costs a lot of time, fees, trouble, and "interesting" discussions

with the particular sysops, until the stuff is posted (or not) on

several BBS and ftp sites.

To publish a program on shareware CD-ROMs, one has only to find out,

which are the most suitable. For this, it is best to buy a dozen or

two (and a CD-ROM drive), and to send the software to the publishers

of those that seem to be the most popular.

The interested users finally have to purchase modems or CD-ROM drives,

and pay the same fees, or buy the same CD-ROMs, to get the "freeware"

again from these public sources.

After all, it may be cheaper, faster, and more convenient, to simply buy

a professional software solution (if any) in the PC shop at the corner.

But it's not half the fun!

ASEM-51 V1.1 had been distributed (and mirrored) to more than 60 ftp

sites all over the world, uploaded to so many BBS, and published on

at least two shareware CD-ROMs.

But I only received mails from 9 users, a local cockroach, and an

international monster. The latter two asked me for permission, to

sell ASEM-51 for (their) profit, and failed miserably.

Most of the user mails started with "I have copied your assembler

from an ftp site, which I don't remember. It is looking fine on the

first glance! By the way, have you got a data sheet of the 80Cxyz?",

or something like that.

During all the time, I have received one single error report only.

Since it had been reported by phone, I couldn't reproduce it.

Nevertheless two serious bugs have been fixed since version 1.1, but

I have found them by myself in November 1995 both.

Sure ASEM-51 is no mainstream software, but to be honest, I am a

little disappointed of the poor user feedback!

Finally, I should thank the persons, who helped me to release ASEM-51:

Andreas Kies has tested all previous beta versions of the assembler. He

has distributed the first releases, and maintained a free ASEM-51 support

account right from the beginning.

Gabriele Novak has checked the orthography of all the documentation files.

Werner Allinger has tested the latest beta version and the bootstrap program.

Last but not least, I want to thank all interested users for their comments

and suggestions.

Deisenhofen,  January 22, 1996

W.W. Heinz

I. Introduction

===============

ASEM-51 is a generic two-pass cross assembler for the MCS-51 microcontroller

family. It is running on IBM-PC/XT/AT computers and all true compatibles.

It only requires 256K of memory and MS-DOS 2.0 (or higher).

The ASEM-51 assembly language is a subset of the Intel standard that

guarantees maximum compatibility with existing 8051 assembler sources.

ASEM-51 generates an Intel-HEX file output, which is directly accepted by

most EPROM blowers. Thus ASEM-51 is suitable for small and medium 8031/51-

based microcontroller projects in the non-profit area.

However, ASEM-51 has been designed to process also very large programs!

Its most important features are:

- fast, compact, reliable, easy to use, and well-documented

- easy installation, no configuration required

- command line operation, batch and networking capability

- support for easy integration into the popular Borland IDE

- support for installation under Windows 3.1

- Intel-compatible syntax

- five location counters, one for each of the 8051 address spaces

- built-in symbols for 8051 special function registers (can be disabled)

- 8051 register bank support

- assembly-time evaluation of arithmetic and logical expressions

- segment type checking for instruction operands

- automatic code optimization of generic jumps and calls

- nested include file processing

- nested conditional assembly

- detailed assembler listing with symbol table or cross reference

- further fancy printing facilities  ;-)

- Intel-HEX file output

- hex-to-binary conversion utility

- direct support of more than thirty 8051 derivatives

- support of user-defined 8051 derivatives

- special support of the Philips 83C75x family

- bootstrap program for testing on the MCS-51 target board

- limited update service by the author

- also running under OS/2 2.x and Warp     (in the DOS window)

- also running under Windows 3.1 and 95    (in the DOS box)

- also running under LINUX 1.1.12 and up   (with dosemu 0.52 and up)

- YES, it runs with Novell DOS 7.

All executable programs of the ASEM-51 package have been developed with:

Turbo-Pascal 7.01 for DOS (c) Borland International 1992

II. Getting started

===================

II.1 Files

----------

Your ASEM-51 distribution disk or archive should contain the following

groups of files:

1.)  ASEM_51.DOC          ASEM-51 User's Manual  (this file)

ASEM.EXE          cross assembler

ASEM.PIF          program information file for Windows 3.1

ASEM.ICO          icon file for Windows 3.1

ASEM2MSG.EXE          ASEM-51 message filter for Borland-IDE

ASEM2MSG.PAS          Turbo-Pascal source of ASEM2MSG.EXE

HEXBIN.EXE          hex-to-binary conversion utility

DEMO.A51          a sample 8051 assembler program

*.MCU          processor definition files of 8051 derivatives

(for a detailed list of MCU files see chapter

"VI. Support of 8051 Derivatives")

2.)   BOOT51.DOC          BOOT-51 user's manual

BOOT51.A51          BOOT-51 assembler source (requires ASEM-51 V1.2)

CUSTOMIZ.EXE          BOOT-51 customization utility

BOOT.BAT          batch file for application program upload

LOAD.BAT          called by BOOT.BAT only

RESET.EXE          program to reset target system via PC printer port

BLINK.A51          sample test program for BOOT-51

3.)  LICENSE.DOC          ASEM-51 License Agreement

RELEASE.120          ASEM-51 V1.2 Release Notes

UPDATES.DOC          ASEM-51 Updating Guide

KILLASEM.BAT          deletes all files of the ASEM-51 package

FILE_ID.DIZ          a brief description of the ASEM-51 archive

The first group contains all files directly associated with the assembler.

The second group contains all files directly associated with the bootstrap

program. The third group contains general support and documentation files

that apply to the whole package.

II.2 ASEM-51 Installation

-------------------------

In principle ASEM-51 doesn't require a fuzzy software installation or

configuration. In the simplest case you can copy all files of the package

to your working directory, and enjoy the benefits of true plug-and-play

compatibility!

On the other hand, an installation of ASEM-51 under MS-DOS is very simple:

- Create a new directory on your harddisk, e.g. C:\ASEM51.

- Copy all files of the ASEM-51 package into this directory.

- Append it to your PATH statement in file AUTOEXEC.BAT, e.g.

PATH C:\DOS;C:\UTIL;C:\ASEM51

- Reboot your PC.

II.3 Invoking ASEM

------------------

ASEM-51 provides full support of DOS command line operation and batch

capability as the best commercial development tools.   :-)

Nevertheless, it can be integrated into foreign development environments,

if desired.

II.3.1 DOS Command Line Operation

---------------------------------

ASEM-51 is invoked by typing:

ASEM <source> [<hex> [<list>]] [/INCLUDES:p] [/DEFINE:s:v:t] [/COLUMNS]

where <source> is the 8051 assembler source, <hex> is the output file in

Intel-HEX format, and <list> is the assembler list file.

The parameters <hex> and <list> are optional. When omitted, the file names

are derived from the <source> name, but with extensions HEX and LST.

All file names may be specified without extensions. In these cases, the

assembler adds default extensions as shown below:

file             extension

--------------------------

<source>         .A51

<hex>            .HEX

<list>           .LST

If you want a file name to have no extension, terminate it with a '.'!

Instead of file names you may also specify device names to redirect the

output to character I/O ports. Device names may be terminated with a ':'!

It is not checked, whether the device is existing or suitable for the task.

Although it is possible to read the source file from a character device

(e.g. CON:) instead of a file, this cannot be recommended: Since ASEM-51

is a two-pass assembler, it always reads the source file twice!

When the /INCLUDES option is used, the assembler searches the specified

path p for include files that cannot be found in the working directory.

p may be any number of directories separated by ';' characters.

The directories will be searched from left to right.

The /DEFINE option is useful for selecting particular program variants

from the command line that have been implemented with conditional assembly.

It allows to define a symbol s with value v and segment type t in the

command line. v and t are optional. The segment type of the symbol defaults

to NUMBER, if t is omitted. The symbol value will be 0, if v is omitted.

v may be any numeric constant. t must be one of the following characters:

C   =   CODE

D   =   DATA

I   =   IDATA

X   =   XDATA

B   =   BIT

N   =   NUMBER    (default)

Options may be abbreviated as long as they remain unique!

Examples:

1.)      ASEM PROGRAM

will assemble the 8051 assembly language program PROGRAM.A51 and

produce an Intel-HEX file PROGRAM.HEX and a listing PROGRAM.LST.

2.)      ASEM TARZAN.ASM JANE JUNGLE.PRN

will assemble the 8051 assembly language program TARZAN.ASM and

produce an Intel-HEX file JANE.HEX and a listing JUNGLE.PRN.

3.)      ASEM PROJECT EPROM.

will assemble the 8051 assembly language program PROJECT.A51 and

produce an Intel-HEX file EPROM and a listing PROJECT.LST.

4.)      ASEM sample COM2: NUL

will assemble the 8051 assembly language program SAMPLE.A51, send

the HEX file output to the serial interface COM2 and suppress the

list file output by sending it to the NUL device.

5.)      ASEM APPLICAT /INC:C:\ASEM51;D:\MICROS\8051\HEADERS

will assemble the program APPLICAT.A51, while all required include

files will be searched first in the default directory, then in

C:\ASEM51, and finally in D:\MICROS\8051\HEADERS.

6.)      ASEM UNIVERSL /D:Eva_Board:8000H:C

will assemble the program UNIVERSL.A51, while the CODE symbol

EVA_BOARD will be predefined with value 8000H during assembly.

When program errors are detected, they are flagged on the console. This

may look as follows:

MCS-51 Family Cross Assembler ASEM-51 V1.2

APPLICAT.A51(14): must be known on first pass

USERBITS.INC(6): attempt to divide by zero

DEFINES.INC(37): symbol not defined

APPLICAT.A51(20): symbol not defined

APPLICAT.A51(27): no END statement found

5 errors detected

Every error is flagged with the name of the source or include file, the

local line number where it was found, and the error message itself.

This output format makes it easy to integrate ASEM-51 into existing foreign

development environments or workbenches.

A perfect fit for the Turbo C++ IDE (and perhaps others) can be reached

with the /COLUMNS option. When specified, the column numbers of program

errors are output additionally after the line numbers:

MCS-51 Family Cross Assembler ASEM-51 V1.2

APPLICAT.A51(14,12): must be known on first pass

USERBITS.INC(6,27): attempt to divide by zero

DEFINES.INC(37,18): symbol not defined

APPLICAT.A51(20,18): symbol not defined

APPLICAT.A51(27,1): no END statement found

5 errors detected

When terminating ASEM-51 returns an exit code to the operating system:

situation                           ERRORLEVEL

----------------------------------------------

no errors                               0

program errors detected                 1

fatal runtime error                     2

II.3.2 Running ASEM-51 in the Borland-IDE

-----------------------------------------

Turbo C++ (1.0 thru 3.0) users will appreciate the possibility to invoke

ASEM-51 as a transfer program from the Borland IDE.

For this, the filter program ASEM2MSG for the ASEM-51 error messages

has been provided. To integrate ASEM-51 into the Borland IDE, perform

the following steps:

- Be sure that ASEM-51 has been installed properly as described before,

or that ASEM.EXE and ASEM2MSG.EXE are somewhere in your PATH.

- Start the Turbo C++ (or Borland C++) IDE for DOS.

- For Turbo C++ 1.0, first click:  Options | Full menus | ON

- Click from the menu bar:         Options | Transfer

- When the "Transfer" dialog box is active, press the Edit button.

- Now the "Modify/New Transfer Item" dialog box should be active.

Fill in the following items:

Program Title:   ASEM-~51

Program Path:    ASEM

Command Line:    $NOSWAP $SAVE CUR $CAP MSG(ASEM2MSG) $EDNAME /C

Translator:      [X]

Hot key:         Shift F8

Then press the New button.

- When returned to the "Transfer" dialog box, press the OK button.

- Click from the menu bar:         Options | Save | OK

Now it should be possible, to assemble the file in the active edit

window with ASEM-51, when pressing Shift-F8. The error messages (if any)

should appear in the "Message" window. You can browse through the errors,

and jump into the source text by simply pressing <Enter>. This even works,

if the error is not in the program itself, but in an include file!

Turbo-Pascal 7.0 users can also employ their Borland IDE for assembly.

To integrate ASEM-51 into the Turbo-Pascal IDE, perform the following

steps:

- Be sure that ASEM-51 has been installed properly as described before,

or that ASEM.EXE and ASEM2MSG.EXE are somewhere in your PATH.

- Start the Turbo-Pascal 7.0 (or Borland-Pascal 7.0) IDE for DOS.

- Click from the menu bar:         Options | Tools

- When the "Tools" dialog box is active, press the New button.

- Now the "Modify/New Tool" dialog box should be active.

Fill in the following items:

Title:           ASEM-~5~1

Program path:    ASEM

Command line:    $NOSWAP $SAVE CUR $CAP MSG(ASEM2MSG) $EDNAME

Hot keys:        Shift+F8

Then press the OK button.

- When returned to the "Tools" dialog box, press the OK button.

- Click from the menu bar:         Options | Environment | Preferences

- When the "Preferences" dialog box is active, disable the "Close on

go to source" item in the "Options" checkbox. Then press the OK button.

- Finally click from the menu bar: Options | Save

Now ASEM-51 can be invoked with Shift F8, to assemble the program in the

active edit window, while error messages (if any) appear in the "Messages"

window.

Users of both Turbo C++ and Turbo-Pascal should prefer the Turbo C++ IDE.

In the Turbo-Pascal 7.0 IDE, the /COLUMNS (or /C) option has no effect!

II.3.3 Running ASEM-51 from Windows 3.1

---------------------------------------

For integration into the Windows 3.1 desktop, the files ASEM.PIF and

ASEM.ICO have been provided. To insert ASEM-51 into a group of the

Program Manager, perform the following steps:

- Be sure that ASEM-51 has been installed properly for MS-DOS

as described before.

- Start Windows 3.1 and expand the Program Manager window to its

full screen size representation, if necessary.

- Focus the program group in which ASEM-51 is to be inserted,

e.g. "Applications".

- Click from the Program Manager menu bar:   File | New

- When the "New Program Object" dialog box is active, choose

the option "program", and click the OK button.

- Now the "Program Properties" dialog box should be active.

Fill in the following items:

Description:          ASEM-51

Command line:         ASEM.PIF

Working Directory:                  (whatever you want)

Hot key:                            (whatever you want)

as a Symbol:          [ ]

Then press the [Change Icon] button.

- Now a message box appears with the error message

"no icon available for the specified file".

Simply press the OK button.

- The "Select Icon" dialog box should be displayed now. Fill in

File Name:      ASEM.ICO

and press the OK button. Now the ASEM-51 icon should be displayed

in the icon field. Press the OK button again.

- When returned to the "Program Properties" dialog box, press the

OK button.

(Since I had no English Windows version to try this, things may look

slightly different in real life.)

Now ASEM-51 can be invoked by simply clicking the ASEM-51 icon twice.

After entering the program parameters in a corresponding dialog box,

ASEM-51 is running in a DOS window, which remains open after program

termination, to let you have a look on the error messages.

II.3.4  Running ASEM-51 from BRIEF

----------------------------------

BRIEF 3.x users can integrate ASEM-51 into their editor by simply

defining another DOS environment variable in their AUTOEXEC.BAT with

SET BCA51="ASEM %%s"

This specifies the command for compiling files with extension *.A51.

After that, ASEM-51 can be invoked from BRIEF with Alt-F10.

II.3.5  Running ASEM-51 in a UNIX Environment

---------------------------------------------

If you are running ASEM-51 on a Linux or another PC-based UNIX system

with an MS-DOS emulator, note that all executable programs of the ASEM-51

package can only read ASCII files in DOS format with CR/LF at the end of

line. Native UNIX text files with only a LF at the end of line cannot be

processed!

Consequently, all ASCII files, generated by programs of the ASEM-51

package, are written in MS-DOS format.

II.4 The HEXBIN Utility

-----------------------

Most EPROM blowers accept the Intel-HEX object file format that is output

by ASEM-51. However, for dumb EPROM blowers and special purposes it might be

useful to convert the HEX file to a pure binary image file.

For this the conversion utility HEXBIN is provided.

It is invoked as follows:

HEXBIN <hex> [<bin>] [/OFFSET:o] [/LENGTH:l] [/FILL:f]

where <hex> is the input file in Intel-HEX format, and <bin> is the

binary output file. The parameter <bin> is optional. When omitted, the

file name is derived from the <hex> file name, but with the extension BIN.

All file names may be specified without extensions. In these cases, the

program adds default extensions as shown below:

file             extension

------------------------------

<hex>            .HEX

<bin>            .BIN

If you want a file name to have no extension, terminate it with a '.'!

Instead of file names you may also specify device names to redirect the

output to character I/O ports. Device names may be terminated with a ':'!

It is not checked, whether the device is existing or suitable for the task.

The binary file output can also be controlled with the options /OFFSET,

/FILL and /LENGTH.

Normally the first byte in the binary file is the first byte of the HEX

record with the lowest load address. If a number of dummy bytes is to be

inserted on top of the file (e.g. for alignment in an EPROM image), this

can be performed with the /OFFSET option:

/OFFSET:1000

would insert 4096 dummy bytes before the first byte of the first HEX record

loaded. The offset must always be specified as a hex number. The default

offset is 0.

Since there may be peepholes between the HEX records, a fill byte value can

be defined with the /FILL option:

/FILL:0

would fill all peepholes between the HEX records with zero bytes as well as

all the dummy bytes that might have been inserted with the /OFFSET or /LENGTH

option. The fill byte value must always be specified as a hex number.

The default fill byte is the EPROM-friendly FFH.

By default the last byte in the binary file is the last byte of the HEX

record with the highest load address. If the binary file should have a

well defined length, then a number of dummy bytes can be appended to the

file (e.g. for exactly matching an EPROM length), this can be performed

with the /LENGTH option:

/LENGTH:8000

would append as many dummy bytes behind the last byte of the file, that the

total file length becomes exactly 32768 bytes. The file length must always

be specified as a hex number.

Options may be abbreviated as long as they remain unique!

Examples:

1.)      HEXBIN PROGRAM

will convert the Intel-HEX file PROGRAM.HEX to a pure binary image file

PROGRAM.BIN.

2.)      HEXBIN TARZAN.OBJ JUNGLE/FILL:E5

will convert the Intel-HEX file TARZAN.OBJ to a binary image file

JUNGLE.BIN and fill all peepholes between the HEX file records with

the binary value E5H.

3.)      HEXBIN PROJECT EPROM. /off:8000 /length:10000 /f:0

will convert the Intel-HEX file PROJECT.HEX to a binary image file

EPROM, insert 32K dummy bytes on top of file, fill all peepholes

and the dummy bytes with nulls, and extend the file to exactly 64K.

When terminating HEXBIN returns an exit code to the operating system:

situation                           ERRORLEVEL

----------------------------------------------

no errors                               0

conversion errors detected              1

fatal runtime error                     2

II.5 The DEMO Program

---------------------

For getting started with a new assembler, it is always good to have a

program, that can be assembled with it. For this purpose, the 8051

assembler program DEMO.A51 is provided, that can be used for a first

test of ASEM-51. Be sure to have all files of the ASEM-51 package on your

disk! Then simply type

ASEM DEMO

HEXBIN DEMO

at the DOS prompt. ASEM and HEXBIN should finish without errors and you

should have the following new files on your disk:

DEMO.HEX          Intel-HEX file

DEMO.LST          assembler list file of DEMO.A51

DEMO.BIN          binary image file of DEMO.HEX

When something goes wrong, there may be files missing in your distribution!

DEMO.A51 may also serve as a sample assembler program that includes

examples for all machine instructions, pseudo instructions and assembler

controls, that have been implemented in ASEM-51. Whenever in doubt how to

use a particular command, DEMO.A51 may be a valuable help.

III.  The ASEM-51 Assembly Language

===================================

The user should be familiar with 8051 microcontrollers and assembly

language programming. This manual will not explain the architecture of

the MCS-51 microcontroller family nor will it discuss the basic concepts

of assembly language programming. It only describes the general syntax

of assembler statements and the assembler instructions that have been

implemented in ASEM-51.

III.1 Statements

----------------

Source files consist of a sequence of statements of one of the forms:

[symbol:]  [instruction [arguments]]     [;comment]

symbol     instruction  argument        [;comment]

$control   [argument]                    [;comment]

Everything that is written in brackets is optional.

The maximum length of source code lines is 255 characters.

Everything from the ';' character to the end of line is assumed to be

commentary. Blank lines are considered to be commentary, too.

The lexical elements of a statement may be separated by blanks and tabs.

Aside of character string constants, upper and lower case letters are

equivalent.

Examples:     HERE:   MOV A,#0FFH     ;define label HERE and load A with FFH

YEAR EQU 1996   ;define symbol for current year

$INCLUDE (80C517.MCU)   ;include SAB80C517 register definitions

III.2 Symbols

-------------

Symbols are user-defined names for addresses or numbers.

Their maximum significant length is 31 characters. They can be even

longer, but everything behind the first 31 characters is ignored.

Symbols may consist of letters, digits, '_' and '?' characters.

A symbol name must not start with a digit!

Upper and lower case letters are considered to be equivalent.

Note: Assembly language keywords must not be redefined as user symbols!

Example:     Is_this_really_a_SYMBOL_?       is a legal symbol name!

III.3 Constants

---------------

Numeric constants consist of a sequence of digits, followed by a radix

specifier. The first character must always be a decimal digit.

The legal digits and radix specifiers are:

constant     digits        radix

------------------------------------

binary       0 ... 1       B

octal        0 ... 7       Q or O

decimal      0 ... 9       D or none

hex          0 ... F       H

Thus, for example, the following constants are equivalent:

1111111B      binary

177Q      octal

177o      octal

127      decimal

127d      decimal

07FH      hex

Character constants may be used wherever a numeric value is allowed.

A character constant consists of one or two printing characters enclosed

in single or double quotes. The quote character itself can be represented

by two subsequent quotes. For example:

'X'       8 bit constant:       58H

"a@"      16 bit constant:     6140H

''''       8 bit constant:       27H

In DB statements, character constants may have any length.

In this case, we call it a character string. For example:

"It's only text!"

III.4 Expressions

-----------------

Arithmetic expressions are composed of operands, operators and parentheses.

Operands may be user-defined symbols, constants or special assembler symbols.

All operands are treated as unsigned 16-bit numbers.

Special assembler symbols, that can be used as operands are:

AR0, ... , AR7     direct addresses of registers R0 thru R7

$                  the location counter of the currently active segment

(start address of the current assembler statement)

The following operators are implemented:

Unary operators:   +          identity:             +x = x

-          two's complement:     -x = 0-x

NOT        one's complement:  NOT x = FFFFH-x

HIGH       high order byte

LOW        low order byte

Binary operators:  +          unsigned addition

-          unsigned subtraction

*          unsigned multiplication

/          unsigned division

MOD        unsigned remainder

SHL        logical shift left

SHR        logical shift right

AND        logical and

OR         logical or

XOR        exclusive or

.          bit operator used for bit-adressable locations

EQ  or =   equal to                ----.

NE  or <>  not equal to                |  results are:

LT  or <   less than                   |

LE  or <=  less or equal than          |     0 if FALSE

GT  or >   greater than                |  FFFF if TRUE

GE  or >=  greater or equal than   ____|

Operators that are no special characters but keywords as SHR or AND must

be separated from their operands by at least one blank or tab.

In general expressions are evaluated from left to right according to

operator precedence, which may be overridden by parentheses.

Parentheses may be nested to any level.

Expressions always evaluate to unsigned 16-bit numbers, while overflows

are ignored. When an expression result is to be assigned to an 8-bit

quantity, the high byte must be either 00 or FF.

Operator precedence:

-----------------------------------------------------------------

( )                                                   ^   highest

+ - NOT HIGH LOW                       (unary)        |

.                                                     |

* / MOD                                               |

SHL SHR                                               |

+ -                                    (binary)       |

EQ = NE <> LT < LE <= GT > GE >=                      |

AND                                                   |

OR XOR                                                v   lowest

-----------------------------------------------------------------

Example:  The expression  P1.((87+3)/10 AND -1 SHR 0DH)  will evaluate to 91H.

III.5 The 8051 Instruction Set

------------------------------

ASEM-51 implements all 8051 machine instructions including generic jumps

and calls. The assembler implements two instructions

JMP  <address>

CALL <address>

that do not represent a specific opcode: generic jump and call.

These instructions will always evaluate to a jump or call, not necessarily

the shortest, that will reach the specified address.

JMP may assemble to SJMP, AJMP or LJMP, while CALL can only evaluate to

ACALL or LCALL. Note that the assembler decision may not be optimal. For

code addresses that are forward references, the assembler always generates

LJMP or LCALL respectively. However, for backward references this is a

powerful tool to reduce code size without extra trouble.

With the $PHILIPS control, ASEM-51 can be switched to the reduced instruction

set of the Philips 83C75x family of microcontrollers. This disables the LJMP,

LCALL, and MOVX instructions as well as the XDATA and XSEG pseudo instructions,

and generic jumps and calls will always assemble to absolute addressing.

The rest of the 8051 instruction mnemonics is listed in Appendix D.

Appendices I and J are containing tables of all 8051 instructions with

their opcodes, mnemonics, arguments, lengths, affected flags and durations.

The comprehensive example program DEMO.A51 provided shows all the 8051

instructions in a syntactical context.

All 8051 instruction mnemonics are copyright (c) by Intel corporation!

III.6 Pseudo Instructions

-------------------------

In the subsequent paragraphs, all ASEM-51 pseudo instructions are described.

Lexical symbols are written in lower case letters, while assembler keywords

are written in upper case.

Instruction arguments are represented by <arg>, <arg1> or something like

that. Expressions are represented by <expr>, <expr1> and so on.

Syntax elements enclosed in brackets are optional.

The ellipsis "..." means always "a list with any number of elements".

DB <arg1> [,<arg2> [,<arg3> ... ]]              define bytes

The DB instruction reserves and initializes a number of bytes with

the values defined by the arguments. The arguments may either be

expressions (which must evaluate to 8-bit values) or character

strings of any length. DB is only allowed in the CODE segment!

Example:   DB 11,'January',96,(3*7+12)/11

DW <expr1> [,<expr2> [,<expr3> ... ]]           define words

The DW instruction reserves and initializes a number of words with

the values defined by the arguments. Every argument may be an

arbitrary expression and requires two bytes of space.

DW is only allowed in the CODE segment!

Example:   DW 0,0C800H,1996,4711

DS <expr>                                       define space

Reserves a number of uninitialized bytes in the current segment.

The value of <expr> must be known on pass 1!

DS is allowed in every segment!

Example:   DS 200H

DBIT <expr>                                     define bits

Reserves a number of uninitialized bits.

The value of <expr> must be known on pass 1!

DBIT is only allowed in the BIT segment!

Example:   DBIT 16

NAME <symbol>                                   define module name

Defines a name for the program module. The module name must be a

legal symbol. Only one NAME instruction is allowed in the program.

Since the NAME instruction has been introduced for compatibility

purposes only, the symbol is not currently used. It may be redefined

in the subsequent program.

ORG <expr>                                      origin of segment

Sets the location counter of the current segment to the value <expr>.

Additional ORG statements may be used to generate program segments

which will load at different locations.

The value of <expr> must be known on pass 1!

The default value of all location counters at program start is 0.

Example:   ORG 08000H

USING <expr>                                    using register bank

Sets the register bank used to <expr>, which must be in the range

of 0...3. The USING instruction only affects the values of the

special assembler symbols AR0, ... , AR7 representing the direct

addresses of registers R0, ... , R7 in the current register bank.

The value of <expr> must be known on pass 1!

The default value for the register bank is 0.

Example:   USING 1

END                                             end of program

This must be the last statement in the source file. After the END

statement only commentary and blank lines are allowed!

Example:   END       ;end of program

<symbol> EQU <expr>                             define constant

<symbol> SET <expr>                             define variable

The EQU instruction defines a symbolic constant of the type NUMBER.

A symbol defined with EQU can never be changed!

The SET instruction defines a symbolic value of the type NUMBER,

that may be changed with subsequent SET statements.

The value of <expr> must be known on pass 1!

A symbol that has been SET, cannot be redefined with EQU!

A symbol that has been EQU'd cannot be reSET!

On pass 2, forward references to a SET symbol always evaluate

to the last value, the symbol has been SET to on pass 1.

Examples:   MAXMONTH  EQU 12

OCTOBER   EQU MAXMONTH-2

CHAPTER   SET  1

CHAPTER   SET  CHAPTER+1

<symbol> CODE  <expr>                           define ROM address

<symbol> DATA  <expr>                           define direct RAM address

<symbol> IDATA <expr>                           define indirect RAM address

<symbol> BIT   <expr>                           define bit address

<symbol> XDATA <expr>                           define external RAM address

These instructions define symbolic addresses for the five 8051

memory segments (address spaces). For DATA, IDATA and BIT type

symbols, the value of <expr> must not exceed 0FFH!

The value of <expr> must be known on pass 1!

Once defined with one of the above instructions, the symbols cannot

be redefined.

Examples:   EPROM    CODE  08000H

STACK    DATA       7

V24BUF   IDATA   080H

REDLED   BIT     P1.5

SAMPLER  XDATA  0100H

CSEG [AT <expr>]                           switch to CODE  space [at address]

DSEG [AT <expr>]                           switch to DATA  space [at address]

ISEG [AT <expr>]                           switch to IDATA space [at address]

BSEG [AT <expr>]                           switch to BIT   space [at address]

XSEG [AT <expr>]                           switch to XDATA space [at address]

These instructions switch to one of the five 8051 memory segments

(address spaces) and optionally set the location counter of that

segment to a particular address <expr>.

When the construction "AT <expr>" is omitted, the location counter

keeps its previous value.

The value of <expr> must be known on pass 1!

At program start the default segment is CODE and all the location

counters are set to zero.

Examples:   DSEG            ;switch to DATA segment

CSEG AT 8000h   ;switch to CODE segment at address 8000H

XSEG at 0       ;switch to XDATA segment at address 0

III.7 Segment Type

------------------

Every assembly time expression is assigned a segment type, depending on

its operands and operators. The segment type indicates the address space,

the expression result might belong to, if it were used as an address.

There are six possible segment types:

CODE

DATA

IDATA

XDATA

BIT

NUMBER    (typeless)

Most expression results have the segment type NUMBER. That means they are

assumed to be typeless. However, in some cases it may be useful to assign

a particular segment type!

The following six rules apply when the segment type is evaluated:

1. Numerical constants are always typeless.

Consequently their segment type is NUMBER.

2. Symbols are assigned a segment type during definition. Symbols

that are defined with EQU or SET have no segment type.

Labels get the segment type of the currently active segment.

3. The result of a unary operation (+, -, NOT, HIGH, LOW) will have

the segment type of its operand.

4. The results of all binary operations (except "+", "-" and ".") will

have no segment type.

5. If only one operand in a binary "+" or "-" operation has a segment

type, then the result will have that segment type, too. In all other

cases, the result will have no segment type.

6. The result of the bit operation "." will always have the segment

type BIT.

Examples:

---------       The following symbols have been defined in a program:

OFFSET  EQU   16

START   CODE  30H

DOIT    CODE  0100H

REDLED  BIT   P1.3

VARIAB4 DATA  20H

PORT    DATA  0C8H

RELAY   EQU   5

1.)  The expression  START+OFFSET+3  will have the segment type CODE.

2.)  The expression  START+DOIT  will be typeless.

3.)  The expression  DOIT-REDLED  will be typeless.

4.)  The expression  2*VARIAB4  will be typeless.

5.)  The expression  PORT.RELAY  will have the segment type BIT.

The segment type is checked, when expressions appear as addresses. When the

expression result is not typeless and does not have the segment type of the

corresponding segment, the instruction is flagged with an error message.

The only exceptions are the segment types DATA and IDATA, which are assumed

to be compatible in the address range of 0 to 7FH. Since ASEM-51 does only

support absolute segments, those addresses are really always pointing to the

same physical location in the internal memory.

Example:

--------

Line  I  Addr  Code            Source

1:          N        30             DSEG AT 030H     ;internal RAM

2:      30  N        01     COUNT:  DS 1             ;counter variable

3:

4:                                  CSEG             ;ROM

5:    0000  C2 30           START:  CLR COUNT

^

@@@@@ segment type mismatch @@@@@

The CLR instruction is flagged with the error message "segment type mismatch"

in the assembler list file, because only a BIT type address is allowed here.

However, COUNT is a label with the segment type DATA!

III.8 Assembler Controls

------------------------

ASEM-51 implements a number of assembler controls that influence the

assembly process and list file generation. There are two groups of

controls: primary and general controls.

Primary controls can only be used at the beginning of the program and

remain in effect throughout the assembly. They may be preceded only

by control statements, blank and commentary lines. If the same primary

control is used multiple times with different parameters, the last one

counts.

General controls may be used everywhere in the program. They perform a

single action, or remain in effect until they are cancelled or changed by

a subsequent control statement.

Assembler controls may have a number or string type operand.

Number type operands are arithmetic expressions that must be known on pass 1.

String type operands are character strings which are enclosed in parentheses

instead of quotes. In analogy to quoted strings, no control characters

(including tabs) are allowed within these strings! The string delimiter ')'

can be represented by two subsequent ')' characters.

The subsequent paragraphs contain detailed explanations of the implemented

controls and their abbreviations:

Control        Type  Default    Abbreviation            Meaning

------------------------------------------------------------------------------

$DATE(string)    P      ''      $DA      inserts date string into page header

------------------------------------------------------------------------------

$DEBUG           P   $NODEBUG   $DB      (currently dummy)

$NODEBUG         P              $NODB    (   "        "  )

------------------------------------------------------------------------------

$EJECT           G              $EJ      start a new page in list file

------------------------------------------------------------------------------

$INCLUDE(file)   G              $IC      include a source file

------------------------------------------------------------------------------

$LIST            G   $LIST      $LI      list subsequent source lines

$NOLIST          G              $NOLI    don't list subsequent source lines

------------------------------------------------------------------------------

$MOD51           P   $MOD51     $MO      enable predefined SFR symbols

$NOMOD51         P              $NOMO    disable predefined SFR symbols

------------------------------------------------------------------------------

$PAGING          P   $PAGING    $PI      enable listing page formatting

$NOPAGING        P              $NOPI    disable listing page formatting

------------------------------------------------------------------------------

$PAGELENGTH(n)   P   n=64       $PL      set lines per page for listing

------------------------------------------------------------------------------

$PAGEWIDTH(n)    P   n=132      $PW      set columns per line for listing

------------------------------------------------------------------------------

$PHILIPS         P   MCS-51     ---      switch on 83C75x family support

------------------------------------------------------------------------------

$SYMBOLS         P   $SYMBOLS   $SB      create symbol table

$NOSYMBOLS       P              $NOSB    don't create symbol table

------------------------------------------------------------------------------

$NOTABS          P   use tabs   ---      don't use tabs in list file

------------------------------------------------------------------------------

$TITLE(string)   G   copyright  $TT      inserts title string into page header

------------------------------------------------------------------------------

$XREF            P   $NOXREF    $XR      create cross reference

$NOXREF          P              $NOXR    don't create cross reference

III.8.1 Primary Controls

------------------------

$DATE (string)    Inserts a date string into the list file page header.

If $DATE() is specified, the actual date is inserted.

Date strings will be truncated to a maximum length of

11 characters.

Default is: no date string.

The control has no effect, when the $NOPAGING control has

been specified.

$DEBUG            Dummy. (For compatibility purposes only!)

$NODEBUG          Dummy. (For compatibility purposes only!)

$MOD51            Switches on the built-in 8051 special function register

and interrupt symbol definitions. (Default!)

$NOMOD51          Switches off the built-in 8051 special function register

and interrupt symbol definitions.

$PAGING           Switches on the page formatting in the list file.

(Default!)

$NOPAGING         Switches off the page formatting in the list file.

$PAGELENGTH (n)   Sets the list file page length to n lines.

(12 <= n <= 65535)

Default is n=64.

The control has no effect, when the $NOPAGING control has

been specified.

$PAGEWIDTH (n)    Sets the list file page width to n columns.

(72 <= n <= 255)

Default is n=132.

$PHILIPS          Switches on the Philips 83C75x family support option.

This disables the LJMP, LCALL, and MOVX instructions as

well as the XDATA and XSEG pseudo instructions. Generic

jumps and calls will always assemble to absolute addressing.

$SYMBOLS          Generates the symbol table at the end of the list file.

(Default!)

When the $XREF control is active, $SYMBOLS has no effect!

$NOSYMBOLS        Suppresses the symbol table at the end of the list file.

When the $XREF control is active, $NOSYMBOLS has no effect!

$NOTABS           Expands all tab characters in the list file output to

blanks.

$XREF             Generates a cross-reference listing instead of a symbol

table. Note that this slightly slows down assembly, and

consumes about 67 % more memory space!

$NOXREF           Generates a symbol table instead of a cross-reference

listing. (Default!)

Examples:   $NOMOD51              ;switch off 8051 SFR symbol definitions

$PAGELENGTH(60)       ;set page length to 60 lines per page

$PW(80)               ;set page width to 80 characters per line

$NOSYMBOLS            ;no symbol table required

$NOTABS               ;printer doesn't support tab characters

$DATE(2. 8. 95)       ;date of latest version

$XREF                 ;generate a cross-reference listing

III.8.2 General Controls

------------------------

$EJECT            Starts a new page in the list file.

The control has no effect, when the $NOPAGING control has

been specified.

$INCLUDE (file)   Includes an external source file into the assembler program

just behind the $INCLUDE statement. If the include file

cannot be found in the default directory, the include file

path, specified with the /INCLUDES command line option, is

searched. Include files may also be nested.

$NOLIST           After this control statement source code lines are no longer

listed, provided they do not contain errors, until the next

$LIST             statement occurs.

$TITLE (string)   Inserts a title string into the list file page header.

Titles may be truncated according to the specified (or

default) page width.

Default: ASEM-51 copyright information.

The control has no effect, when the $NOPAGING control has

been specified.

Examples:   $NOLIST               ;switch off listing

$INCLUDE (8052.MCU)   ;include 8052 SFR symbol definition file

$LIST                 ;switch on listing

$TITLE (Computer-Controlled Combustion Unit for Motorcycles)

$EJ                   ;new page with new title

III.9 Predefined Symbols

------------------------

For easy access to the 8051 special function register and interrupt

addresses, ASEM-51 has a number of predefined (built-in) DATA, BIT and

CODE symbols.

These predefined symbols can be switched off with the $NOMOD51 control.

For detailed information on symbols and addresses refer to Appendix C.

III.10 Conditional Assembly

---------------------------

To support easy configuration control and maintenance of 8051 application

programs, the following fife meta instructions have been implemented for

conditional assembly:

IF     <expr>

IFDEF  <symbol>

IFNDEF <symbol>

ELSE

ENDIF

For the IF statement, conditional assembly works as follows:

When the expression <expr> in the IF statement is not equal to 0 (TRUE),

then the statements 1 to n are assembled and the statements n+1 to n+m

are ignored.

IF <expr>

<statement 1>

<statement 2>       ;assembled when <expr> is TRUE

.

.

<statement n>

ELSE

<statement n+1>

<statement n+2>     ;assembled when <expr> is FALSE

.

.

<statement n+m>

ENDIF

Should <expr> be equal to 0 (FALSE), it is exactly vice versa! That means

the statements 1 to n are ignored and the statements n+1 to n+m are

assembled.

This works also, when the IF or ELSE branches contain no statements at all.

When the ELSE branch doesn't enclose any statements, the whole construction

can be simplified to a special case:

IF <expr>

<statement 1>

<statement 2>       ;assembled when <expr> is TRUE

.

.

<statement n>

ENDIF

Then the statements 1 to n are assembled, if <expr> is not equal to 0,

otherwise they are ignored.

The value of <expr> must be known on pass 1!

Example:     TARGET EQU 0    ;configuration:  1 for application board

;--------------  0 for evaluation board

IF TARGET

ORG 0         ;program start address of application board

ELSE

ORG 08000H    ;program start address of evaluation board

ENDIF

The IFDEF and IFNDEF statements are working quite similar:

IFDEF <symbol>

.

ELSE

.

ENDIF

When the <symbol> is defined in the program, the statements in the IFDEF

branch are assembled, and those in the ELSE branch (if any) are ignored.

When the <symbol> is not defined in the program, it is exactly vice versa!

IFNDEF <symbol>

.

ELSE

.

ENDIF

When the <symbol> is defined in the program, the statements in the IFNDEF

branch are ignored, and those in the ELSE branch (if any) are assembled.

When the <symbol> is not defined in the program, it is vice versa!

Example:    ;EVA_537 EQU 0      ;symbol undefined: 80C537 application board

;symbol defined:   80C537 evaluation board

IFNDEF EVA_537

CLOCK EQU 16     ;clock frequency of application board

CSEG  AT  0      ;program start address of application board

ELSE

CLOCK EQU 12     ;clock frequency of evaluation board

CSEG  AT  08000H ;program start address of evaluation board

ENDIF

Currently the program is configured for the application board version.

The <symbol> operands of the IFDEF and IFNDEF instructions must be either

undefined or defined on pass 1!

IF/IFDEF/IFNDEF...ELSE...ENDIF constructions may be nested to any depth!

IV. Compatibility with the Intel Assembler

==========================================

The ASEM-51 assembly language is a subset of the Intel standard, that

guarantees maximum compatibility with existing 8051 assembler sources.

It implements all 8051 instruction mnemonics as well as a useful subset

of the Intel pseudo instructions and assembler controls.

IV.1 Restrictions

-----------------

Since ASEM-51 generates an Intel-HEX file output instead of relocatable

object modules, the whole source code of an 8051 application program has to

reside in one single file. Consequently all pseudo instructions, that deal

with relocatable segments or external symbols, have not been implemented:

PUBLIC

EXTRN

SEGMENT

RSEG

The SET command cannot redefine special assembler symbols (e.g. registers)!

Macros are not supported!

Up to now only the following assembler controls and their abbreviations have

been implemented:

|  primary controls         abbrev. |  general controls    abbrev.

---------+-----------------------------------+-----------------------------

|  $DATE (<string>)          $DA    |  $EJECT               $EJ

|  $DEBUG                    $DB    |  $INCLUDE (<file>)    $IC

|  $NODEBUG                  $NODB  |  $LIST                $LI

|  $MOD51                    $MO    |  $NOLIST              $NOLI

Intel-   |  $NOMOD51                  $NOMO  |  $TITLE (<string>)    $TT

|  $PAGING                   $PI    |

controls |  $NOPAGING                 $NOPI  |

|  $SYMBOLS                  $SB    |

|  $NOSYMBOLS                $NOSB  |

|  $PAGELENGTH (<lines>)     $PL    |

|  $PAGEWIDTH (<columns>)    $PW    |

---------+-----------------------------------+-----------------------------

ASEM-51  |  $NOTABS                   -----  |

controls |  $PHILIPS                  -----  |

IV.2 Extensions

---------------

Assembler controls need not start in column 1, but may be preceded by any

number of blanks and tabs. Primary controls may also be preceded by

$INCLUDE statements, provided the corresponding include files are only

containing other control statements.

Character strings may also be enclosed in double quotes.

The DATA symbol for the special function register PCON is predefined.

The bit operator '.' is allowed in all expressions, not only in those that

have to match the segment type BIT.

To support easy conditional assembly, the following fife meta instructions

have been implemented:

IF     <expression>

IFDEF  <symbol>

IFNDEF <symbol>

ELSE

ENDIF

IV.3 Further Differences

------------------------

To make semantics unique, especially the precedence of unary operators in

expressions is slightly different. Furthermore, expressions with a bit

operation "." evaluate to a BIT type result, not to NUMBER. This is described

in detail in chapters "III.4 Expressions" and "III.7 Segment Type".

Except in DB instructions, the zero length string constant '' is illegal.

The $NOMOD51 control disables also the predefined CODE addresses.

The special assembler symbols AR0...AR7 are predefined for bank 0 before

the first USING statement occurs.

V. List File Format

===================

The ASEM-51 list file format has been designed to give the user as much

information about the generated code as possible.

Besides the source code listed, there are five basic layout structures

in the listing:

- the page header

- the file header

- the line headings

- the error diagnosis

- the symbol table or cross-reference listing

Normally every page of the listing starts with a page header as shown below:

ASEM-51 V1.2              Copyright (c) 1996 by W.W. Heinz               PAGE 1

It identifies the assembler, contains the copyright information and shows the

actual page number at the right margin. After the page header, source lines

are output in the list file format. When the maximum number of lines per page

is reached, another page header is output after a form feed character. When

the printer used doesn't work with form feeds, the page header can be

suppressed with the $NOPAGING control. The number of lines per page can be

adjusted to the paper format with the $PAGELENGTH control. The width of the

page header (and all other lines) can be set with the $PAGEWIDTH control.

The file header appears only on the first page. It identifies the assembler,

lists all input and output files and marks the columns for the line headings.

A typical file header is looking as shown below:

MCS-51 Family Cross Assembler   A S E M - 5 1   V 1.2

=====================================================

Source File:    DEMO.A51

Object File:    DEMO.HEX

List File:      DEMO.LST

Line  I  Addr  Code            Source

Directly after the file header starts the listing of the source code lines.

Every source code line is preceded by a line heading. The line heading

consists of four columns: line number, include file level, line address,

and generated code.

The column "Line" contains the source file line number. It is not necessarily

the local line number within the particular source file, but a global line

number, that is counted over the main source and all include files.

The column "I" flags the level of include file nesting. In the main source,

this column is empty. The first include file gets level 1. If this include

file includes another include file, this one gets level 2, and so on.

The column "Addr" shows the start address of the listed line in the currently

active segment (8051 address space). All addresses are represented as hex

numbers. The addresses in the CODE and XDATA segments are four-digit numbers.

Addresses in all other segments are two-digit numbers. For lines that cannot

be assigned to a particular segment, the "Addr" field is left blank.

The "Code" column may contain up to four bytes of generated code, which is

sufficient for all 8051 instructions. The code is listed in hex byte

quantities starting from the left margin of the "Code" column.

However, the code generated for DB and DW instructions may be longer than

four bytes. In these cases, the source code line is followed by additional

line headings until the whole code of the line is listed.

The "Code" column does not always contain code, that consumes space in the

8051 CODE segment. In contrast to many other assemblers, ASEM-51 lists the

evaluation results of all expressions that may appear in pseudo instructions

or assembler controls. These values are listed in hex representation at the

right margin of the "Code" column. The segment type of those expressions is

flagged with one single character at the left margin of the "Code" column:

C         CODE

D         DATA

I         IDATA

X         XDATA

B         BIT

N         number

The "Source" column finally contains the original source code line.

A typical source code listing is looking as follows:

Line  I  Addr  Code            Source

1:                                  ;A sample List File Demo Program

2:                                  ;-------------------------------

3:                          $NOMOD51                 ;no 8051 SFR

4:          N      004F     $PAGEWIDTH (79)          ;79 columns per line

5:                          $NOTABS                  ;expand tabs

6:          N        90             P1    DATA 090H  ;port 1 address

7:          B        93             INPUT BIT  P1.3  ;pulse input

8:

9:          N      8000             ORG  08000H      ;set location counter

10:    8000  80 20                   SJMP START       ;jump to start address

11:

12:    8002  01 07                   DB  1,7          ;define bytes

13:    8004  00 02 00 0C             DW  2,12,9       ;define words

8008  00 09

14:    800A  63 6F 66 66             DB  'coffeeright (c) 1996',0    ;string

800E  65 65 72 69

8012  67 68 74 20

8016  28 63 29 20

801A  31 39 39 36

801E  00

15:    801F  N      0003             DS  3            ;define space

16:

17:    8022  75 30 00        START:  MOV COUNT,#0     ;reset counter

18:    8025  30 93 FD        LLEVEL: JNB INPUT,LLEVEL ;wait for high

19:    8028  20 93 FD        HLEVEL: JB  INPUT,HLEVEL ;wait for low

20:    802B  05 30                   INC COUNT        ;count pulse

21:    802D  80 F6                   JMP LLEVEL       ;next pulse

22:

23:          N        30             DSEG AT 030H     ;internal RAM

24:      30  N        01     COUNT:  DS 1             ;counter variable

25:

26:                                  END

If an error is detected in a source line, its position is flagged with a ^

character as good as possible, and a comprehensive error message is inserted.

This is looking as shown below:

17:    8022  75 30 00        START:  MOV COUNT,#0     ;reset counter

18:    8025  30 93 FD        LLEVEL: JNB INPUT,LLEVEL ;wait for high

19:    8028  20 93 00        HLEVEL: JB  INPUT,HLEUEL ;wait for low

^

@@@@@ symbol not defined @@@@@

20:    802B  05 30                   INC COUNT        ;count pulse

21:    802D  80 F6                   JMP LLEVEL       ;next pulse

The error diagnosis at the end of program lists the register banks used,

and the total number of errors detected throughout the assembly:

register banks used:  0, 1, 3

187 errors detected

A register bank counts as "used", if the program had switched to that

bank with a USING instruction, or one of the special assembler symbols

AR0 ... AR7 has been used, while the bank was active. The message

register banks used:  ---

means, that no bank has been used explicitly, and that the program

code may, but need not, be register bank independent.

After the source code listing and error diagnosis, the symbol table or

cross-reference listing starts. By default, a symbol table is generated.

The symbol table lists all the symbols of a program in alphabetical order

with their symbol name, segment type, hex value and first definition line.

Predefined symbols are listed without a definition line number.

The symbol table listing can be suppressed with the $NOSYMBOLS control.

A typical symbol table listing is looking as shown below:

L I S T   O F   S Y M B O L S

=============================

SYMBOL                            TYPE     VALUE        LINE

------------------------------------------------------------

COUNT                             DATA        30          46

HLEVEL                            CODE      802E          35

INPUT                             BIT         93          12

LLEVEL                            CODE      802B          34

MY_PROGRAM                        MODULE                  14

P1                                DATA        90

QUANT                             NUMBER    0013          22

SP                                DATA        81

STACK                             IDATA       80          17

START                             CODE      8022          31

VOLTDC                            XDATA     D785          49

If the $XREF control is specified, a cross-reference listing is generated

instead of a symbol table. The corresponding cross-reference listing for

the above symbol table is looking as follows:

C R O S S - R E F E R E N C E - L I S T I N G

=============================================

SYMBOL                           TYPE     VALUE     DEFINED  REFERENCED

-------------------------------------------------------------------------------

COUNT                            DATA        30          46          32      39

42      43

HLEVEL                           CODE      802E          35          35

INPUT                            BIT         93          12          34      35

LLEVEL                           CODE      802B          34          34      40

MY_PROGRAM                       MODULE                  14

P1                               DATA        90                      12

QUANT                            NUMBER    0007          22          43

NUMBER    0013          37

SP                               DATA        81                      31

STACK                            IDATA       80          17          31

START                            CODE      8022          31          24

TRASH                            undef.    ----                      41

VOLTDC                           XDATA     D785          49          33

It lists all the symbols of the program in alphabetical order, with their

symbol name, all definitions including definition lines, segment types, and

numerical values. Furthermore, all symbol references are listed as well.

The SYMBOL column contains the symbol name, while the columns TYPE, VALUE,

and DEFINED may contain the segment types, numerical values, and definition

lines of one, more, or no symbol defintions.

Module names have the symbol type "MODULE", and symbols that have been

referenced but not defined, are flagged with "undef." in the TYPE column.

Starting from column REFERENCED up to the right margin, there is a number of

columns (depending on the page width), containing all line numbers of symbol

references (if any).

The cross-reference listing does not distinguish, whether multiple definitions

of, or references to a particular symbol are legal or not. For this, refer to

the error messages in the source listing.

VI. Support of 8051 Derivatives

===============================

Today a large number of 8051 derivatives is available that grows almost

monthly! They all use the same instruction set of the 8051 processor kernel,

but are different in peripheral components, to cover a wide range of

applications. The difference for the assembly language programmer is mainly

the varying set of special function registers and interrupt addresses.

It is always good practice to use the same SFR names in a microcontroller

application program that the manufacturer of the derivative used has defined.

For this the processor definition files *.MCU are provided. They all are

include files with the special function register definitions of a particular

8051 derivative. However, the predefined symbols of ASEM-51 must be switched

off prior to including the SFR definitions of another derivative as shown

below:

$NOMOD51

$INCLUDE (80C515.MCU)

This would switch off the predefined symbols of the 8051 and include the

register definitions of the 80C515 or 80C535 respectively.

Hence it is easy for the user to adapt ASEM-51 to a brandnew 8051 derivative!

All what he has to do is to write a corresponding include file with the SFR

definitions derived from the manufacturer's data sheet.

The name of every processor definition file is corresponding to the ROM

version of a particular derivative. Of course it also applies to the EPROM,

EEPROM, and ROM-less versions (if any) of that derivative.

By the way, the file 8051.MCU provided contains exactly the predefined

symbols of ASEM-51, because its internal symbol table has been generated

from it!

To switch ASEM-51 to the reduced instruction set of the Philips 83C75x

family of microcontrollers, the $PHILIPS control can be used.

Currently the following processor definition files are provided with ASEM-51:

Name        Manufacturer    Versions

---------------------------------------------------------------------

8051.MCU    Intel           8051, 8031, 8751BH

(and others)    8051AH, 8031AH, 8751H, 8051AHP, 8751H-8

80C51BH, 80C31BH, 87C51, 80C51BHP

8052.MCU    Intel           8052AH, 8032AH, 8752BH

SIEMENS         80513, 8352-5

80C52.MCU    Intel           80C52, 80C32

83C51F.MCU    Intel           83C51FA, 80C51FA, 87C51FA

83C51FB, 87C51FB, 87C51FC

83C152.MCU    Intel           80C152JA, 83C152JA, 80C152JB

80C152JC, 83C152JC, 80C152JD

83C452.MCU    Intel           83C452, 80C452

8044.MCU    Intel           8044AH, 8344AH, 8744AH

80512.MCU    SIEMENS         80512, 80532

80515.MCU    SIEMENS         80515, 80535, 80515K, 83515-4

80C515.MCU    SIEMENS         80C515, 80C535, 83C515H

83C515A.MCU    SIEMENS         83C515A-5, 80C515A

80C517.MCU    SIEMENS         80C517, 80C537

83C517A.MCU    SIEMENS         83C517A-5, 80C517A

C501.MCU    SIEMENS         C501-1R, C501-L

C502.MCU    SIEMENS         C502-2R, C502-L

C503.MCU    SIEMENS         C503-1R, C503-L

C511.MCU    SIEMENS         C511, C511A

C513.MCU    SIEMENS         C513, C513A, C513A-H

83C451.MCU    Philips         83C451, 80C451, 87C451

83C528.MCU    Philips         83C528, 80C528, 87C528, 83C524, 87C524

83CE528, 80CE528, 89CE528

83C550.MCU    Philips         83C550, 80C550, 87C550

83C552.MCU    Philips         83C552, 80C552, 87C552

83C562.MCU    Philips         83C562, 80C562

83C652.MCU    Philips         83C652, 80C652, 87C652

83C654, 87C654, 83CE654, 80CE654

83C750.MCU    Philips         83C750, 87C750

83C751.MCU    Philips         83C751, 87C751

83C752.MCU    Philips         83C752, 87C752

83C851.MCU    Philips         83C851, 80C851

83C852.MCU    Philips         83C852

80C521.MCU    AMD             80C521, 80C541, 87C521, 87C541, 80C321

80C324.MCU    AMD             80C324

83C154.MCU    OKI             83C154, 80C154, 85C154VS

80C320.MCU    DALLAS          80C320, 87C320, 80C323, 87C323

COM20051.MCU    SMC             COM20051

89C1051.MCU    Atmel           89C1051, 89C2051

Appendix A

==========

ASEM-51 Error Messages

----------------------

A.1 Assembly Errors:

--------------------

Assembly errors apply to the consistency of the assembly language

program in syntax and semantics. If one of these errors is detected,

it is flagged in the list file, and program execution continues.

When assembly is finished, ASEM terminates with ERRORLEVEL 1:

address out of range                    The address of a jump or call

instruction cannot be reached with

the selected addressing mode.

attempt to divide by zero               During evaluation of an assembly time

expression, the assembler has to

divide by zero.

binary operator expected                In this position of an expression,

only binary operators are allowed.

comma expected                          There should be a ',' character in

the marked position.

commands after END statement            The END statement is followed by

further assembler statements.

constant out of range                   A numerical constant is greater

than 65535.

ENDIF statement expected                There are IF, IFDEF, or IFNDEF meta

instructions, which are not terminated

with an ENDIF instruction.

expression out of range                 The result of an expression is too

big or too small for that purpose.

file name expected                      There should be a valid file name

in this position.

illegal character                       A statement contains characters, which

are not allowed in MCS-51 assembly

language.

illegal constant                        There are syntax errors in a

numeric constant.

illegal control statement               A statement is starting with an

unknown keyword beginning with a $.

illegal operand                         In this position of an expression,

a valid operand had been expected.

illegal operator                        An expression contains a special

character or keyword instead of an

arithmetical or logical operator,

which is not allowed in that place.

illegal statement syntax                A statement contains a syntax element,

which is not allowed in this context.

invalid base address                    A DATA address that is not bit-

addressable has been used on the

left side of a '.' operator.

invalid bit number                      A number greater than 7 has been

used on the right side of a '.'

operator.

invalid instruction                     The instruction has previously been

disabled with the $PHILIPS control.

module name already defined             There are more than one NAME

statements in the program.

must be known on first pass             The result of an expression must

fully evaluate on pass 1 of assembly.

must be preceded by IF                  An ELSE or ENDIF meta instruction

occurs without a preceding IF,

IFDEF, or IFNDEF instruction.

no END statement found                  The program ends without an END

statement.

not allowed in BIT segment              Instruction is not allowed in a

BIT segment.

only allowed in BIT segment             Instruction is only allowed in a

BIT segment.

only allowed in CODE segment            Instruction is only allowed in a

CODE segment.

operand expected                        An instruction ends, before it is

syntactically complete.

phase error                             On pass 2, a symbol value has evaluated

to another value than on pass 1.

This is a serious, internal assembler

error, and should be reported to the

author immediately!

preceded by non-control lines           A primary control occurs after

statements that are no assembler

controls.

segment limit exceeded                  The location counter exceeds the

boundaries of the current segment.

segment type mismatch                   The segment type of an operand does

not match the type of the instruction.

string exceeds end of line              A character string is not properly

terminated with a quote.

symbol already defined                  Attempt to redefine a symbol, which

is already defined.

symbol name expected                    There should be a valid symbol name

in this position.

symbol not defined                      A symbol is referenced, which has

never been defined.

too many closing parentheses            An expression contains more closing

than opening parentheses.

too many opening parentheses            An expression contains more opening

than closing parentheses.

too many operands                       An instruction contains more operands

than expected.

unary operator expected                 In this position of an expression,

only unary operators are allowed.

A.2 Runtime Errors:

-------------------

Runtime errors are operational errors, or I/O errors.

If one of these errors is detected, it is flagged on the console,

and ASEM is aborting with ERRORLEVEL 2:

access denied                 No priviledge for attempted operation.

disk full                     No more free disk space.

disk write protected          Attempt to write to a write-protected disk.

drive not ready               Disk drive is off, or no media mounted.

duplicate file name           Attempt to overwrite an input or output file.

fatal I/O error               General (unknown) disk or device I/O error.

file not found                Source or include file not found.

invalid argument              Option has an illegal argument value.

no input file                 There is no file name in the command line.

no parameters                 ASEM has been invoked without any parameters.

out of memory                 Heap overflow!

path not found                Disk or directory not found.

too many open files           No more free file handles.

too many parameters           More than three file names have been specified.

unknown option                Option is not implemented.

Appendix B

==========

HEXBIN Error Messages

---------------------

B.1 Conversion Errors:

----------------------

Conversion errors apply to the consistency of Intel-HEX file and

program options. If one of these errors is detected, it is flagged

on the console, and HEXBIN is aborting with ERRORLEVEL 1:

checksum error                        Checksum is not correct.

data after EOF record                 Type 0 records after type 1 record.

file length out of range              /LENGTH option makes file too large.

fill-byte out of range                /FILL option defines byte value > 255.

hex file format error                 Certainly no Intel-HEX file.

illegal hex digit                     Character is no valid hex digit.

illegal record type                   Record type is none of 0 or 1.

invalid record length                 Record length doesn't match the record.

multiple EOF records                  More than one type 1 record.

no data records found                 File doesn't contain any type 0 records.

no EOF record found                   File ends without a type 1 record.

offset out of range                   /OFFSET option makes file too large.

record exceeds FFFFH                  Address space wrap around in record.

record exceeds file length            /LENGTH option made file too short.

B.2 Runtime Errors:

-------------------

Runtime errors are operational errors, or I/O errors.

If one of these errors is detected, it is flagged on the console,

and HEXBIN is aborting with ERRORLEVEL 2:

access denied                 No priviledge for attempted operation.

disk full                     No more free disk space.

disk write protected          Attempt to write to a write-protected disk.

drive not ready               Disk drive is off, or no media mounted.

duplicate file name           Attempt to overwrite an input or output file.

fatal I/O error               General (unknown) disk or device I/O error.

file not found                Intel-HEX file not found.

invalid argument              Option has an illegal argument value.

no input file                 There is no file name in the command line.

no parameters                 HEXBIN has been invoked without any parameters.

path not found                Disk or directory not found.

too many open files           No more free file handles.

too many parameters           More than two file names have been specified.

unknown option                Option is not implemented.

Appendix C

==========

Predefined Symbols

------------------

DATA Addresses:

---------------

P0          080H                        P1          090H

SP          081H                        SCON        098H

DPL         082H                        SBUF        099H

DPH         083H                        P2          0A0H

PCON        087H                        IE          0A8H

TCON        088H                        P3          0B0H

TMOD        089H                        IP          0B8H

TL0         08AH                        PSW         0D0H

TL1         08BH                        ACC         0E0H

TH0         08CH                        B           0F0H

TH1         08DH

BIT Addresses:

--------------

IT0         088H                        EA          0AFH

IE0         089H                        RXD         0B0H

IT1         08AH                        TXD         0B1H

IE1         08BH                        INT0        0B2H

TR0         08CH                        INT1        0B3H

TF0         08DH                        T0          0B4H

TR1         08EH                        T1          0B5H

TF1         08FH                        WR          0B6H

RI          098H                        RD          0B7H

TI          099H                        PX0         0B8H

RB8         09AH                        PT0         0B9H

TB8         09BH                        PX1         0BAH

REN         09CH                        PT1         0BBH

SM2         09DH                        PS          0BCH

SM1         09EH                        P           0D0H

SM0         09FH                        OV          0D2H

EX0         0A8H                        RS0         0D3H

ET0         0A9H                        RS1         0D4H

EX1         0AAH                        F0          0D5H

ET1         0ABH                        AC          0D6H

ES          0ACH                        CY          0D7H

CODE Addresses:

---------------

RESET      0000H                        EXTI1      0013H

EXTI0      0003H                        TIMER1     001BH

TIMER0     000BH                        SINT       0023H

Appendix D

==========

Reserved Keywords

-----------------

Special Assembler Symbols:

--------------------------

$                                   location counter

A                                   accumulator

AB                                  A/B register pair

AR0,AR1,AR2,AR3,AR4,AR5,AR6,AR7     direct register addresses

C                                   carry flag

DPTR                                data pointer

PC                                  program counter

R0, R1, R2, R3, R4, R5, R6, R7      registers

Instruction Mnemonics

---------------------

ACALL       DA          JNB         MUL         RR

ADD         DEC         JNC         NOP         RRC

ADDC        DIV         JNZ         ORL         SETB

AJMP        DJNZ        JZ          POP         SJMP

ANL         INC         LCALL       PUSH        SUBB

CALL        JB          LJMP        RET         SWAP

CJNE        JBC         MOV         RETI        XCH

CLR         JC          MOVC        RL          XCHD

CPL         JMP         MOVX        RLC         XRL

Pseudo Instructions

-------------------

AT          DATA        DSEG        IDATA       SET

BIT         DB          DW          ISEG        USING

BSEG        DBIT        END         NAME        XDATA

CODE        DS          EQU         ORG         XSEG

CSEG

Operators

---------

AND         GT          LOW         NE          SHL

EQ          HIGH        LT          NOT         SHR

GE          LE          MOD         OR          XOR

Assembler Controls

------------------

$DA         $LI         $NOMO       $NOXREF     $SB

$DATE       $LIST       $NOMOD51    $PAGING     $SYMBOLS

$DEBUG      $MO         $NOPAGING   $PAGELENGTH $TITLE

$DB         $MOD51      $NOPI       $PAGEWIDTH  $TT

$EJ         $NODB       $NOSB       $PHILIPS    $XR

$EJECT      $NODEBUG    $NOSYMBOLS  $PI         $XREF

$IC         $NOLI       $NOTABS     $PL

$INCLUDE    $NOLIST     $NOXR       $PW

Meta Instructions

-----------------

ELSE        ENDIF       IF          IFDEF       IFNDEF

Appendix E

==========

Specification of the Intel-HEX Format

-------------------------------------

This object file format is supported by many cross assemblers, utilities,

and most EPROM blowers.

An Intel-HEX file is a 7-bit ASCII text file, that contains a sequence of

data records and an end record. Every record is a line of text that starts

with a colon and ends with CR and LF.

Data records contain up to 16 data bytes, a 16-bit load address, a record

type byte and an 8-bit checksum. All numbers are represented by upper case

ASCII-hex characters.

DATA RECORD:

Byte 1               colon (:)

2 and 3         number of binary data bytes for this record

4 and 5         load address for this record, high byte

6 and 7         load address  "    "     "     low byte

8 and 9         record type: 00  (data record)

10 to  x         data bytes, two characters each

x+1 to  x+2       checksum (two characters)

x+3 to  x+4       CR and LF

A typical data record looks like

:10E0000002E003E4F588758910F58DF58BD28E302A

The end record is the last line of the file.

In principle it is structured like a data record, but the number of data

bytes is 00, the record type is 01 and the load-address field is 0000.

END RECORD:

Byte 1               colon (:)

2 and 3         00   (number of data bytes)

4 and 5         00   (load address, high byte)

6 and 7         00   (load address, low byte)

8 and 9         record type: 01  (end record)

10 and 11        checksum (two characters)

12 and 13        CR and LF

The typical END record looks like

:00000001FF

The checksum is the two's complement of the 8-bit sum, without carry, of

the byte count, the two load address bytes, the record type byte and all

data bytes.

Appendix F:

===========

The ASCII Character Set

-----------------------

hex |   00   10   20   30   40   50   60   70

-----+------------------------------------------

0  |  NUL  DLE         0    @    P    `    p

1  |  SOH  DC1    !    1    A    Q    a    q

2  |  STX  DC2    "    2    B    R    b    r

3  |  ETX  DC3    #    3    C    S    c    s

4  |  EOT  DC4    $    4    D    T    d    t

5  |  ENQ  NAK    %    5    E    U    e    u

6  |  ACK  SYN    &    6    F    V    f    v

7  |  BEL  ETB    '    7    G    W    g    w

8  |   BS  CAN    (    8    H    X    h    x

9  |   HT   EM    )    9    I    Y    i    y

A  |   LF  SUB    *    :    J    Z    j    z

B  |   VT  ESC    +    ;    K    [    k    {

C  |   FF   FS    ,    <    L    \    l    |

D  |   CR   GS    -    =    M    ]    m    }

E  |   SO   RS    .    >    N    ^    n    ~

F  |   SI   US    /    ?    O    _    o  DEL

Appendix G:

===========

Literature

----------

Ordering Number

Intel:        MCS-51 Macro Assembler User's Guide

8-Bit Embedded Controllers 1990

SIEMENS:      SAB 80512/80532 User's Manual                B2-B3808-X-X-7600

SAB 80515/80535 User's Manual                B2-B3976-X-X-7600

SAB 80C515/80C535 Data Sheet

SAB 80C515A/83C515A-5 Addendum             B158-H6613-X-X-7600

SAB 80C515A/83C515A-5 Data Sheet           B158-H6605-X-X-7600

SAB 80C517/80C537 User's Manual            B258-B6075-X-X-7600

SAB 80C517A/83C517A-5 Addendum             B158-H6612-X-X-7600

SAB 80C517A/83C517A-5 Data Sheet           B158-H6581-X-X-7600

SIEMENS Microcontrollers Data Catalog      B158-H6569-X-X-7600

SAB 80513/8352-5 Data Sheet                B158-B6245-X-X-7600

SAB-C501 User's Manual                     B158-H6723-G1-X-7600

SAB-C502 User's Manual                     B158-H6722-G1-X-7600

SAB-C503 User's Manual                     B158-H6650-G1-X-7600

Application Notes and User Manuals, CD     B193-H6900-X-X-7400

Philips:      PCB83C552 User's Manual

PCB83C552, PCB80C552 Development Data

8051-Based 8-Bit Microcontrollers - Data Handbook 1994

AMD:          Eight-Bit 80C51 Embedded Processors - Data Book 1990

OKI:          MSM80C154, MSM83C154 User's Manual

Microcontroller Data Book, 5th Edition 1990

DALLAS:       High-Speed Micro User's Guide, V1.3 January 1994        011994

DS80C320 High-Speed Micro, PRELIMINARY                  081793

Andreas Roth: Das MICROCONTROLLER Kochbuch, 1992, iWT     ISBN 3-88322-225-9

Appendix H:

===========

Trademarks

----------

ASEM-51 is a trademark of W.W. Heinz.

MCS-51 is a trademark of Intel Corporation.

Turbo-Pascal and Borland-Pascal are trademarks of Borland International, Inc.

Turbo C++ and Borland C++ are trademarks of Borland International, Inc.

IBM-PC, IBM-XT, IBM-AT and OS/2 are trademarks of IBM Corporation.

MS-DOS and Windows are trademarks of Microsoft Corporation.

Novell DOS is a trademark of Novell, Inc.

BRIEF is a trademark of SDC Partners II L.P.

All device codes of 8051 derivatives are trademarks of the manufacturers.

Other brand and product names are trademarks of their respective holders.

Appendix I:

===========

8051 Instructions in numerical Order

------------------------------------

Abbreviations:      direct   =   8-bit DATA address in internal memory

const8   =   8-bit constant in CODE memory

const16  =  16-bit constant in CODE memory

addr16   =  16-bit long CODE address

addr11   =  11-bit absolute CODE address

rel      =  signed 8-bit relative CODE address

bit      =   8-bit BIT address in internal memory

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

00      NOP                           1                    1

01      AJMP    addr11                2                    2

02      LJMP    addr16                3                    2

03      RR      A                     1                    1

04      INC     A                     1              P     1

05      INC     direct                2                    1

06      INC     @R0                   1                    1

07      INC     @R1                   1                    1

08      INC     R0                    1                    1

09      INC     R1                    1                    1

0A      INC     R2                    1                    1

0B      INC     R3                    1                    1

0C      INC     R4                    1                    1

0D      INC     R5                    1                    1

0E      INC     R6                    1                    1

0F      INC     R7                    1                    1

10      JBC     bit, rel              3                    2

11      ACALL   addr11                2                    2

12      LCALL   addr16                3                    2

13      RRC     A                     1     CY       P     1

14      DEC     A                     1              P     1

15      DEC     direct                2                    1

16      DEC     @R0                   1                    1

17      DEC     @R1                   1                    1

18      DEC     R0                    1                    1

19      DEC     R1                    1                    1

1A      DEC     R2                    1                    1

1B      DEC     R3                    1                    1

1C      DEC     R4                    1                    1

1D      DEC     R5                    1                    1

1E      DEC     R6                    1                    1

1F      DEC     R7                    1                    1

20      JB      bit, rel              3                    2

21      AJMP    addr11                2                    2

22      RET                           1                    2

23      RL      A                     1                    1

24      ADD     A, #const8            2     CY AC OV P     1

25      ADD     A, direct             2     CY AC OV P     1

26      ADD     A, @R0                1     CY AC OV P     1

27      ADD     A, @R1                1     CY AC OV P     1

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

28      ADD     A, R0                 1     CY AC OV P     1

29      ADD     A, R1                 1     CY AC OV P     1

2A      ADD     A, R2                 1     CY AC OV P     1

2B      ADD     A, R3                 1     CY AC OV P     1

2C      ADD     A, R4                 1     CY AC OV P     1

2D      ADD     A, R5                 1     CY AC OV P     1

2E      ADD     A, R6                 1     CY AC OV P     1

2F      ADD     A, R7                 1     CY AC OV P     1

30      JNB     bit, rel              3                    2

31      ACALL   addr11                2                    2

32      RETI                          1                    2

33      RLC     A                     1     CY       P     1

34      ADDC    A, #const8            2     CY AC OV P     1

35      ADDC    A, direct             2     CY AC OV P     1

36      ADDC    A, @R0                1     CY AC OV P     1

37      ADDC    A, @R1                1     CY AC OV P     1

38      ADDC    A, R0                 1     CY AC OV P     1

39      ADDC    A, R1                 1     CY AC OV P     1

3A      ADDC    A, R2                 1     CY AC OV P     1

3B      ADDC    A, R3                 1     CY AC OV P     1

3C      ADDC    A, R4                 1     CY AC OV P     1

3D      ADDC    A, R5                 1     CY AC OV P     1

3E      ADDC    A, R6                 1     CY AC OV P     1

3F      ADDC    A, R7                 1     CY AC OV P     1

40      JC      rel                   2                    2

41      AJMP    addr11                2                    2

42      ORL     direct, A             2                    1

43      ORL     direct, #const8       3                    2

44      ORL     A, #const8            2              P     1

45      ORL     A, direct             2              P     1

46      ORL     A, @R0                1              P     1

47      ORL     A, @R1                1              P     1

48      ORL     A, R0                 1              P     1

49      ORL     A, R1                 1              P     1

4A      ORL     A, R2                 1              P     1

4B      ORL     A, R3                 1              P     1

4C      ORL     A, R4                 1              P     1

4D      ORL     A, R5                 1              P     1

4E      ORL     A, R6                 1              P     1

4F      ORL     A, R7                 1              P     1

50      JNC     rel                   2                    2

51      ACALL   addr11                2                    2

52      ANL     direct, A             2                    1

53      ANL     direct, #const8       3                    2

54      ANL     A, #const8            2              P     1

55      ANL     A, direct             2              P     1

56      ANL     A, @R0                1              P     1

57      ANL     A, @R1                1              P     1

58      ANL     A, R0                 1              P     1

59      ANL     A, R1                 1              P     1

5A      ANL     A, R2                 1              P     1

5B      ANL     A, R3                 1              P     1

5C      ANL     A, R4                 1              P     1

5D      ANL     A, R5                 1              P     1

5E      ANL     A, R6                 1              P     1

5F      ANL     A, R7                 1              P     1

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

60      JZ      rel                   2                    2

61      AJMP    addr11                2                    2

62      XRL     direct, A             2                    1

63      XRL     direct, #const8       3                    2

64      XRL     A, #const8            2              P     1

65      XRL     A, direct             2              P     1

66      XRL     A, @R0                1              P     1

67      XRL     A, @R1                1              P     1

68      XRL     A, R0                 1              P     1

69      XRL     A, R1                 1              P     1

6A      XRL     A, R2                 1              P     1

6B      XRL     A, R3                 1              P     1

6C      XRL     A, R4                 1              P     1

6D      XRL     A, R5                 1              P     1

6E      XRL     A, R6                 1              P     1

6F      XRL     A, R7                 1              P     1

70      JNZ     rel                   2                    2

71      ACALL   addr11                2                    2

72      ORL     C, bit                2     CY             2

73      JMP     @A+DPTR               1                    2

74      MOV     A, #const8            2              P     1

75      MOV     direct, #const8       3                    2

76      MOV     @R0, #const8          2                    1

77      MOV     @R1, #const8          2                    1

78      MOV     R0, #const8           2                    1

79      MOV     R1, #const8           2                    1

7A      MOV     R2, #const8           2                    1

7B      MOV     R3, #const8           2                    1

7C      MOV     R4, #const8           2                    1

7D      MOV     R5, #const8           2                    1

7E      MOV     R6, #const8           2                    1

7F      MOV     R7, #const8           2                    1

80      SJMP    rel                   2                    2

81      AJMP    addr11                2                    2

82      ANL     C, bit                2     CY             2

83      MOVC    A, @A+PC              1              P     2

84      DIV     AB                    1     CY    OV P     4

85      MOV     direct, direct        3                    2

86      MOV     direct, @R0           2                    2

87      MOV     direct, @R1           2                    2

88      MOV     direct, R0            2                    2

89      MOV     direct, R1            2                    2

8A      MOV     direct, R2            2                    2

8B      MOV     direct, R3            2                    2

8C      MOV     direct, R4            2                    2

8D      MOV     direct, R5            2                    2

8E      MOV     direct, R6            2                    2

8F      MOV     direct, R7            2                    2

90      MOV     DPTR, #const16        3                    2

91      ACALL   addr11                2                    2

92      MOV     bit, C                2                    2

93      MOVC    A, @A+DPTR            1              P     2

94      SUBB    A, #const8            2     CY AC OV P     1

95      SUBB    A, direct             2     CY AC OV P     1

96      SUBB    A, @R0                1     CY AC OV P     1

97      SUBB    A, @R1                1     CY AC OV P     1

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

98      SUBB    A, R0                 1     CY AC OV P     1

99      SUBB    A, R1                 1     CY AC OV P     1

9A      SUBB    A, R2                 1     CY AC OV P     1

9B      SUBB    A, R3                 1     CY AC OV P     1

9C      SUBB    A, R4                 1     CY AC OV P     1

9D      SUBB    A, R5                 1     CY AC OV P     1

9E      SUBB    A, R6                 1     CY AC OV P     1

9F      SUBB    A, R7                 1     CY AC OV P     1

A0      ORL     C, /bit               2     CY             2

A1      AJMP    addr11                2                    2

A2      MOV     C, bit                2     CY             1

A3      INC     DPTR                  1                    2

A4      MUL     AB                    1     CY    OV P     4

A5      illegal opcode

A6      MOV     @R0, direct           2                    2

A7      MOV     @R1, direct           2                    2

A8      MOV     R0, direct            2                    2

A9      MOV     R1, direct            2                    2

AA      MOV     R2, direct            2                    2

AB      MOV     R3, direct            2                    2

AC      MOV     R4, direct            2                    2

AD      MOV     R5, direct            2                    2

AE      MOV     R6, direct            2                    2

AF      MOV     R7, direct            2                    2

B0      ANL     C, /bit               2     CY             2

B1      ACALL   addr11                2                    2

B2      CPL     bit                   2                    1

B3      CPL     C                     1     CY             1

B4      CJNE    A, #const8, rel       3     CY             2

B5      CJNE    A, direct, rel        3     CY             2

B6      CJNE    @R0, #const8, rel     3     CY             2

B7      CJNE    @R1, #const8, rel     3     CY             2

B8      CJNE    R0, #const8, rel      3     CY             2

B9      CJNE    R1, #const8, rel      3     CY             2

BA      CJNE    R2, #const8, rel      3     CY             2

BB      CJNE    R3, #const8, rel      3     CY             2

BC      CJNE    R4, #const8, rel      3     CY             2

BD      CJNE    R5, #const8, rel      3     CY             2

BE      CJNE    R6, #const8, rel      3     CY             2

BF      CJNE    R7, #const8, rel      3     CY             2

C0      PUSH    direct                2                    2

C1      AJMP    addr11                2                    2

C2      CLR     bit                   2                    1

C3      CLR     C                     1     CY             1

C4      SWAP    A                     1                    1

C5      XCH     A, direct             2              P     1

C6      XCH     A, @R0                1              P     1

C7      XCH     A, @R1                1              P     1

C8      XCH     A, R0                 1              P     1

C9      XCH     A, R1                 1              P     1

CA      XCH     A, R2                 1              P     1

CB      XCH     A, R3                 1              P     1

CC      XCH     A, R4                 1              P     1

CD      XCH     A, R5                 1              P     1

CE      XCH     A, R6                 1              P     1

CF      XCH     A, R7                 1              P     1

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

D0      POP     direct                2                    2

D1      ACALL   addr11                2                    2

D2      SETB    bit                   2                    1

D3      SETB    C                     1     CY             1

D4      DA      A                     1     CY       P     1

D5      DJNZ    direct, rel           3                    2

D6      XCHD    A, @R0                1              P     1

D7      XCHD    A, @R1                1              P     1

D8      DJNZ    R0, rel               2                    2

D9      DJNZ    R1, rel               2                    2

DA      DJNZ    R2, rel               2                    2

DB      DJNZ    R3, rel               2                    2

DC      DJNZ    R4, rel               2                    2

DD      DJNZ    R5, rel               2                    2

DE      DJNZ    R6, rel               2                    2

DF      DJNZ    R7, rel               2                    2

E0      MOVX    A, @DPTR              1              P     2

E1      AJMP    addr11                2                    2

E2      MOVX    A, @R0                1              P     2

E3      MOVX    A, @R1                1              P     2

E4      CLR     A                     1              P     1

E5      MOV     A, direct             2              P     1

E6      MOV     A, @R0                1              P     1

E7      MOV     A, @R1                1              P     1

E8      MOV     A, R0                 1              P     1

E9      MOV     A, R1                 1              P     1

EA      MOV     A, R2                 1              P     1

EB      MOV     A, R3                 1              P     1

EC      MOV     A, R4                 1              P     1

ED      MOV     A, R5                 1              P     1

EE      MOV     A, R6                 1              P     1

EF      MOV     A, R7                 1              P     1

F0      MOVX    @DPTR, A              1                    2

F1      ACALL   addr11                2                    2

F2      MOVX    @R0, A                1                    2

F3      MOVX    @R1, A                1                    2

F4      CPL     A                     1              P     1

F5      MOV     direct, A             2                    1

F6      MOV     @R0, A                1                    1

F7      MOV     @R1, A                1                    1

F8      MOV     R0, A                 1                    1

F9      MOV     R1, A                 1                    1

FA      MOV     R2, A                 1                    1

FB      MOV     R3, A                 1                    1

FC      MOV     R4, A                 1                    1

FD      MOV     R5, A                 1                    1

FE      MOV     R6, A                 1                    1

FF      MOV     R7, A                 1                    1

Appendix J:

===========

8051 Instructions in lexical Order

----------------------------------

Abbreviations:      direct   =   8-bit DATA address in internal memory

const8   =   8-bit constant in CODE memory

const16  =  16-bit constant in CODE memory

addr16   =  16-bit long CODE address

addr11   =  11-bit absolute CODE address

rel      =  signed 8-bit relative CODE address

bit      =   8-bit BIT address in internal memory

i  =  register numbers 0 or 1

n  =  register numbers 0 thru 7

a  =  32 * m

m  =  the 3 most significant bits of an absolute address

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

11+a    ACALL   addr11                2                    2

24      ADD     A, #const8            2     CY AC OV P     1

26+i    ADD     A, @Ri                1     CY AC OV P     1

25      ADD     A, direct             2     CY AC OV P     1

28+n    ADD     A, Rn                 1     CY AC OV P     1

34      ADDC    A, #const8            2     CY AC OV P     1

36+i    ADDC    A, @Ri                1     CY AC OV P     1

35      ADDC    A, direct             2     CY AC OV P     1

38+n    ADDC    A, Rn                 1     CY AC OV P     1

01+a    AJMP    addr11                2                    2

54      ANL     A, #const8            2              P     1

56+i    ANL     A, @Ri                1              P     1

55      ANL     A, direct             2              P     1

58+n    ANL     A, Rn                 1              P     1

B0      ANL     C, /bit               2     CY             2

82      ANL     C, bit                2     CY             2

53      ANL     direct, #const8       3                    2

52      ANL     direct, A             2                    1

B6+i    CJNE    @Ri, #const8, rel     3     CY             2

B4      CJNE    A, #const8, rel       3     CY             2

B5      CJNE    A, direct, rel        3     CY             2

B8+n    CJNE    Rn, #const8, rel      3     CY             2

E4      CLR     A                     1              P     1

C2      CLR     bit                   2                    1

C3      CLR     C                     1     CY             1

F4      CPL     A                     1              P     1

B2      CPL     bit                   2                    1

B3      CPL     C                     1     CY             1

D4      DA      A                     1     CY       P     1

16+i    DEC     @Ri                   1                    1

14      DEC     A                     1              P     1

15      DEC     direct                2                    1

18+n    DEC     Rn                    1                    1

84      DIV     AB                    1     CY    OV P     4

D5      DJNZ    direct, rel           3                    2

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

D8+n    DJNZ    Rn, rel               2                    2

06+i    INC     @Ri                   1                    1

04      INC     A                     1              P     1

05      INC     direct                2                    1

A3      INC     DPTR                  1                    2

08+n    INC     Rn                    1                    1

20      JB      bit, rel              3                    2

10      JBC     bit, rel              3                    2

40      JC      rel                   2                    2

73      JMP     @A+DPTR               1                    2

30      JNB     bit, rel              3                    2

50      JNC     rel                   2                    2

70      JNZ     rel                   2                    2

60      JZ      rel                   2                    2

12      LCALL   addr16                3                    2

02      LJMP    addr16                3                    2

76+i    MOV     @Ri, #const8          2                    1

F6+i    MOV     @Ri, A                1                    1

A6+i    MOV     @Ri, direct           2                    2

74      MOV     A, #const8            2              P     1

E6+i    MOV     A, @Ri                1              P     1

E5      MOV     A, direct             2              P     1

E8+n    MOV     A, Rn                 1              P     1

92      MOV     bit, C                2                    2

A2      MOV     C, bit                2     CY             1

75      MOV     direct, #const8       3                    2

86+i    MOV     direct, @Ri           2                    2

F5      MOV     direct, A             2                    1

85      MOV     direct, direct        3                    2

88+n    MOV     direct, Rn            2                    2

90      MOV     DPTR, #const16        3                    2

78+n    MOV     Rn, #const8           2                    1

F8+n    MOV     Rn, A                 1                    1

A8+n    MOV     Rn, direct            2                    2

93      MOVC    A, @A+DPTR            1              P     2

83      MOVC    A, @A+PC              1              P     2

F0      MOVX    @DPTR, A              1                    2

F2+i    MOVX    @Ri, A                1                    2

E0      MOVX    A, @DPTR              1              P     2

E2+i    MOVX    A, @Ri                1              P     2

A4      MUL     AB                    1     CY    OV P     4

00      NOP                           1                    1

44      ORL     A, #const8            2              P     1

46+i    ORL     A, @Ri                1              P     1

45      ORL     A, direct             2              P     1

48+n    ORL     A, Rn                 1              P     1

A0      ORL     C, /bit               2     CY             2

72      ORL     C, bit                2     CY             2

43      ORL     direct, #const8       3                    2

42      ORL     direct, A             2                    1

D0      POP     direct                2                    2

C0      PUSH    direct                2                    2

22      RET                           1                    2

32      RETI                          1                    2

23      RL      A                     1                    1

33      RLC     A                     1     CY       P     1

Opcode  Mnemonic  Operands            Bytes     Flags     Cycles

----------------------------------------------------------------

03      RR      A                     1                    1

13      RRC     A                     1     CY       P     1

D2      SETB    bit                   2                    1

D3      SETB    C                     1     CY             1

80      SJMP    rel                   2                    2

94      SUBB    A, #const8            2     CY AC OV P     1

96+i    SUBB    A, @Ri                1     CY AC OV P     1

95      SUBB    A, direct             2     CY AC OV P     1

98+n    SUBB    A, Rn                 1     CY AC OV P     1

C4      SWAP    A                     1                    1

C6+i    XCH     A, @Ri                1              P     1

C5      XCH     A, direct             2              P     1

C8+n    XCH     A, Rn                 1              P     1

D6+i    XCHD    A, @Ri                1              P     1

64      XRL     A, #const8            2              P     1

66+i    XRL     A, @Ri                1              P     1

65      XRL     A, direct             2              P     1

68+n    XRL     A, Rn                 1              P     1

63      XRL     direct, #const8       3                    2

62      XRL     direct, A             2                    1