Command Line Options

--------------------

Door Source has numerous command line options in order to provide

flexibility with most computer systems. Here is a list:

LOCAL       - Forces Door Source to run in local mode. This

is really meant for testing purposes, but it

still reads the configuration file, but doesn't

read the bbs interface file.

NETWORK     - Forces file sharing to be active incase you

are using something other than SHARE to manage

file locking.

IRQ=n       - Lets you change the IRQ used for the com port

specified in the bbs interface file.

(See below for more information)

ADDR=&Hnnnn - Lets you change the address used for the com

port specified in the bbs interface file.

(See below for more information)

PS2         - Forces the PS/2 IRQs and addresses if a PS/2

isn't detected (Normally it is, but you can't

always tell!)

RBBS        - Forces Door Source into RBBS/QBBS mode

WILDCAT     - Forces Door Source into Wildcat 2.0 mode

DOORSYS     - Forces Door Source into 31-Line DOOR.SYS mode

PCB121      - Forces Door Source into PCBoard 11.8/12.x mode

with COM 1 active

PCB122      - Forces Door Source into PCBoard 11.8/12.x mode

with COM 2 active

PCB14       - (DEFAULT) Forces Door Source into PCBoard 14.x

mode

Door Source 4.0

Programmer's Manual - Page 15

When you use the IRQ or the ADDR option, you must include the other.

You can't use just the IRQ and not the ADDR, and you can't use just

the ADDR and not the IRQ. Here's a chart with the STANDARD IRQs

and addresses for the PS/2s and IBM PCs:

Non PS/2

Com port       IRQ     Address

1             4       &H3F8

2             3       &H2F8

3             4       &H3E8

4             3       &H2E8

5-8            4       &H3F8

PS/2

Com port       IRQ     Address

1             4       &H3F8

2             3       &H2F8

3             3       &H3220

4             3       &H3228

5             3       &H4220

6             3       &H4228

7             3       &H5220

8             3       &H5228