SOUND BLASTER 2.0 LATEST INFORMATION

====================================

This file  contains  information  that is not  available  during the

printing  of  the  manual.  Please  read all the sections before you

proceed with any installation procedure .

This file contains the following sections:

1. Software Installation

2. MIDI Files

3. Maximum Sampling Rates For Packed Voice Files

4. PLAYCMF Music Player

5. Trouble Shooting For Installing Windows 3.1 Drivers

1. SOFTWARE INSTALLATION

The   installation   program,  INST-HD.EXE,   will   modify   your

AUTOEXEC.BAT and rename your original file with a B~K extension.

The program  also  allows  you  to  add  a  command  that runs the

WINSETUP.EXE  program  to  your  Windows 3.1  WIN.INI  file.  This

program  (WINSETUP.EXE)  automatically  sets  up the Sound Blaster

Audio Drivers and Windows 3.1 applications when you run Windows.

The procedure for the installation is as follows.

a. Test  your  Sound Blaster card with the  program  TEST-SBC.EXE,

which can be found on DIsk #1.

b. Run the program INST-HD.EXE  from DOS prompt.  This program can

be found on Disk #1.

c. Select "Begin Installation" and follow  the instructions on the

screen.

d. Quit the installation program when it completes successfully.

e. REBOOT YOUR SYSTEM.

f. If you choose to have  the  installation  program  modify  your

WIN.INI  file, Sound  Blaster  Audio  Drivers  and  Windows 3.1

applications  will be set up.  Otherwise, follow the procedures

below.

i.   From the FILE menu in Program Manager, choose RUN.

ii.  Type  "C:\SB\WINSETUP" and press ENTER, assuming you  have

installed Sound Blaster software onto drive C.

iii. Follow the instructions on  the  screen  to  complete  the

installation.

iv.  Restart Windows.

NOTE

====

WINSETUP will NOT set up the drivers and applications properly  if

your  BLASTER  environment string is incorrectly set.  Please make

sure that the BLASTER environment string is set up properly before

executing  WINSETUP.EXE.  You  may  enter  "SET" at  DOS prompt to

check  the  environment  settings.  Refer to  your  Sound  Blaster

manual for more details on the BLASTER environment string.

In normal cases, WINSETUP.EXE only  needs  to be run once.  If you

happen to  change  the hardware settings of your  Sound  card, you

should  use  the  DRIVER  applet  in  CONTROL  PANEL to change the

settings  of  your  Windows drivers.  Refer to your Windows manual

for more details on the DRIVER applet.

2. MIDI FILES

When playing MIDI files with MMJBOX, a warning message in a dialog

box may appear.  The message is only an indication of whether  the

MIDI file conforms to Microsoft MIDI standard.

Even if the file conforms to  the standard, the  message may still

appear if the code defined by  Microsoft  is  not indicated in the

MIDI  file.  Please ignore  the  message if the MIDI files created

conforms to the standard.

For more  information  on  how  to  indicate the code in your MIDI

files, refer to the "Microsoft Windows Software Developer Kit".

For a start, you  can try out the MMJBOX application with the MIDI

file CANYON.MID which can be found in your Windows directory.  The

MIDI files provided in the sub-directory  \PLAYMIDI are created to

support the MIDI MAPPER and they comply fully to the Multimedia PC

rules.  When  you  use MMJBOX in WINDOWS 3.1 to play these  files,

make sure that  your MIDI MAPPER is set to either the SB EXT FM or

SB BASIC FM.  Using it with SB ALL FM will cause the song to sound

"cluttered".

Refer  to  the manual for more information on MIDI file format and

MIDI Mapper.

3. MAXIMUM SAMPLING RATES FOR PACKED VOICE FILES

Sound Blaster 2.0 implements  some  compression schemes for  voice

files.  These allow  space-savings of 2, 3 or  4  times, depending

on the scheme used (2:1, 3:1  or  4:1  compression, respectively).

This diversity in compression schemes results in different maximum

sampling rates for each  scheme.  The  valid sampling rates are as

follows:

8-bit (uncompressed)    4 KHz - 44.1 KHz

2:1 compression         4 KHz - 12 KHz

3:1 compression         4 KHz - 13 KHz

4:1 compression         4 KHz - 11 KHz

As a result  of  these limits,  you cannot pack voice  files  that

have sampling  rates  outside  these  bounds.

4. PLAYCMF MUSIC PLAYER

PLAYCMF.EXE is a FM music  player program that plays ".CMF"  music

files.

Before  running  PLAYCMF, you  must  first load Sound Blaster's FM

Music Driver - SBFMDRV.COM.

SBFMDRV is a memory  resident  driver  that  is  found in the  \SB

sub-directory  after  you  install the software to your hard disk.

To  install  SBFMDRV, change  directory  to  \SB, type SBFMDRV and

press Enter.

Usage:

PLAYCMF music-filename  [/Q] [/S="command-line"]

Where:

"music-file" is the .CMF file to play

/Q :    set quiet screen mode

/S :    execute the DOS command-line specified.

You  may  play music in the background and use the "/S=" parameter

to "Shell" to execute another program.

We  enclosed  some  .CMF songs for you as samplers.  Watch out for

more  .CMF songs, Sing-Alongs  and  other  applications  that  are

coming on the way.

5. TROUBLE SHOOTING FOR INSTALLING WINDOWS 3.1 DRIVERS

The   installation   of   Windows   3.1   drivers  will   be  done

automatically when you run the  program WINSETUP.EXE.  However, if

you  prefer  to  install these drivers  manually, please  refer to

the Sound  Blaster manual or your  Microsoft  Windows  3.1 manual.

If  somehow you are having  problems  installing the drivers, here

are some points to check before you call Technical Support.

a. If you installed Windows 3.1 using the "Express  Installation,"

Windows  scanned  your  hard disk for programs to be installed.

As a result, older  versions  of the Jukebox were installed  if

they  were  on  your  hard  disk  at  the time of installation.

Please remove these icons.  To do this, click on the icon  once

to select it, then  press the delete key.  Windows will ask you

for confirmation.  Repeat for each icon.

b. Open  the  Driver applet in Control panel.  You should at least

have the following driver names in the Installed Drivers list:

Creative Sound Blaster *** MIDI Synthesizer

Creative Sound Blaster *** Wave and MIDI

MIDI Mapper

Timer

[MCI] CD Audio  ;this is only for owners of CD-ROM drives

[MCI] MIDI Sequencer

[MCI] Sound

where *** correspond to the type of card you are having.

If one or  more of these drivers do not exist, you must install

the driver(s)  using  the  Driver  applet in Control Panel (See

the Windows manual for more details).

c. Check your SYSTEM.INI by choosing RUN from the File menu of the

Program Manager, and typing SYSEDIT  in  the  command line box.

Examine the [drivers] and  [sndblst.drv] sections.  They should

look something like this:

[drivers]

Timer=timer.drv

Midimapper=midimap.drv

MIDI=sbfm.drv

Wave=sb20snd.drv

MIDI1=sb20snd.drv

[sndblst.drv]

port=220

int=7

dmachannel=1

d. If  Windows  refuses  to install a device driver or you get the

error  message "Device not found" when playing a Wave  or  MIDI

file  or  a CD, be sure  you have the following settings in the

[mci] section of SYSTEM.INI

[mci]

WaveAudio=mciwave.drv

Sequencer=mciseq.drv

CDAudio=mcicda.drv  ;this is only for owners of CD-ROM drives

If one or more of these do  not  exist, you  must  install  the

driver(s) using the Driver applet  in  Control  Panel (See  the

Windows manual for instructions).  Do not type  in the  entries

into the SYSTEM.INI because the files have to be installed into

your SYSTEM directory by Windows.

*********  End of README  *********