SMCPWRN driver for Intel [tm] Windows NT [tm] 3.5

- Version V2.2 -

1. Copyright Notice:

First Edition, April 1994

Updated, December 1994

Updated, February 1995

Digital Equipment Corporation makes no representations that

the use of its products in the manner described in this

publication will not infringe on existing or future patent

rights, nor do the descriptions contained in this

publication imply the granting of licenses to make, use, or

sell equipment or software in accordance with the

description.

Copyright Digital Equipment Corporation 1994.

All rights reserved.

Do not redistribute the driver files.

Intel is a trademark of Intel Corporation.

Windows, Windows NT, and NT are trademarks of

Microsoft Corporation.

Microsoft, MS, MS-DOS are registered trademarks of

Microsoft Corporation.

Alpha, DEC, DECchip, DECnet, Digital, PATHWORKS,

ThinWire, and the DIGITAL logo, are  trademarks of

Digital Equipment Corporation.

All other trademarks and registered trademarks are the

property of their respective holders.

2. Overview:

This file explains how to install the Windows NT driver

for x86-based platforms.  The driver is currently supported for

the SMC 8432 PCI Ethernet Controller and the SMC 9332 PCI

Fast Ethernet Controller.

3. Requirements:

A.  Hardware Requirements:

o A personal computer with 80386 or higher microprocessor

and EISA or PCI bus.

o A DECchip 21040-based Ethernet adapter or a DECchip 21140-based Fast

Ethernet adapter for that bus.

B.  Software Revision Requirements:

Windows NT Version 3.5 or later

4. Files and directories:

Table 1:  The Server Directory Files

---------------------------------------------------------------------

INSTALL.DOC       This file.

---------------------------------------------------

SMCPWRN.SYS       The driver.

---------------------------------------------------

OEMSETUP.INF      The driver's setup file.

---------------------------------------------------------------------

5. Installing the Hardware Boards:

A. Installing the SMC 8432 PCI Ethernet Controller:

To install the SMC 8432 see the board's User Guide.

B. Installing the SMC 9332 PCI Fast Ethernet Controller:

To install the SMC 9332 see the board's User Guide.

6. Installing the SMCPWRN NT Driver Software:

1. From the Windows NT Main Menu, select the Control Panel icon.

2. From the Control Panel menu select the Network icon.

If a dialog box asks you "Do you want to install NT

Network?" select YES and Continue. Then place

the Windows NT CDROM in the CD-ROM reader.

Select the drive that matches the configuration of your system

and complete the install.

3. You will see several messages. To add a network adapter, select Yes

and then Continue

4. For each additional message, select continue.

5. From the Add Network Adapter dialog box, scroll through the menu to

select Other.

6. When the Insert Diskettes menu prompts you for a disk, insert the

SMCPWRN driver installation diskette for Windows NT into drive A.

Then select OK.

7. Select your option

[SMC 8432 EtherPower PCI Ethernet Adapter       ]

[SMC 8434 EtherPower2 PCI Ethernet Adapter      ]

[SMC 9332 EtherPower10/100 PCI Ethernet Adapter ]

from the Select OEM Option box, then select OK.

8. Modify the default distribution path in the Windows NT [tm]

Setup box to point to the correct drive and path, then click

on [Continue]

9. For PCI options, a Dialog box pops up to select the:

DataRate:

------------------------------------------------------------------------

SMC 8432

-------------------------------------------------------------------------

o AutoDetect                  In AutoDetect mode the driver

checks first the Twisted Pair port.

If the Link Test fails the driver

checks the BNC Thinwire port. If no

carrier is detected the driver switches

to the AUI Thickwire port.

o Twisted_Pair

o Twisted_Pair Full_Duplex    To connect to Full Duplex hubs

o Twisted_Pair No_Link_Test   To connect hubs not supporting

the Link Integrity Test

o BNC - Thinwire              BNC connector

o AUI - Thickwire             15_pin connector

------------------------------------------------------------------------

SMC 9332

-------------------------------------------------------------------------

o 100Mbps                    To connect 100BaseT Ethernet hubs

o 100Mbps Full_Duplex        To connect 100BaseT Full Duplex hubs

o 10Mbps                     To connect 10BaseT Ethernet hubs

10. Upon completion, one of the following adapters

[ [n]SMC 8432 EtherPower PCI Ethernet Adapter             ]

[ [n]SMC 8434 EtherPower2 PCI Ethernet Adapter            ]

[ [n]SMC 9332 EtherPower10/100 PCI Fast Ethernet Adapter  ]

appears in the Installed Adapter Cards list of the

Network Setting box (n is the adapter number).

11. Click on [Close] to complete the Network Setting

initialization.

12. If needed, modify the values in the registry (see section 7 below).

13. Shutdown and reboot the system.

14. Installing multiple PCI boards:

Repeat the procedure above.

7. Parameters

To overwrite the driver default values, add or edit the

following values in the Registry SMCPWRN_<adapter_number> Parameters

subtree:

NetworkAddress = REG_SZ { xx-xx-xx-xx-xx-xx }

Mandatory if no Serial ID PROM is plugged in

Overwrite the Serial ID PROM if plugged in

NetworkAddress 00-00-00-00-00-00 is ignored

SoftwareCRC = REG_DWORD { 0 |  1 }  (rev 1.x, 2.0 & 2.2 only)

0 The CRC is generated by the adapter

1 (default) The CRC is generated by software for

transmit packets bigger than the transmit threshold.

NOTE: This option affects performances and could be

disabled if the adapter does not report transmit

underflows in its statistic counters

The following values allow to overwrite the SMCPWR default setting.

Refer to the DECchip 21040 & DECchip 21140 Engineering specifications

for detailled description:

ReceiveBuffers = REG_DWORD {8..64} (default 16)

Number of buffers allocated by the

driver in the Receive ring.

BurstLength = REG_DWORD  { 1 | 2 | 4 | 8 | 16 | 32 }

CSR0<PBL>  1  doubleword

2  doublewords

4  doublewords

8  doublewords   (default)

16  doublewords

32  doublewords

FifoArbitration = REG_DWORD { 0 | 1 }

CSR0<BAR>   0 Rcv/Txm Arbitration (default)

1 Round_Robin

StopBackoffCounter = REG_DWORD { 0 | 1 }

CSR6<SB>    0  Disabed (Default)

1  Enabled

BackPressure = REG_DWORD { 0 | 1 }

CSR6<BP>    0  Disabed (default)

1  Enabled

CaptureEffect = REG_DWORD { 0 | 1 }

CSR6<CA>    0  Disabed (Default)

1  Enabled

8. Error messages

The error messages are logged in the Entry Log and can be seen using the Entry Log Viewer (in

WNT's Administration Tools group).

Each error message has a standard Ndis error code and an driver's additional code.

The additional codes are:

DC21X4_ERRMSG_REGISTRY            0x01

DC21X4_ERRMSG_ALLOC_MEMORY        0x02

DC21X4_ERRMSG_SROM                0x03

DC21X4_ERRMSG_MEDIA               0x04

DC21X4_ERRMSG_LOAD_CAM            0x05

DC21X4_ERRMSG_SYSTEM_ERROR        0x06

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT  0x07

8.1 Driver initialization errors:

o NDIS_ERROR_CODE_UNSUPPORTED_CONFIGURATION

DC21X4_ERRMSG_REGISTRY

The AdapterType Registry's key is missing or its value is unsupported by the adapter board

o NDIS_ERROR_CODE_ADAPTER_NOT_FOUND

No board matching the AdapterCFID Registry's key value was found

plugged into the PCI bus or in the slot specified in the Registry

o NDIS_ERROR_CODE_OUT_OF_RESOURCES

DC21X4_ERRMSG_ALLOC_MEMORY

Not enough memory to allocate space for the Adapter data block

o NDIS_ERROR_CODE_INVALID_VALUE_FROM_ADAPTER

DC21X4_ERRMSG_SROM

Invalid data read from the adapter's serial ROM

o NDIS_ERROR_CODE_NETWORK_ADDRESS

No readable burnt_in or software configured station address

o NDIS_ERROR_CODE_UNSUPPORTED_CONFIGURATION

DC21X4_ERRMSG_MEDIA

The selected media port is not supported by the board.

o NDIS_ERROR_CODE_INTERRUPT_CONNECT

Interrupt initialization failed

o NDIS_ERROR_CODE_TIMEOUT

DC21X4_ERRMSG_LOAD_CAM

DC21X4 CAM initialization failed

8.2 Driver run time errors:

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_SYSTEM_ERROR

System_Error interrupt (bus parity error)

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT

Transmit Jabber timer expired