************************************************************************

AUTO-REPEAT FOR THE H19/H89

************************************************************************

For those interested in a low-bucks approach to auto-repeat on

your H19/88/89, take a gander at this. Word processing aficianados will

appreciate the ability to move the cursor around without having to

hold down the REPEAT key.

THEORY OF OPS

Diodes D1-D10 monitor the outputs of keyboard plug pins

14,16,18,20,22,24,26,28,30,32 and clamp the outputs at 5 volts. This

is input to a dual retriggerable multivibrator(C1), which sets up the

delay. If, after a delay, the output at pins 4 and 5 are still low,

they feed a PNP transistor which puts a low out on the repeat

line (keyboard pin 31).

ASSEMBLY

The circuit uses 1 chip, 1 transistor, 4 resistors, 12 diodes, 2

capacitors and 1 variable potentiometer.

Point to point wiring was used, with the entire circuit mounted on a

1.5" by 3" piece of perfboard attached to the underside of the

keyboard with stand-offs and sheet metal screws used to secure the

board. The inputs to the diodes were soldered directly to the lands on

the keyboard (plug pins 14,16,18,20,22,24,26,28,30,32), as was the

output going to the repeat function (plug pin 31) using small gauge

wire. The +5 volts and ground were taken directly off the 5 volt

regulator using alligator type connectors in the event keyboard

removal is necessary. The delay time is established by potentiometer

P1. Adjust P1 until you find the optimum delay for your typing speed.

There are times this feature may be a liability, especially in game

playing. Some games have you hold down a key to move a piece on the

screen, this circuit will not inhibit any functions but the keyboard

clicking will drive you crazy. Since I have my +5v attached with

alligator clips, I just pull off one lead to stop auto repeat. An

alternative would be a small switch located on the back of the case.

Be careful when attaching the circuit board to the underside of

the keyboard. There are lands on the reverse side that are difficult

to see.

I have found a "bug" of sorts.	After I turn on my system it

takes a minute or two before the circuit warms up and starts

repeating. However, this is usually no problem because it takes me a

few minutes to go through the bootup procedure.

********************************************************************************

|	      KEYBOARD		     |

|				     |

|	      (P 301)		     |

|				     |

|  14 16 18 20 22 24 26 28 30 32      |

`-------------------------------------'

|  |  |  |  |  |  |	|  |  |

/-D1---|<-----------'  |  |  |  |  |  |	|  |  |

|-D2-----|<------------'  |  |  |  |  |	|  |  |

|-D3-------|<-------------'  |  |  |  |	|  |  |

|-D4---------|<--------------'  |  |  |	|  |  |

_________|-D5-----------|<---------------'  |  |	|  |  |

|   |    |-D6-------------|<----------------'  |	|  |  |

|   *    |-D7---------------|<-----------------'	|  |  |

|   *R3  |-D8-----------------|<------------------'  |  |

|   *    |-D9-------------------|<-------------------'  |

|   |    \-D10--------------------|<--------------------'

|  Vgrd

|

| 			       jumper  13 to 10

| 	   Vcc		       ________________

| 	   |		      | 	       |

| 	   |  R1    C1	      | 	  Vcc  |      Vgrd

| 	   |-***-+--)|-+      | 	  |    |      |

| 	   |	 |     |      | 	  |    |      |

| 	   |	 |     |      |    NU	  |    |      |

| 	   ____________________________________________

| 	  |16	 15    14    13    12	 11    10     9|

| 	  |					       |

| 	  |		   C1  (74123)		       |

| 	  |					       |

| 	  | 1	  2	3     4     5	  6	7     8|

| 	   --------------------------------------------

| 	   |	  |	|     |     |	  |	|     |

| 	   |	  |	|     |     |	  |	|     |

| 	   Vgrd   |	Vcc   |     |	  | C2	|     Vgrd

| 		  |	      |     |	  `-|(--|

`-------------------'	      |     |		`--*@*-----Vcc

D11     |     |		   P1

Vgrd---***--+----\-----|<------'     |

R4   |	 \----|<------------'	 Vgrd

|	      D12	    _______|

|			|  /C

|		       B| /

`--------------------|<

| *

Q1 |  \E___________>REPEAT

|	(pin 31

R2  |	 on keyboard)

Vcc--***-'

SYMBOL DEFINITIONS:

-------------------

---***---	RESISTOR

---*@*---	VARIABLE POT.		     |	/Collector

---|(---	CAPACITOR		Base | /

---|<---	DIODE		       ------|<      TRANSISTOR

Vcc		+5V D.C.		     |	*

---|(---	CAPACITOR		     |	 \Emitter

Vgrd		GROUND

________________

|	      |  I. C. CHIP

|______________|

PARTS LIST

----------

D1-D12  SIGNAL DIODES, 1N914

R1	  27K  OHM RESISTOR

R2	  400	"     "

R3,R4   1K	"     "

P1	  100K MINI POTENTIOMETER

C1	  .01 uf DISK CAPACITOR

C2	  10  uf CAPACITOR

Q1	  PNP TRANSISTOR, 2N3638 (or equivelant)

C1	  DUAL RE-TRIGGERABLE ONE-SHOT, SN74123

####################################################################

There are numerous ways to implement auto-repeat in your

computer, each with varying degrees of sophistication. This circuit

was chosen as the most direct solution to the problem. The design

credit goes to Ronny Toups, an inveterate hardware hacker who would

rather tinker than eat, drink, or sleep. If you have any questions, try

reaching me at home (214-385-1367), or leave a message on the DF/W HUG

BB. (214-742-1380)

ANDY PALUCH

Dallas/Fort Worth Heath Users' Group

EOF  AUTORPT.DOC

#the DF/W HU