Program name:  GC1000 V5.6

Author:  Jim Schuster

Introduction: GC1000 V5.6 is a MACRO-86 program that interfaces the Heath

GC-1000 Most Accurate Clock to the H/Z100 using either ZDOS 1.25 or MS-DOS

2.13.  There are many new features of this version that were not contained

in my fisrt version (V1.0) as published in REMark. Volume 5, Issue 1, pages

73-75.  (1) The ability to set and read the current year in the directory

entry, thus ignoring the GC-1000's DIP switch setting.  (2) Detection of the

transmission of a '?' from the GC-1000 in the 1/10 second digit, and the

subsequent zeroing of that digit.  (3) Detection and correction of the

months-with-30-days error as received from the clock.*  (4) Transparent

port configuration-- the selected port is configured as required and restored

to its original configuration at program termination.  This allows port

sharing with other devices such as a MODEM.   *(Not in all clocks, but it is

in mine.)

Requirements: Assembly of this program was under ZDOS 1.25 using MASM V1.07,

LINK V1.10 and EXE2BIN V1.5.  The supplied COM files work under ZDOS 1.25 and,

with many thanks to Arthur Calhoun of Huntington, NY, it has also been tested

under MS-DOS 2.13.  Also required (for any re-assembly) are DEFMS.ASM and

DEFCHR.ASM from ZDOS 1.25.

Required hardware is any H/Z100, using either serial port A or

B, the Heath GC-1000, and a 2 wire serial cable.  Both cable pin-outs are given

in the program listing, along with the required GC-1000's internal DIP switch

settings.

Included files are GC1000A.COM (serial port A at 9600 baud), and GC1000B.COM

(serial port B at 9600 baud).  GC1000.ASM is the source code which has been

preset to serial port B at 9600 baud, as both the port and baud rate are

selectable only at time of assembly.  The desired COM file must be renamed

GC1000.COM in order to read the current year.  READGC.DOC gives as summary

of the required set-up and use.  A ZBASIC program, GC1000.BAS, is also

included.  (A shortened version of this was tested on a PC.)

Program content:  GC1000.COM begins by saving the existing AUX: port

configuration and swapping it for the necessary configuration to run the

clock.  (It should be emphasized here that the AUX: device may even be the

parallel port; for the duration of this program, the AUX: device is this

clock.  Yes, the port may also be hardware-shared with a 300 baud MODEM.)

The command line is then parsed and inspected for a four digit year entry.

If one is found, it will be used to change the date in the directory for this

program near the end of this program.

The DOS has an internal character buffer which is then cleared just

prior to reading the GC-1000's data string.  Upon receipt of the 24

character string, the time is converted to the format required by the DOS

and the time is set.

Before the date can be set, the desired year must be determined.  The

year must come from one of 3 sources: (1) the command line, (2) the directory

entry, or (3) the GC-1000.  The command line year is used if it is present.

If not the directory entry is the next attempted source for the year.  Should

the directory entry, for any reason, fail, then the year from the GC-1000 is

used.

If the year is a non-leap year, then the date is adjusted.  With the

GC-1000 set for a leap year, all dates after February 28 will require a one

day adjustment.  (If the GC-1000 were set for a non-leap year, then all leap

years would be one day short.  I don't honestly know what the GC-1000 would

emit on the last day of those years.)

Once all of this is completed, the date is set.

Following the date set, the directory entry is updated with the current

time and date if and only if there was a command line entry of the year.

Just prior to program termination, the previously saved AUX: port

configuration is swapped back in to the system.