%OP%BON

%OP%AMM

%OP%HM3

%OP%FM3

%OP%BM0

%OP%LM10

%OP%HE/@T@

%OP%FO//Z88 Developer's Notes V3, @P@//

%OP%DEZ88 Hardware

%CO:A,12,80%%C%%H1%%H2%Z 8 8  H a r d w a r e%H2%%H1%

It is generally programming practice to use the operating system calls provided

rather than manipulating the hardware directly; it avoids compatibility

problems with future releases, and treading on the toes of the operating

system. We are therefore NOT going to encourage the reader to poke to the

screen or disable the operating system! However we shall attempt to give as

full a description of the hardware that exists.

%H2%Overview of the BLINK chip%H2%

The following table lists all the registers in the BLINK chip, which are

accessed through the Z80 I/O ports. Not all of the registers are useful to

programmers, but they are all included for completeness. Some are described in

more detail later in this section. Note that because registers can only be

written to or read from, but not both, it would be possible to lose track of

register contents. Therefore the operating system use addresses $0400 to $04FF

to store 'soft copies' of the values in the BLINK registers, with the low byte

of the address being taken from the address of the I/O port. You can read these

soft copies, to check out the contents of the registers, but most importantly

you should always write to the soft copy for a register before writing to a

register itself, for reasons explained below.

%H4%I/O port%H4%

$70

$71

$72

$73

$74

$B0

$B1

$B2

$B3

$B4

$B5

$B6

$D0

$D1

$D2

$D3

$D4

$E0

$E1

$E2

$E3

$E4

$E5

$E6

NOTE: RTC stands for "Real Time Clock", int. stands for "Interrupt", and ack.

stands for "acknowledge".

%P0%

%H2%The COM register%H2%

This register is detailed first because it controls various diverse aspects of

BLINK operations:

%H4%Bit%H4%

7

6

5

4

3

2

1

0

The two speaker control bits operate in the following fashion:

%H4%SRUN%H4%

0

0

1

1

RAMS is cleared on reset thus paging in ROM at logical addresses $0000 to

$1FFF. This is necessary since the Z80 program counter is loaded with zero on

reset. In normal operation RAMS is set and the lower 8K is the system RAM,

which is also used for application static workspace and stack.

%H2%Bank Switching%H2%

System calls inevitably require a significant time overhead while they decode

their input parameters and perform general housekeeping. This could have an

effect on some programs which need to switch banks around a great deal, so we

present here the method for binding banks via hardware. It should not be

necessary to use this code because of the existence of the fast code interface

described in "Miscellaneous useful routines". This section is included to

provide some explanations as how things work. If you want your code to be

compatible with future versions of the machine you should only use the fast

code interface.

To rebind one of the segments 0 to 3, the program writes the bank number into

the relevant "segment register"; there are four segment registers, one for each

segment. These are held in the BLINK chip and adressed via the Z80 I/O ports

$D0 to $D3. Thus:

will bind bank 20 (internal RAM) to segment 3.

Because the segment registers are write-only, the operating system needs 'soft

copies' of the current bindings so it can restore the old bindings correctly

after it has rebound banks at the start of OS calls or interrupt service

routines. These soft copies are stored at machine locations $04D0 to $04D3. It

is VITAL that the desired bindings are stored here BEFORE actually binding the

bank; if the bank were bound first, then in the event of an interrupt occurring

immediatly after the output instruction, the interrupt service routine would

rebind the bank to the contents of the soft copy (assuming it does rebind the

relevant segment), and the desired change would be lost. The equivalent of an

OS_MPB would be:

Similarly, the equivalent of the OS_MGB would be simply to read the soft copy:

%H2%EPROMs%H2%

No convenient system call is provided for blowing individual bytes to EPROM.

Hence we detail a method for doing so by directly manipulating the gate array

registers.

To write a byte to EPROM, the relevant physical address must be in slot 3, ie.

banks $C0 to $FF. The user first sets up the correct programming signals for

the type of EPROM concerned. This is done by writing to the BLINK register EPR,

addressed by I/O port $B3.

EPR, EPROM programming register:

%H4%BIT%H4%

7

6

5

4

3

2

1

0

NOTE: For 32K EPROMs write $48 (@@01001000) to EPR and for 128K, 256K EPROMs use

$69 (@@01101001).

The delay periods are as follows, but note that during overprogramming (when

OVERP is set in COM) the period is tripled. The porch period is not exact but

is never less than 2.4us.

%H4%PD1%H4%

0

0

1

1

To enter EPROM-programming mode, the user must set VPPON and PROGRAM in the COM

register, addressed by I/O port $B0, and also switch off the LCD by clearing

the LCDON bit. When this is done, programming can be achieved by writing to the

relevant address (which obviously needs to be a physical address in slot 3 ie.

banks $C0 to $FF) as though it were RAM. However, writing to slot 3 addresses

in program mode causes BLINK to take over. BLINK will hold the data and address

busses with the appropriate data and then do a porch cycle, followed by a delay

cycle and then another porch cycle before returning control to the processor.

Once a byte has been programmed, it needs to be verified. This is done by

exiting programming mode - clear VPPON and PROGRAM - and reading the byte back

to see if it has been successfully blown. No more than 75 attempts should be

made to blow any byte, and if it has not been blown by that stage, something is

clearly wrong. Note that a byte will usually be successfully blown in one

attempt. Once a byte has been successfully programmed, OVERP (in COM) should be

set and the byte overprogrammed. Each byte should be overprogrammed the same

number of times as it first took to program.

A code fragment to blow a byte in register C to logical address (HL) - assumed

to be bound to an EPROM address - is reproduced below. Note that when the Filer

writes to the EPROM it does so by blowing blocks of bytes, rather than writing

one byte at a time, and so operates a little faster than blowing each byte

individually.

.start

.proloop

; handle failure to write byte here...

.byte

.ovploop

The Z88 Assembler Workbench application %H2%Zprom%H2% uses the above algorithm in

principle, but blows to EPROM in blocks of max. 16K in one go (depending of

the user defined range) by using the standard Z80 instruction LDIR.

%P0%

%H2%UART Receive registers%H2%

RXC, receive data register

7   6   5   4   3   2   1   0

-------------------------------

D7  D6  D5  D4  D3  D2  D1  D0

-------------------------------

RXE, extended receive data register

%H4%BIT%H4%

7

6

5

4

3

2

1

0

RXC, receive control register

%H4%BIT%H4%

7

6

5

4

3

2

1

0

SHTW is used to align RXC register with the incoming data such that the byte

you want ends up in RXC and you do not get stuck with parity or stop bits. We

can see the effect of SHTW by showing how an incoming data stream is mapped

onto RXC and RXE. Suppose the incoming data is

--> S2 S1 D7 D6 D5 D4 D3 D2 D1 D0 ST --> Z88

%H4%BITS%H4%

S2

S1

D7

D7-D0

ST

When ARTS is set then RTS will be asserted every time a new character is

received and cleared when the receive register has been read. Manual control of

RTS can be achieved by using IRTS to change the current state of the line.

%P0%

The baud rates set by BAUD 2 to BAUD 0 are as follows:

%H4%Value%H4%

0

1

2

3

4

5

6

7

%H2%UART Transmit registers%H2%

The transmit data registers is 11 bits wide with the upper 3 bits being written

by the address lines A8-A10 during the OUT instruction. The register looks like

this:

If SHTW (in RXC) is set then STOP2 will not be sent out and only 10 bits will

be sent in all. To load the register you must use a code sequence like:

TXC, Transmit control register

%H4%BIT%H4%

7

6

5

4

3

2

1

0

When ATX is set the data in TXD will not be sent until CTS is asserted. Manual

control of the TxD pin can be achieved by using ITX to change the current state

of the line.

%P0%

%H2%UART interrupts and status%H2%

UIT, UART interrupt status register

%H4%BIT%H4%

7

6

5

4

3

2

1

0

The registers UMK and UAK control the enabling and acknowledging of the UART

interrupts. CTS and DCD can both be enabled to cause interrupts and the

interrupts can be set to occur on either state of the line (IDCD and ICTS in

the TXC register). To clear a DCD interrupt the state of IDCD needs to be

changed (since the interrupt is level and not edge triggered) and then the DCD

bit should be set in the interrupt acknowledge register UAK. CTS works in

exactly the same way as DCD.

UMK, UART interrupt mask register

%H4%BIT%H4%

7

6

5

4

3

2

1

UAK, UART interrupt acknowledge register

%H4%BIT%H4%

7

6

5

4 - 0

To clear TDRE and RDRE interrupts you write to the transmit data register or

read from the receive data register respectively.

%H2%UART examples%H2%

It is sometimes desirable to explicitly manipulate the RTS line of the serial

interface for example when auto-dialing modems. This may be done by

manipulating the IRTS bit in the RXC register. In programming terms this is bit

3 of I/O port $E2. It is important that the soft copy is updated first.

Bit 3 is XOR'ed with the normal state of the RTS line, as defined by RDRF in

UIT (receive data register full) and ARTS (auto RTS) in RXC.

The logic is RTS=[[RDRD AND ARTS] XOR IRTS]. This bit therefore toggles the RTS

line. A code fragment to achieve this might be:

The TxD (transmit data) line can be inverted by changing the state of ITX in

TXC (the transmit control register) using bit 3 of I/O port $E4 (soft copy

address $04E4) in a similar way to the RTS line. The value of the RxD line

(receive data) can be found by reading RXDB in RXE (extended receive data

register, using bit 4 of I/O port $E1 (soft copy $04E1 - the soft copy will not

necessarily be up to date).

%H2%The Keyboard%H2%

The keyboard is read by outputting a row on the address line A8 to A15 and

reading the KBD register in BLINK. The keyboard matrix looks like this:

-------------------------------------------------------------------------

| D7     D6      D5      D4      D3      D2      D1      D0

-------------------------------------------------------------------------

A15 (#7) | RSH    SQR     ESC     INDEX   CAPS    .       /       `

A14 (#6) | HELP   LSH     TAB     DIA     MENU    ,       ;       '

A13 (#5) | [      SPACE   1       Q       A       Z       L       0

A12 (#4) | ]      LFT     2       W       S       X       M       P

A11 (#3) | -      RGT     3       E       D       C       K       9

A10 (#2) | =      DWN     4       R       F       V       J       O

A9  (#1) | \      UP      5       T       G       B       U       I

A8  (#0) | DEL    ENTER   6       Y       H       N       7       8

-------------------------------------------------------------------------

DIA

SQR

LSH

RSH

LFT

RGT

DWN

UP

The keyboard can be read directly using a piece of code like below, although

note that while reading the keyboard, BLINK will stop the processor for up to

40us:

If KWAIT (in INT) is set then performing a key read read will send the machine

into Snooze state. When a key press is made the machine will wake up and if KEY

(in INT) is set then a keyboard interrupt will occur.

%P0%

%H2%Interrupts%H2%

The INT register controls which interrupts are enabled:

%H4%BIT%H4%

7

6

5

4

3

2

1

0

The ACK register is used to acknowledge and thus clear an interrupt:

%H4%BIT%H4%

7

6

5

4

3

2

1

0

The INT register provides information about which interrupt has actually

occurred:

%H4%BIT%H4%

7

6

5

4

3

2

1

0

%H2%Coma and Snooze%H2%

The COMA state is entered via a HALT instruction. Any interrupt can cause a

wake up, providing there is enough power to run the machine and the interrupt

is enabled. If all interrupts are disabled withing BLINK (regardless of whether

they are enabled in Z80 - the interrupt line to the Z80 comes from BLINK)

before a HALT instruction, then the machine will never be able to wake up

again. One special way of waking the machine is by ensuring KEY and KWAIT (in

INT) are set before executing HALT. During HALT the upper 8 address bits hold

the contents of the Z80 I (Interrupt) register. If this is set appropriately, a

special key sequence can cause an interrupt to occur, waking the machine. This

is how the pressing the two <SHIFT> keys wakes up the machine. The I register

is loaded with @@00111111 before HALT is executed. The two <SHIFT> keys occur in

the two left hand columns of the keyboard matrix, so they can be sensed with

the address lines in this state. Note that the minimum time between entering

COMA and waking up again is 5ms, so the COMA state would not be suitable for

waiting for regular interrupts.

Finally note that the UART will not work in COMA because all the UART clocks

are shut down.

During snooze the Z80 clock is stopped, but all the other system clocks

continue to run. Therefore it is possible to use the UART from snooze. Any

keyboard column going low will cause a wakeup from snooze, even if the keyboard

interrupts are disabled. Note that the keyboard data returned after wakeup will

not necessarily be valid, so the keyboard should definitely be read again.

%H2%Real Time Clock%H2%

%H4%Register%H4%

TIM0

TIM1

TIM2

TIM3

TIM4

The clock is reset to zero by settings RESTIM in the COM register. The clock

will be held in reset, ie. it won't count until RESTIM is cleared again. In

order to guarantee a valid result from any of these registers, software needs

to read until two equal values are returned. This should always occur within

three read cycles.

There are three interrupts that the real time clock can generate and these are

TICK (100 times a second), SEC (once a second) and MIN (once a minute). Three

registers are associated with these interrupts:

TSTA, Timer interrupt status

%H4%BIT%H4%

7 - 3

2

1

0

TMK, Timer interrupt mask

%H4%BIT%H4%

7 - 3

2

1

0

TACK, Timer interrupt acknowledge

%H4%BIT%H4%

7 - 3

2

1

0

%P0%

%H2%Unused I/O ports%H2%

The I/O ports $F0 to $FF are not used by the system and so are available to

hardware developers. The soft copy page $0400 is entirely reserve for I/O port

use and should be used. You must not use this memory for any other purpose.

Remember that when updating a soft copy, it must always be done before updating

the port because of the possibility of an interrupt occurring and receiving

bogus information about the state of a port.

%H2%The Screen and its Intricacies%H2%

The screen is generated from a 2K file containing character codes, window and

status information with four more files which contain definitions of those

characters. The four character sets are called LORES0, LORES1, HIRES0 and

HIRES1 where the LORES character sets consist of 6 by 8 pixel characters (used

for printing characters) and the HIRES characters are 8  by 8 (used for the

map area and the OZ window).

Each character position on screen is defined by two bytes in the screen file.

The format of these two bytes is like this:

Attribute 1 (even address):

7     6     5     4     3     2     1     0

---------------------------------------------

ch7   ch6   ch5   ch4  ch3   ch2    ch1   ch0

---------------------------------------------

Attribute 2 (odd address):

7     6     5     4     3     2     1     0

---------------------------------------------

sw1   sw2   hrs   rev   fls   gry   und   ch8

---------------------------------------------

The Attribute information makes the following contribution:

5     4     3     2     1     0,  7-0

hrs   rev   fls   gry   und   ch8-ch0     Description

--------------------------------------------------------------------

0     v     v     v     v     000-1BF     LORES1

0     v     v     v     v     01C-1FF     LORES0

1     1     1     v     v     000-1FF     LORES CURSOR

1     1     0     1     xxx    -  xxx     Null Character

1     0     v     v     000    -  2FF     HIRES0

1     0     v     v     300    -  3FF     HIRES1

x = don't care, v = valid, (ie. the attribute depends on the value of the bit).

The addresses of the screen files are held in BLINK registers PB0 to PB4 and

SBR. The role of these registers is as follows:

%H4%Register%H4%

PB0

PB1

PB2

PB3

SBR

%P0%

%H2%Uses of the Screen Registers%H2%

Obviously, any routines which look directly at screen information are very

hardware specific. Use of the OS_SCI call means that in future such routines

would not cause code to fail, in the way that writing directly to hardware

registers might, but if the system hardware arrangement did change, any code

which relies on the screen information, almost certainly would not work

properly. Therefore it makes sense that code using the OS_SCI call checks for

error returns and can cope with the concept of the screen information being

unavailable.

It is safe to move the address of the LORES0 and possibly HIRES0 as well. The

other areas are safe to read (find the address by using OS_SCI with a dummy

address and writing the old value straight away), but could only be moved with

extreme care. The reason for this is that in order to synchronize the display,

certain pixel patterns must be present on the extreme right (they're not

actually displayed). If there patterns are not present, the screem tends to

turn black and start flicker violently. The screen file is terminated by a NUL

character (shown in the attribute table above).

%P0%

%CO:B,12,68%

%H4%WRITE%H4%

PB0, pixel base reg.0

PB1, pixel base reg.1

PB2, pixel base reg.2

PB3, pixel base reg.3

SBR, screen base reg.

COM, command register

INT, mask & control

-

EPR, EPROM programming

TACK, RTC ack.

TMK, RTC int. mask

ACK, main int. mask

SR0, segment reg. 0

SR1, segment reg. 1

SR2, segment reg. 2

SR3, segment reg. 3

-

-

-

RXC, receiver control

TXD, transmit data

TXC, transmit control

UMK, UART int. mask

UAK, UART int. mask

%H4%Name%H4%

SRUN

SBIT

OVERP

RESTIM

PROGRAM

RAMS

VPPON

LCDON

%H4%SBIT%H4%

1

1

0

1

ld   a,20

out  ($D3),a

ld   bc, $04D0 + segment

ld   a, bank

ld   (bc),a

out  (c),a

ld   a,($04D0 + segment)

%H4%NAME%H4%

PD1

PD0

PGMD

EOED

SE3D

PGMP

EOEP

SE3P

%H4%PD0%H4%

0

1

0

1

ld   b, 75

ld   a, $48

out  ($B3),a

ld   a, $0E

out  ($B0),a

ld   (hl),c

ld   a, $04

out  ($B0),a

ld   a,(hl)

cp   c

jr   z, byteok

djnz proloop

ret

ld   a, 76

sub  b

ld   a,b

ld   a, $2E

out  ($B0),a

ld   (hl),c

ld   a, $04

out  ($B0),a

djnz ovploop

ld   a, $05

out  ($B0),a

ret

%H4%NAME%H4%

-

-

FE

RXD

TCLK

RCLK

PAR

START

%H4%NAME%H4%

SHTW

LOOP

UART

ARTS

IRTS

BAUD 2

BAUD 1

BAUD 0

%H4%If SHTW=0%H4%

FE (inverted

PAR

D7

D7-D0

START

%H4%(Baud2-0)%H4%

000

001

010

011

100

101

110

111

TXD: Transmit data register

10

--------------------------------------------

STOP2

--------------------------------------------

ld   a, data

ld   b, @@00000110

ld   c, TXD

out  (a),a

%H4%NAME%H4%

UTEST

IDCD

ICTS

ATX

ITX

BAUD2

BAUD1

BAUD0

%H4%NAME%H4%

RSRD

DCDI

CTSI

TDRE

-

RDRF

DCD

CTS

%H4%NAME%H4%

-

DCD

CTS

TDRE

-

RDRF

-

%H4%NAME%H4%

-

DCD

CTS

-

ld   bc, $04E2

ld   a,(bc)

or   8

ld   (bc),a

out  (c),a

<DIAMOND> key

<SQUARE> key

Left <SHIFT> key

Right <SHIFT> key

<LEFT> arrow key

<RIGHT> arrow key

<DOWN> arrow key

<UP> arror key

ld   c, $B2

ld   b, row

in   a, (c)

%H4%NAME%H4%

KWAIT

A19

FLAP

UART

BTL

KEY

TIME

GINT

%H4%NAME%H4%

-

A19

FLAP

-

BTL

KEY

-

-

%H4%NAME%H4%

FLAPOPEN

A19

FLAP

UART

BTL

KEY

-

TIME

%H4%Period%H4%

5ms

1 second

1 minute

256 minutes

64K minutes

%H4%NAME%H4%

-

MIN

SEC

TICK

%H4%NAME%H4%

-

MIN

SEC

TICK

%H4%NAME%H4%

-

MIN

SEC

TICK

%H4%Name%H4%

SC_LR0

SC_LR1

SC_HR0

SC_HR1

SC_SBR

%CO:C,12,56%

%H4%Function%H4%

Speaker source (0=SBIT, 1=TxD or 3200Khz)

SRUN=0: 0=low, 1=high; SRUN=1: 0=3200 Khz, 1=TxD

Set to overprogram EPROMs

Set to reset the RTC, clear to continue

Set to enable EPROM programming

Binding of lower 8K of segment 0: 0=bank 0, 1=bank 20

Set to turn programming voltage ON

Set to turn LCD ON, clear to turn LCD OFF

%H4%Effect%H4%

Speaker line low

Speaker line high

Speaker line oscillates at 3200 Khz

Speaker line attached to Tx data (Tx data still output

to comms. port)

%H4%FUNCTION%H4%

Two bits representing the length of delay period

State of program pulse during delay period

State of EOE during delay period

State of slot 3 select during delay period

State of program pulse during porch period

State of EOE during porch period

State of slot 3 select during porch period

%H4%Period%H4%

4.88us

312.5us

2.5ms

10ms

%H4%Function%H4%

Always clear

Always clear

Frame error, set if error

Value of RXD pin on BLINK

Internal transmit clock

Internal receince clock

Parity or first stop bit

Start bit (should be zero)

%H4%Function%H4%

Set to select short word mode

Set to connect transmit to receive (used for testing)

Set to hold UART in RESET

Auto RTS mode

Invert RTS

These three bits define receiver baud rate

)

Baud rate

75

300

600

1200

2400

9600

19200

38400

9

STOP1

%H4%Function%H4%

Set fast baud rate for receive and transmit

If set, DCD interrupts when low (else when high)

If set, CTS interrupts when low (else when high)

Auto transmit mode

Invert Tx data output pin

These three bits define transmit baud reate

(See above table for baud rates)

%H4%Function%H4%

Receive shift register full (cleared when read)

DCD interrupt

CTS interrupt

Transmit data register empty

-

Receive data register full

State of DCD line

State of CTS line

%H4%Function%H4%

-

If set, DCD interrupts are enabled

If set, CTS interrupts are enabled

If set, transmit data register empty interrupt enabled

-

If set, receive data register empty interrupt enabled

-

%H4%Function%H4%

-

Set to acknowledge DCD interrupt

Set to acknowledge CTS interrupt

-

%H4%Function%H4%

If set, reading the keyboard will Snooze

If set, an active high on A19 will exit Coma

If set, flap interrupts are enabled

If set, UART interrupts are enabled

If set, battery low interrupts are enabled

If set, keyboard interrupts (Snooze or Coma) are enabl.

If set, RTC interrupts are enabled

If clear, no interrupts get out of blink

%H4%Function%H4%

-

Acknowledge A19 interrupt

Acknowledge FLAP interrupt

-

Acknowledge battery low interrupt

Acknowledge keyboard interrupt

-

-

%H4%Function%H4%

If set, flap open else flap closed

If set, high level on A19 occurred during coma

If set, positive edge has occurred on FLAPOPEN

If set, an enabled UART interrupt is active

If set, battery low pin is active

If set, a column has gone low in snooze (or coma)

-

If set, an enabled TIME interrupt is active

%H4%Counts to%H4%

199

59

255

255

31

%H4%Function%H4%

-

Set if minute interrupt has occurred

Set if second interrupt has occurred

Set if tick interrupt has occurred

%H4%Function%H4%

-

Set to enable minute interrupt

Set to enable second interrupt

Set to enable tick interrupt

%H4%Function%H4%

-

Set to acknowledge minute interrupt

Set to acknowledge second interrupt

Set to acknowledge tick interrupt

%H4%Role%H4%

LORES0

LORES1

HIRES0

HIRES1

Screen file

%CO:D,12,44%

%H4%READ%H4%

-

-

-

-

-

-

STA, interrupt status

KBD, keyboard

-

-

TSTA, RTC int. status

-

TIM0, RTC 5ms counter

TIM1, RTC seconds counter (6 bits)

TIM2, RTC minutes counter

TIM3, RTC minutes/256 counter

TIM4, RTC minutes/64K counter (5 bits)

RXD, UART receive data register

RXE, extended receiver data

-

-

-

UIT, UART int. status

-

; do not use this code...

; under any circumstances

; Max. number of attempts

; for 32K EPROMs

; Set EPROM programming signals

; Set VPP and PROGRAM bits

; write byte to EPROM (address)

; Reset VPP and PROGRAM bits

; verify byte

; one more than maximum number of attempts

; same number of times it took to program

; Reset VPP, PROGRAM

; and OVERPROGRAM

; write byte

; reset VPP, PROGRAM and OVERPROGRAM

; set COM register to

; turn screen back on

%H4%If SHTW=1%H4%

Not clocked in

FE (inverted)

PAR

D7-D0

START

8

START

; 8 bits to send

; 2 stops, 1 start

; I/O port $B3

; transmit data

; address of soft copy

; fetch old value

; or AND 247 to reset

; update soft copy

; update hardware

; I/O port $B2

; one row of A15-8

; get column data in A

%H4%Width in bits%H4%

8

6

8

8

5

%H4%Granularity%H4%

512 bytes

4K bytes

8K bytes

2K bytes

2K bytes

%CO:E,12,32%

; address of soft copy

; bank number

; update the soft copy

; bind in the bank

7-0

D7 - D0

%CO:F,12,20%%CO:G,12,8%