%OP%BON

%OP%AMM

%OP%HM3

%OP%FM3

%OP%BM0

%OP%LM10

%OP%HE/@T@

%OP%FO//Z88 Developer's Notes V3, @P@//

%OP%DEGlossary

%CO:A,12,80%%C%%H1%%H2%G l o s s a r y%H2%%H1%

%H4%APPLICATION%H4%

%H4%BANK%H4%

%H4%BLINK%H4%

%H4%CARD MANAGER%H4%

%H4%CLI%H4%

%H4%DIRECTOR%H4%

%H4%DOR%H4%

%H4%EXPLICIT FILENAME%H4%

%H4%EXTENSION%H4%

%H4%FILTER%H4%

%H4%HANDLE%H4%

%H4%INSTANTIATION%H4%

%H4%LOGICAL ADDRESS%H4%

%H4%MOS%H4%

%H4%PAGE%H4%

%H4%PHYSICAL ADDRESS%H4%

%H4%PRE-EMPTION%H4%

%H4%PRINTER FILTER%H4%

%H4%PROCESS%H4%

%H4%SEGMENT%H4%

%H4%SLOT%H4%

%H4%STREAM%H4%

%P0%

%CO:B,12,68%%CO:C,12,56%

A program set up according to the appropriate machine

conventions. The program is blown into ROM or EPROM

cards using static data structures that identify the

hotkey to activate it, menu and help pages for

commands.

16K chunk of memory, always on a 16K boundary in the

logical address space. See also SEGMENT.

The name given to the Z88's gate array chip. The chip

handles the screen, serial port, keyboard, memory

switching, interrupts, etc.

This refers to the code which deals with inserting and

removing ROM cards. The basic rules are that all

interchange should be done from the INDEX and that the

machine is switched ON. If an application is using a

ROM card then the card's slot number is displayed in

the column next to the application name in the INDEX

popdown window. RAM cards can only be inserted, never

removed. File EPROM cards can be inserted and removed

at most time, but obviously not while the FILER is

actively reading or writing slot 3.

Command Line Interpreter. This is used to simulate the

keyboard and to rebind streams (re-direct or copy I/O

information).

synomym for INDEX (the name used for it while much of

the code was being written, hence DC operating

system calls, not IC calls).

node in dynamic tree structure used in applications and

in the RAM filing system.

A filename which includes the device and root

directories. For example "queries.pip" might become

":RAM.1/office.dir/queries.pip".

Up to three letters after the filename, usually used to

classify the file, eg. you might add .pip to all your

PipeDream files to aid identification.

Conceptual 'object' (actually a set of routines) which

perform transformations in a byte sequense, Also see

Printer Filter.

A handle is a 16bit value, usually returned in the IX

register, used in file-like operations (eg. files,

filters, etc.) to direct input and output. For example,

a handle is returned when a file is opened and is

subsequently used whenever the file is accessed. A file

can have several handles associated with it and

attributes like the sequential pointer into a file are

associated with each handle and not with the file

itself. There are a finite number of handles in the

machine, initially around 150, and the current number

available is reflected in the high word of the

sequential pointer associated with a handle value of -1

(In BASIC, PTR#-1).

This refers to one 'run' of an application. For example

if two copies of PipeDream are current, than each copy

is called an 'instantiation' and although the same

program is running, the data associated with each are

independent.

Address which the Z80 CPU 'sees', ie. a standard 16bit

Z80 address. See also PHYSICAL ADDRESS.

Machine operating system. Also referred to as OZ.

256 byte chunk of memory, on a 256-byte boundary. This

is the basic unit of memory on which the allocation

routines operate.

24 byte address referring to an actual memory location

(only 22bits are significant, giving an addressing

range of 4MB). The memory are organised, seen in 256

16K blocks, each slot addressing 1MB, giving a total of

4MB (14bit logical address + 8bit bank number = 22 bit)

The suspension of a process, normally prior to the main

body of a system call.

This is a filter controlled by the Printer Editor. It

allows applications to use special printing effects by

specifying filter codes and letting the filter generate

codes appropriate to the printer actually in use.

A particular instantiation of a program (normally an

application).

1) One of the four 16K portions of the logical memory

map (which the Z80 can see). All memory banks in the

system can be bound to one of the four memory segments

(64K/4 = 16K, hence four segments).

2) One of the portions of a filename separated by '/',

ie. a file or a directory name.

1) One of the three physical holes in the front of the

machine, which can accept a RAM, EPROM or ROM card. The

internal memory of the machine is thought of as Slot 0.

2) The transmission form for a character of data in the

serial system, ie. the byte to be transmitted plus

appropriate start and stop bits.

3) Concept used in PipeDream. Roughly approximates to a

spreadsheet cell.

This is a communication channel with an associated

handle.

%CO:D,12,44%%CO:E,12,32%%CO:F,12,20%%CO:G,12,8%